| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E368 |
| --- | --- |

| Title: | Crosscheck of Sharp's contribution (JCTVC-E411) on enforcing the 16-bit inverse transform dynamic range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-658-4511 Email: rajanj@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Sharp’s proposal (JCTVC-E277) to restrict the dynamic range at the input to the 1st and 2nd stage of the inverse transform. The results of our crosscheck matched those reported by Sharp. The BD-rate results are virtually unchanged compared to HM 2.0.

# Introduction

Sharp’s contribution proposes using clipping at the input to the first stage of the inverse transform. The clipping restricts the bit-depth to 16 bits. The clipping is applied in the dequantization operation before the left shift of (QP/6) is applied. Also at the output of first inverse transform stage, certain least significant bits from the transform coefficients are zeroed out so that the transpose buffer needs to store only 16 bits. In this crosscheck, we confirmed the results reported by Sharp.

- Results

Results for all 6 configurations described in the common test conditions were crosschecked. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding was done on a LINUX cluster and decoding was done on a PC using Microsoft Visual C++ (32-bit).

Table 1: BD-rate results for Sharp’s proposal

# Conclusions

We have verified results reported in Sharp’s proposal on enforcing 16-bit inverse transform dynamic range. The BD-rate results are virtually unchanged compared to HM 2.0. The encoding and decoding times also matched closely.