| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E369 |
| --- | --- |

| Title: | CE6.f: Cross-Verification of Mitsubishi/NHK’s LUT-Based Intra Prediction Filtering Contribution (JCTVC-E069) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | G. Van der Auwera, M. Coban, and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: geertv@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cross-verification of Mitsubishi/NHK’s LUT-based intra prediction filtering contribution (JCTVC-E069) confirms the rate-distortion results. The encoding and decoding times are similar with respect to HM2.0.

# Introduction

The LUT-based intra prediction filtering method filters the top most pixel row and left most pixel column of the DC predicted samples by employing two-tap filters that cross the top and left PU boundaries.

This contribution reports the objective rate-distortion results and the processing times.

# Objective Verification Results

## Rate-Distortion Results

Table 1 enumerates the rate-distortion results as BD-rate differences. The common test conditions are those specified in JCTVC-D600 for intra coding in the high-efficiency and low-complexity configurations. The conclusion is that the BD-rate differences match those reported by Mitsubishi/NHK for intra prediction filtering.

Table 1 Rate-distortion results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.1 | -0.4 | -0.2 | -0.2 |
| Class B | -0.3 | -0.2 | -0.2 | -0.4 | -0.4 | -0.4 |
| Class C | -0.3 | -0.2 | -0.2 | -0.4 | -0.3 | -0.3 |
| Class D | -0.2 | -0.2 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class E | -0.2 | -0.1 | -0.2 | -0.3 | -0.3 | -0.3 |
| All | -0.2 | -0.2 | -0.2 | -0.4 | -0.3 | -0.3 |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

## Encoding and Decoding Times

Table 1 reports the encoding and decoding times for Mitsubishi/NHK’s intra prediction filtering.

The encodings were executed on a Linux computing cluster consisting of machines with various computing performance and memory configurations. The encoder executable was built using GCC version 4.1.2. The differences in encoding times between our measurements and Mitsubishi/NHK’s are likely attributable to the variation in execution times on our computing cluster.

The decodings were executed on a PC with the following configuration: Windows 7, 6 GB memory, Intel Xeon 2.79GHz (W3530), Microsoft Visual Studio 2008. The decoding times are similar compared to the decoding times of the anchors.

# Source Code Evaluation

The method distinguishes between the following three PU sizes for intra prediction filtering after DC prediction: 16×16, 8×8, and 4×4. The top most pixel row and left most pixel column are filtered with filter taps [1  7]/8 for case 16×16, with filter taps [1  3]/4 for case 8×8, and with filter taps [3  5]/8 for case 4×4. The top-left corner points of each PU size are computed with separate three tap filters. The filters are applied across the top and left most PU boundaries.

# Conclusion

This cross-verification matches the rate-distortion results reported in Mitsubishi/NHK’s intra prediction filtering contribution (JCTVC-E069). The encoding and decoding times are similar to those from HM2.0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).