| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E370_r1 |
| --- | --- |

| Title: | CE10: Scaled orthogonal integer transforms supporting recursive factorization structure |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | R. Joshi, Y. Reznik, J. Sole, M. Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-658-4511 Email: rajanj@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In JCTVC-D256, 16 and 32-point scaled orthogonal integer transforms were proposed. These transforms support a recursive factorization structure. In this contribution this design is extended to 4 and 8-point transforms. The 16 and 32-point transforms use 36 and 92 multiplications, respectively, compared to 44 and 116 multiplications for the corresponding transforms in HM 2.0. A multiplication-free implementation for the proposed transforms is also provided. The luma BD-rate performance of the proposed transforms is nearly identical to the existing HM 2.0 transforms for normal and high QP ranges. For low QP range, the average luma BD-rate is -1.15%, averaged over all 6 configurations.

- Introduction

- The existing 16 and 32-point transforms used in HM 2.0 are based on the well-known Chen and Smith factorization [1], whereas the 4 and 8-point transforms in HM 2.0 are identical to the corresponding transforms in AVC/H.264. In [2], an alternative factorization for the 16 and 32-point transforms was proposed. These transforms are scaled orthogonal integer transforms that support recursive factorization. In this contribution, this design is extended to 4 and 8-point transforms.

- Transform specification

Figure 1 shows the 16-point transform that was proposed in [2]. The factors on the right hand side are normalization factors (not including). Thus, the transform is a scaled transform. In order for the transform to be orthogonal, the butterfly factors have to satisfy:

Z4=A42+B42, Z8=A82+B82=C82+D82, and

Z16=A162+B162=C162+D162=E162+F162=G162+H162.

Similar relationship holds for butterfly factors in the odd part of 32-point transform, that is:

Z32=A322+B322=C322+D322 and so on.

Figure 1: Proposed 16-point transform

It should be noted that the upper (even) part of the 16-point transform already contains the specification of the 8-point transforms. The choice of butterfly factors for the 8, 16 and 32-point transforms is shown in Tables 1-3.

| A4 | B4 | A8 | B8 | C8 | D8 |
| --- | --- | --- | --- | --- | --- |
| 17/64 | 41/64 | 111/128 | 22/128 | 94/128 | 63/128 |

| A16 | B16 | C16 | D16 | E16 | F16 | G16 | H16 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 232/256 | 29/256 | 224/256 | 67/256 | 203/256 | 116/256 | 181/256 | 148/256 |

| A32 | C32 | E32 | G32 | I32 | K32 | M32 | O32 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1013/1024 | 1003/1024 | 982/1024 | 958/1024 | 922/1024 | 859/1024 | 827/1024 | 757/1024 |
| B32 | D32 | F32 | H32 | J32 | L32 | N32 | P32 |
| 34/1024 | 146/1024 | 251/1024 | 331/1024 | 421/1024 | 538/1024 | 586/1024 | 674/1024 |

Table 1-3: Butterfly factors for the proposed 16 and 32 point transforms

Tables 1-3 specify the butterfly factors for a 32-point transform. Tables 1-2 specify the butterfly factors for a 16-point transform. Table 1 specifies the butterfly factors for a 8-point transform. In case of 4-point transform, we have chosen A4=2 and B4=5 (with no right shifts) as the butterfly factors.

# Multiplierless implementation

The proposed transforms can be implemented using only shifts and additions. This is accomplished by replacing a pair of multiplies in a butterfly by shifts and additions. Table 4 specifies a multiplierless design for the proposed transforms.

| Pair of factors | 1st factor (A) | 2nd factor (B) | Algorithm: y = A*x; z = B*x; | Complexity |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Additions | Shifts |
| A4, B4 | 2 | 5 | Y = x << 1; z = (x << 2) + x; | 1 | 2 |
| A4, B4 | 17 | 41 | x1 = x << 4; x2 = x << 3; x3 = x1 + x2; y = x + x1; z = x3 + x4; | 3 | 2 |
| A8, B8 | 111 | 22 | x1 = x << 3; x2 = x1 - x; x3 = x2 << 4; x4 = x2 << 1; y = x3 - x; z = x1 + x4; | 3 | 3 |
| C8, D8 | 94 | 63 | x1 = x << 5; x2 = x - x1; z = x1 - x2; y = z - x2; | 3 | 1 |
| A16, B16 | 232 | 29 | x1 = x << 2; x2 = x1 - x; x3 = x1 << 3; z = x3 - x2; y = z << 3; | 2 | 3 |
| C16, D16 | 224 | 67 | x1 = x << 5; x2 = x + x1; x3 = x2 << 1; z = x + x3; x5 = x1 << 3; y = x5 - x1; | 3 | 3 |
| E16, F16 | 203 | 116 | x2 = x + (x<<1); x3 = x << 5; x4 = x3 - x2; x5 = x4 << 3; z = x4 << 2; y = x5 - x4; | 3 | 4 |
| G16, H16 | 181 | 148 | x1 = x << 5; x2 = x << 2; x3 = x + x1; x4 = x2 + x3; z = x4 << 2; y = x3 + z; | 3 | 3 |
| A32, B32 | 1013 | 34 | x1 = x << 3; x2 = x << 1; x3 = x1 + x2; x4 = x << 10; x5 = x4 - x; x6 = x1 << 2; z = x2 + x6; y = x5 - x3; | 4 | 4 |
| C32, D32 | 1003 | 146 | x2 = x+(x<<6); x4 = x2+(x<<3); x6 = x+(x<<2); x8 = (x<<10)-x; x9 = x6<<2; y = x8-x9; z = x4<<1; | 5 | 6 |
| E32, F32 | 982 | 251 | x1 = x << 2; x2 = x << 4; x3 = x + x1; x4 = x2 << 4; z = x4 - x3; x6 = x2 - x4; x7 = z - x6; y = x7 << 1; | 4 | 4 |
| G32, H32 | 958 | 331 | x2 = x+(x<<5); x4 = (x<<9)-x2; x6 = x2+(x5<<2); z = x+(x6<<1); y = x4<<1; | 4 | 5 |
| I32, J32 | 922 | 421 | x2 = (x<<3)-x; x4 = (x2<<1)-x; x6 = x4+(x2<<6); x8 = (x2<<5)-x2; z = (x8<<1)-x4; y = x6<<1; | 5 | 6 |
| K32, L32 | 859 | 538 | x2 = x+(x<<5); x4 = (x<<8)-x2; y = (x4<<2)-x2; x7 = x+(x2<<3); x9 = x7+(x<<2); z = x9<<1); | 5 | 6 |
| M32, N32 | 827 | 586 | x2 = x+(x<<2); x4 = x2+(x<<8); x6 = x4+(x<<5); x8 = (x2<<6)-x2; y = x8+(x<<9); z = x6<<1; | 5 | 6 |
| O32, P32 | 757 | 674 | x2 = (x<<2)-x; x4 = x+(x2<<8); y = x4-(x2<<2); x7 = x+(x2<<7); x9 = x7-(x2<<4); z = x9<<1; | 5 | 6 |

Table 4: Multiplierless implementation of the proposed transforms

- Complexity measurement

## Arithmetic operations

Table 5 compares the complexity of the proposed transforms with the existing transforms in HM 2.0 in terms of multiplications and additions. The entries corresponding to 4 and 8-point transforms in HM 2.0 are blank because their specification is only in multiplierless format. Table 6 specifies the complexity of the proposed multiplierless transforms in terms of shifts and additions

|  |  | HM 2.0 | Proposed transform |
| --- | --- | --- | --- |
| 4-point | Additions |  | 8 |
|  | Multiplications |  | 2 |
| 8-point | Additions |  | 26 |
|  | Multiplications |  | 12 |
| 16-point | Additions | 74 | 72 |
|  | Multiplications | 44 | 36 |
| 32-point | Additions | 194 | 186 |
|  | Multiplications | 116 | 92 |

Table 5: Complexity comparison of the proposed transforms with HM 2.0 transforms

| Transform | Complexity of factorization | Complexity of multiplications | Complexity of full multiplierless transform |
| --- | --- | --- | --- |
| 4-point | 8 adds + 4 mults (2 pairs) | 2 * (1 adds + 2 shifts) | 10 adds + 4 shifts |
| 8-point | 26 adds + 12 mults (6 pairs) | 2 * (9 adds + 6 shifts) | 44 adds + 12 shifts |
| 16-point | 72 adds + 36 mults (18 pairs) | 2 * (26 adds + 23 shifts) | 124 adds + 46 shifts |
| 32-point | 186 adds + 92 mults (46 pairs) | 2 * (81 adds + 78 shifts) | 348 adds + 156 shifts |

Table 6: Transform complexity estimates in terms of shifts and additions.

## Dynamic range analysis

The bitdepth expansion due to each stage of the proposed 4, 8, 16 and 32-point transforms is, in the worst case, 4, 3, 4 and 5 bits, respectively. This is true both for forward transforms and inverse transforms.

The software framework provided by Cisco/TI was used to generate the bitdepths at various stages of the 2D inverse transform when a random matrix was used as an input to the 2D forward transform. This was repeated a large number of times. Table 7 shows the bitdepths at various stages of the 2D inverse transform.

To reduce the storage requirements at the transpose buffer (after 1st stage of inverse transform), we applied the method described in JCTVC-D071 and JCTVC-E411 to the output of the 1st stage of inverse transform. This involves the following 3 steps:

- Shift the transform coefficients to the right by B bits.

- Truncate the result to 16 bits.

- Shift left by B bits.

Here B is dependent on the transform size. With this change, all the entries in the 3rd row of Table 7 can be set to 16.

|  | IBDI ON |  |  |  | IBDI OFF |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4×4 | 8×8 | 16×16 | 32×32 | 4×4 | 8×8 | 16×16 | 32×32 |
| Output of inverse quantization | 12 | 13 | 14 | 15 | 12 | 13 | 14 | 15 |
| Input to the1st stage of inverse transform | 21 | 22 | 22 | 22 | 19 | 20 | 20 | 20 |
| Input to the 2nd stage of inverse transform | 21 | 21 | 23 | 23 | 19 | 19 | 21 | 21 |
| Output of the 2nd stage of inverse transform | 11 | 12 | 12 | 13 | 9 | 9 | 9 | 10 |

Table 7: Bitdepths at various stages of inverse transform

## Quantization matrix storage

The Q_BITS and DQ_BITS for various transform sizes was adjusted so that the quantization matrix for block size of 32×32 contained quantization matrices for block sizes of 16×16 and 8×8. Quantization matrix for 4×4 was stored separately. The storage of 4×4 quant and dequant matrices and the corresponding LUT requires 80 bytes. As described in [2], the storage of 32×32 quant and dequant matrices and the corresponding LUTs requires 3516 bytes. Thus total storage required for the quantization matrices is 3596 bytes.

- Results

The proposed transforms are integrated into HM 2.0. Simulations results are presented for the full sequences for intra, random access (RA) and low delay (LD) configurations for high efficiency (HE) and low complexity (LC) with common test conditions. Tables 8-10 show a summary of the results for the normal QPs, Low QPs and high QPs, respectively. Detailed results are provided in the accompanying excel files. The encoding was performed on a LINUX cluster. The decoding was performed on a PC using Microsoft Visual C++ (32-bit). We would like to thank TI for crosschecking the results for normal QP and Samsung for crosschecking the results for low and high QPs.

Tables 11-13 present additional results when the transpose buffer bitdepth is restricted to 16 bits, as described in Section 4.2. We would like to thank I2R for crosschecking these results for the normal QP range.

Table 8: BD-rate comparison for normal QP range

Table 9: BD-rate comparison for low QP range

Table 10: BD-rate comparison for high QP range

Table 11: BD-rate comparison for normal QP range (transpose buffer size = 16 bits)

Table 12: BD-rate comparison for low QP range (transpose buffer size = 16 bits)

Table 13: BD-rate comparison for high QP range (transpose buffer size = 16 bits)

- Conclusions

This contribution proposes scaled orthogonal integer transforms for replacing the existing core transforms in the HEVC Test Model. The proposed transforms support a recursive factorization structure. The proposed 16 and 32-point transforms use 36 and 92 multiplications, respectively, compared to 44 and 116 multiplications for the corresponding transforms in the existing test model. A multiplication-free implementation for the proposed transforms is also provided. The luma BD-rate performance of the proposed transforms is nearly identical to the existing HM 2.0 transforms for normal and high QP ranges. For low QP range, the average luma BD-rate is -1.15%, averaged over all 6 configurations. Additional results when restricting the size of the transpose buffer to 16 bits are also provided.

- References

- W.-H. Chen, C. H. Smith, and S. C. Fralick, A fast computational algorithm for the discrete cosine transform, IEEE Trans. Commun., vol. COMM-25, pp. 1004-1009, Sep. 1977.

- R. Joshi, Y. Reznik, J. Sole, and M. Karczewicz, “Efficient 16 and 32-point transforms”, JCTVC-D256, 4th JCT-VC Meeting, Daegu, KR, January 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).