| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E371 |
| --- | --- |

| Title: | CE6.c: Cross-check of Differential Coding of Intra Modes (DCIM, JCTVC-E318) by Sejong University |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. W. Seo, J. K. Han (Sejong University) J. Lim (SKT) 910, Choongmu-Gwan, Sejong University, 98 Gunja-Dong, Gwangjin-Gu, Seoul, 143-747, Korea 9-1 Sunae-Dong, Bundang-gu, Seongnam City, Kyeonggi-Do, 463-784, Korea | Tel: Email: | +82-2-3408-3739 sachonwon@gmail.com hjk@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong University and SK telecom |  |  |

_____________________________

# Abstract

This document provides the cross-check of Differential Coding of Intra Modes (DCIM) [1]. This cross-check was performed within the context of CE6 [4]. Sharp provided the source code with the proposal tool based on HM2.0. We inspected and compiled the source code. We also ran the codec with standard test configurations. The cross-check results match those proposed in JCTVC-E318 with similar encoding / decoding time.

# Methods for cross-checking

DCIM derives a candidate prediction mode based on edge information of neighboring blocks [1]. The intra coding mode for the current block is encoded by differential coding using the candidate mode as reference direction. The codes were compiled using Microsoft® Visual Studio® 2008, and executed on workstations with the following configurations: Microsoft® Windows 7 64bits, Intel® Xeon 2 Quad CPUs, 16GB of RAM.

# Simulation results

DCIM was evaluated according to JCTVC-D600 [2] and JCTVC-D606 [4]. Software provided by authors of DCIM [1] was successfully compiled by Microsoft Visual C++ 2008. As for anchor, HM2.0 software is used in this simulation. Intra only structure is tested. BD-rate (ΔBitrate [%]) [3] is measured by Excel sheet (HM2_results_template.xls) provided on the JCTVC reflector. The Table 1 shows the experimental results of DCIM for I slice only coding structure.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -0.5 | -0.3 | -1.0 | -0.2 | -0.3 |
| Class B | -1.3 | -0.9 | -1.0 | -1.2 | -1.3 | -1.5 |
| Class C | -1.8 | -1.5 | -1.8 | -2.2 | -1.9 | -2.1 |
| Class D | -1.3 | -0.9 | -1.0 | -1.7 | -1.3 | -1.4 |
| Class E | -1.8 | -1.7 | -1.6 | -2.3 | -0.5 | -0.9 |
| All | -1.4 | -1.1 | -1.1 | -1.6 | -1.1 | -1.3 |
| Enc Time[%] | 109% |  |  | 112% |  |  |
| Dec Time[%] | 102% |  |  | 108% |  |  |

Table 1. Intra only case

# Conclusions

DCIM achieves 1.4% and 1.6% BD-rate saving in intra high efficiency and intra low complexity conditions, respectively. Encoding and decoding times are increased on average 9%, 12%, 2%, and 8%, respectively. BD-rate saving are exactly matched with the results reported by [1]. Variation of the encoding and decoding times also has consistency with those of [1].

# References

- E. Maani, A. Tabatabai, T. Yamamoto, A. Tanizawa, and V. Drugeon, “Differential Coding of Intra Modes,” JCTVC-E318, 5th JCT-VC Meeting, Geneva, Switzerland, Mar. 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th JCT-VC Meeting, Daegu, Korea, Jan. 2011.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

- A. Tabatabai, K. Chono, et. al., “Core Experiment 6: Intra Prediction Improvement,” 4th JCT-VC Meeting, Daegu, Korea, Jan. 2011.

# Patent rights declaration(s)

Sejong University and SK Telecom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).