| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E372 |
| --- | --- |

| Title: | Cross-Verification: Simplified Planar Intra Prediction (HiSilicon, JCTVC-E289) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. W. Seo, J. K. Han (Sejong University) J. Lim (SKT) 910, Choongmu-Gwan, Sejong University, 98 Gunja-Dong, Gwangjin-Gu, Seoul, 143-747, Korea 9-1 Sunae-Dong, Bundang-gu, Seongnam City, Kyeonggi-Do, 463-784, Korea | Tel: Email: | +82-2-3408-3739 sachonwon@gmail.com hjk@sejong.ac.kr +82-31-710-5196 jeongyeon@sktelecom.com |
| Source: | Sejong University and SK telecom |  |  |

_____________________________

# Abstract

This document provides the cross-check of simplified planar intra prediction. This cross-check was performed within the context of CE6. HilSilicon provided the source code with the proposal tool based on HM2.0. We inspected and compiled the source code. We also ran the codec with standard test configurations. The cross-check results match those proposed in JCTVC-E289 with similar encoding / decoding time.

# Methods for cross-checking

In contribution [1] provided by HilSilicon, two simplified planar intra predictions are proposed. One is diagonal-based Intra prediction and other is sub-block based Intra prediction. The codes were compiled using Microsoft® Visual Studio® 2008, and executed on workstations with the following configurations: Microsoft® Windows 7 64bits, Intel® Xeon 2 Quad CPUs, 16GB of RAM.

# Simulation results

The technique [1] was evaluated according to JCTVC-D600 [2] and JCTVC-D606 [4]. Software provided by authors of HilSilicon was successfully compiled by Microsoft Visual C++ 2008. As for anchor, HM2.0 software is used in this simulation. Intra only structure is tested. BD-rate (ΔBitrate [%]) [3] is measured by Excel sheet (HM2_results_template.xls) provided on the JCTVC reflector. Four kinds of intra prediction methods were tested and the Table 1~4 show the experimental results for I slice only coding structure.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | 0.1 | 0.3 | -0.9 | -0.6 | -0.4 |
| Class B | -0.8 | -0.3 | -0.3 | -0.8 | -1.0 | -0.9 |
| Class C | -0.5 | -0.1 | 0.0 | -0.6 | -0.8 | -0.8 |
| Class D | -0.5 | 0.0 | 0.1 | -0.6 | -0.8 | -0.8 |
| Class E | -0.9 | -0.2 | 0.2 | -0.9 | -0.8 | -0.8 |
| All | -0.7 | -0.1 | 0.0 | -0.7 | -0.8 | -0.8 |
| Enc Time[%] | 104% |  |  | 108% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

Table 1. Performance of the existing Planar Intra Prediction (JCTVC-D235)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | 0.3 | 0.4 | -0.5 | -0.4 | -0.2 |
| Class B | -0.5 | 0.1 | 0.1 | -0.5 | -0.8 | -0.6 |
| Class C | -0.4 | 0.0 | 0.2 | -0.4 | -0.7 | -0.8 |
| Class D | -0.5 | 0.1 | 0.2 | -0.5 | -0.7 | -0.8 |
| Class E | -0.6 | 0.1 | 0.4 | -0.5 | -0.6 | -0.6 |
| All | -0.5 | 0.1 | 0.3 | -0.5 | -0.7 | -0.6 |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |

Table 2. Performance of Diagonal-based Intra Prediction

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | 0.0 | 0.3 | -0.8 | -0.6 | -0.4 |
| Class B | -0.6 | -0.1 | -0.2 | -0.6 | -0.9 | -0.8 |
| Class C | -0.5 | 0.0 | 0.1 | -0.5 | -0.7 | -0.8 |
| Class D | -0.5 | 0.1 | 0.2 | -0.5 | -0.7 | -0.7 |
| Class E | -0.5 | -0.1 | 0.4 | -0.5 | -0.5 | -0.3 |
| All | -0.5 | 0.0 | 0.1 | -0.6 | -0.7 | -0.6 |
| Enc Time[%] | 105% |  |  | 108% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Table 3. Performance of 2x2-based Intra Prediction

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | 0.0 | 0.1 | -0.9 | -0.8 | -0.6 |
| Class B | -0.8 | -0.3 | -0.4 | -0.7 | -1.0 | -0.9 |
| Class C | -0.5 | -0.2 | 0.0 | -0.5 | -0.8 | -0.8 |
| Class D | -0.5 | 0.0 | 0.1 | -0.6 | -0.8 | -0.8 |
| Class E | -0.7 | -0.2 | 0.4 | -0.7 | -0.6 | -0.6 |
| All | -0.7 | -0.2 | 0.0 | -0.7 | -0.8 | -0.8 |
| Enc Time[%] | 105% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table 4. Performance of 4x4-based Intra Prediction

# Conclusions

This techniques achieve from 0.5% to 0.7% BD-rate saving in intra high efficiency and intra low complexity conditions. Encoding and decoding times are increased on average 5%, 7%, 2%, and 1%, respectively. BD-rate saving are exactly matched with the results reported by [1]. Variation of the encoding and decoding times also has consistency with those of [1].

# References

- Y. Lin, L. Liu, C. Lai, and J. Zheng, “Simplified Planar Intra Prediction,” JCTVC-E289, 5th JCT-VC Meeting, Geneva, Switzerland, Mar. 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th JCT-VC Meeting, Daegu, Korea, Jan. 2011.

- Gisle Bjøntegaard, “Improvements of the BD-PSNR model”, ITU-T SG16/Q6, 35th VCEG Meeting, Berlin, Germany, July 2008, Doc. VCEG-AI11.

- A. Tabatabai, K. Chono, et. al., “Core Experiment 6: Intra Prediction Improvement,” 4th JCT-VC Meeting, Daegu, Korea, Jan. 2011.

# Patent rights declaration(s)

Sejong University and SK Telecom may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).