| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E373 |
| --- | --- |

| Title: | CE2: Test results of non-rectangular motion partitioning (NRMP) with overlapped block motion compensation (OBMC) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhen Zheng, HiSilicon Haoping Yu, Huawei Philippe Bordes, Technicolor Liwei Guo, Qualcomm Peisong Chen, Qualcomm | Tel: Email: | zhengxiaozhen@huawei.com philippe.bordes@technicolor.com liweig@qualcomm.com |
| Source: | Huawei & HiSilicon, Qualcomm, Samsung, Technicolor |  |  |

_____________________________

# Abstract

This contribution is related to Core Experiment 2: flexible motion partitioning. In this document, test result of non-rectangular motion partitioning (NRMP) with overlapped block motion partition (OBMC) is provided. The average gain for high efficiency is from 0.9% to 1.6%.

# Introduction

## Non-Rectangular Motion Partitioning (NRMP)

Non-rectangular motion partitioning (NRMP) is a subset of the method proposed at JCTVC-D230[1] and JCTVC-D297[2]. NRMP enables to partition a square block into 2 regions with non-rectangular shape. At the current design, NRMP is applied to the CU with size of 64x64, 32x32, 16x16 and 8x8.

According to CE2 document JCTVC-D602’s test plan, three different non-rectangular motion partitioning methods, which correspond to subtest 1A, 1B and 1C, are tested: 1. 2 diagonal partitioning; 2. 6 diagonal partitioning; 3. 6 not-only diagonal partitioning. The examples of those partitioning methods are illustrated at Figure

Figure 1A : 2 diagonal partitioning (subtest 1A)

Figure 1B: 6 diagonal partitioning (subtest 1B)

Figure 1C: 6 not-diagonal only partitioning (subtest 1C)

Figure 1: Examples of three different partitioning methods

## Overlapped block motion compensation

It is well-known that overlapped block motion compensation (OBMC) used overlapped blocks in motion compensation to reduce blocking artifact along motion partition boundaries [3]. When a coding unit uses asymmetric motion partition, transform blocks of the residual quadtree transform will cross the partition boundaries if the split of residual quadtree transform stops at the first or the second level. Using OBMC can improve coding efficiency by reducing the high frequency energy along the partition boundaries and result in higher coding efficiency and better visual quality. The overlapped region in the luma component is defined as 2 rows (columns) of pixels on both sides of the horizontal (vertical) partition boundary. For chroma components, the overlapped region is reduced to 1 row (column) of pixels. The weighting factors are (3/4, 1/4) for the pixels which are 1 row (column) apart from the boundary and (7/8, 1/8) for pixels which are 2 row (column) apart from the boundary.

# Experimental results

The coding efficiency is evaluated in RandomAccess (RA) and LowDelay (LD) configurations for high efficiency case based on the common conditions defined in JCTVC-D600 [4]. The experimental results shown in Table 1, Table 2 and Table 3 are subtest 1A, 1B and 1C results respectively.

Table 1: Subtest 1A result (2 diagonal partitioning)

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.4 | -1.4 | -1.1 |  |  |  |
| Class B | -0.3 | -1.1 | -1.0 | -0.2 | -1.7 | -1.9 |
| Class C | -1,0 | -1,6 | -1,7 | -1.1 | -2.0 | -2.3 |
| Class D | -1,2 | -1,9 | -1,9 | -1.6 | -2.4 | -2.4 |
| Class E |  |  |  | -0.9 | -2.4 | -1.2 |
| All | -0.7 | -1.5 | -1.4 | -0.9 | -2.1 | -2.0 |
| Enc Time | 154% |  |  | 152% |  |  |
| Dec Time |  |  |  | 113% |  |  |

Table 2: Subtest 1B result (6 diagonal partitioning)

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.4 | -1.4 | -1.5 |  |  |  |
| Class B | -0.4 | -1.2 | -1.3 | -0.3 | -2.1 | -2.3 |
| Class C | -1,2 | -1,9 | -2,0 | -1.3 | -2.3 | -2.6 |
| Class D | -1,3 | -2,0 | -2,0 | -1.7 | -2.9 | -2.6 |
| Class E |  |  |  | -1.1 | -2.7 | -1.8 |
| All | -0.8 | -1.6 | -1.7 | -1.1 | -2.5 | -2.4 |
| Enc Time | 173% |  |  | 172% |  |  |
| Dec Time |  |  |  |  |  |  |

Table 3: Subtest 1C result (6 not-only diagonal partitioning)

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.8 | -1.8 | -2.0 |  |  |  |
| Class B | -0,6 | -1,5 | -1,5 | -0.7 | -2.3 | -2.8 |
| Class C | -1,5 | -2,4 | -2,5 | -1.7 | -2.9 | -3.1 |
| Class D | -1,6 | -2,2 | -2,3 | -2.1 | -2.7 | -3.1 |
| Class E |  |  |  | -2.1 | -3.2 | -2.2 |
| All | -1.1 | -2.0 | -2.0 | -1.6 | -2.7 | -2.8 |
| Enc Time | 175% |  |  | 175% |  |  |
| Dec Time |  |  |  | 119% |  |  |

Compared to HM2.0, in randomaccess case the average bit rate reduction is 0.7% for test 1A, 0.8% for test 1B and 1.1% for test 1C. In lowdelay case, the average bit rate reduction is 0.9 % for test 1A, 1.1 for test 1B and 1.6 for test 1C.

# Conclusion

In this contribution, NRMP + OBMC is proposed. Average gain of coding efficiency against HM2.0 is 0.7% to 1.1% for RA and 0.9% to 1.6% with 50% to 75% encoder time increase for LD in high efficiency case.

# Reference

- Edouard Francois, Philippe Bordes (Technicolor), Liwei Guo, Marta Karczewicz (Qualcomm), “CE2: Simplified Geometry Block Partitioning”, Doc. JCTVC-D230 (m18990), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011

- X.Zheng (HiSilicon), H.Yu (Huawei), “CE2: Huawei & HiSilicon report on flexible motion partitioning coding”, Doc. JCTVC-D297 (m19063), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- M. T. Orchard and G. J. Sullivan, “Overlapped block motion compensation: An estimation theoretic approach,” IEEE Trans. Image Processing, vol. 3, pp. 693–699, Sept. 1994.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-D600 (m19497), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

# Patent rights declaration(s)

Huawei Technologies, Qualcomm, and Technicolor may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).