| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E374 |
| --- | --- |

| Title: | CE2: Unified solution of flexible motion partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Technicolor Peisong Chen, Qualcomm Il-Koo Kim, Samsung Liwei Guo, Qualcomm Haoping Yu, Huawei Xiaozhen Zheng, HiSilicon | Tel: Email: | philippe.bordes@technicolor.com ilkoo.kim@samsung.com liweig@qualcomm.com zhengxiaozhen@huawei.com |
| Source: | Huawei & HiSilicon, Qualcomm, Samsung, Technicolor |  |  |

_____________________________

# Abstract

This contribution is related to Core Experiment 2: flexible motion partitioning. In this contribution, Asymmetric Motion Partitioning (AMP) + Non-Rectangular Motion Partitioning (NRMP) + Overlapped Block Motion Compensation (OBMC) is proposed.

# Introduction

## Asymmetric motion partitioning

The Asymmetric motion partitioning (AMP) was initially proposed in JCTVC-A124[1]. It allowed rectangular shape PU splitting of a block for inter prediction. Figure 1 shows an example of the AMP where a 64x64 block is asymmetrically partitioned into 64x16, 64x48, 16x64, or 48x16 blocks, instead of two 64x32 or 32x64 blocks. The motion partitions shown in the figure correspond to 2NxnU, 2NxnD, nLx2N, and nRx2N, respectively. AMP improves the coding efficiency, since irregular image patterns, which otherwise would be constrained to being represented by a symmetric partition, can now be more efficiently represented without requiring further splitting.

Figure 1 Example of asymmetric motion partitions for 64x64 blocks

## Non-Rectangular Motion Partitioning (NRMP)

Non-rectangular motion partitioning (NRMP) is a subset of the method proposed at JCTVC-D230[4] and JCTVC-D297[5]. NRMP enables to partition a square block into 2 regions with non-rectangular shape. At every CU, 2 diagonal-only partitions (45 degree or 135 degree partitioning) are used. At the current design, NRMP is applied to the CU with size of 64x64, 32x32, 16x16 and 8x8.

Figure 2: Example of 2 diagonal partitioning modes

## Overlapped block motion compensation

It is well-known that overlapped block motion compensation (OBMC) used overlapped blocks in motion compensation to reduce blocking artifact along motion partition boundaries [3]. When a coding unit uses asymmetric motion partition, transform blocks of the residual quadtree transform will cross the partition boundaries if the split of residual quadtree transform stops at the first or the second level. Using OBMC can improve coding efficiency by reducing the high frequency energy along the partition boundaries and result in higher coding efficiency and better visual quality. The overlapped region in the luma component is defined as 2 rows (columns) of pixels on both sides of the horizontal (vertical) partition boundary. For chroma components, the overlapped region is reduced to 1 row (column) of pixels. The weighting factors are (3/4, 1/4) for the pixels which are 1 row (column) apart from the boundary and (7/8, 1/8) for pixels which are 2 row (column) apart from the boundary.

# Experimental results

The coding efficiency is evaluated in RandomAccess (RA) and LowDelay (LD) configurations for high efficiency case based on the common conditions defined in JCTVC-D600 [6]. The coding efficiency is measured by using 4 point BD-Rate. The experimental results shown in Table 1 are AMP + NRMP + OBMC result.

Table 1: AMP + NRMP + OBMC result

|  | Random access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -1.1 | -2.2 | -2.3 |  |  |  |
| Class B | -1.0 | -1.9 | -1.9 | -1.2 | -2.9 | -3.3 |
| Class C | -1.9 | -2.8 | -2.8 | -2.1 | -3.2 | -3.3 |
| Class D | -1.9 | -2.8 | -2.8 | -2.6 | -3.6 | -3.5 |
| Class E |  |  |  | -3.0 | -4.2 | -3.7 |
| All | -1.4 | -2.4 | -2.4 | -2.1 | -3.4 | -3.4 |
| Enc Time | 174% |  |  | 171% |  |  |
| Dec Time | 117% |  |  | 110% |  |  |

Compared to HM2.0, in randomaccess case the average bit rate reduction is 1.4%. In lowdelay case, the average bit rate reduction is 2.1 %..

# Conclusion

In this contribution, AMP + NRMP + OBMC is proposed. Average gain of coding efficiency against HM2.0 is 1.4% for RA and 2.1% for LD in high efficiency case. It is recommended to incorporate AMP + NRMP + OBMC into HEVC.

# Reference

- JCT-VC, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, JCT-VC Meeting, Dresden, DE, April 2010.

- JCT-VC, “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, JCT-VC Meeting, Dresden, DE, April 2010.

- M. T. Orchard and G. J. Sullivan, “Overlapped block motion compensation: An estimation theoretic approach,” IEEE Trans. Image Processing, vol. 3, pp. 693–699, Sept. 1994.

- Edouard Francois, Philippe Bordes (Technicolor), Liwei Guo, Marta Karczewicz (Qualcomm), “CE2: Simplified Geometry Block Partitioning”, Doc. JCTVC-D230 (m18990), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011

- X.Zheng (HiSilicon), H.Yu (Huawei), “CE2: Huawei & HiSilicon report on flexible motion partitioning coding”, Doc. JCTVC-D297 (m19063), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-D600 (m19497), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

# Patent rights declaration(s)

Huawei Technologies, Qualcomm, Samsung and Technicolor may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).