| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E375 WG11 Number: m19906 |
| --- | --- |

| Title: | Design consideration for compatibility with AVC NAL units |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | ChongSoon Lim Wahadaniah Viktor Blk 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | (65) 65505487 ChongSoon.Lim@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution presents a suggestion to JCTVC to consider the functionality of backward compatibility in HEVC NAL unit header design with AVC NAL unit header using reserved NAL unit type values of AVC.

- HEVC NAL units

In the current HEVC WD 2.0, HEVC coded data are contained in NAL units similar to AVC design.  The structure of the NAL unit header is the same as AVC where the first byte contains forbidden_zero_bit, nal_ref_idc and nal_unit_type information.  Since the structure for the HEVC NAL unit header is similar to AVC design, one possible design consideration for HEVC NAL units is the support for the functionality to be compatible to AVC NAL units.

- AVC + HEVC hybrid coding

AVC standard is widely used today in several applications.  HEVC can provide better compression gain over AVC but to replace AVC completely in some of today’s applications may not be possible in near term.  One of the potential use cases for HEVC is to extend the functionalities of today’s application with better compression gain.

An example of a coded stream with both AVC NAL unit and HEVC NAL unit:

AVC NAL unit

HEVC NAL unit

AVC NAL unit

HEVC NAL unit

Basic services with legacy players

When decoding such a coded stream, legacy AVC decoder will only decode the AVC NAL units and will ignore the HEVC NAL units.

Enhanced services with new players

When decoding such a coded stream, an AVC+HEVC decoder will decode both AVC and HEVC NAL units to provide additional functionality with improved coding efficiency.

Examples of such functionalities include:

- Increasing the frame rate (where additional frames are coded in HEVC)

- Increasing the number of views (where two views are coded in AVC Stereo High profile and additional views are coded in HEVC)

This design consideration is similar to MVC and SVC where the base view or base layer is compatible to AVC.

- Reserved NAL unit type values

The AVC standard still have some reserved NAL unit type values.  They are NAL unit type values 16, 17, 18, 21, 22, 23 (Total = 6).  In the 4th JCT-VC meeting in Daegu, JCTVC-D080 provides a suggestion to reduce one bit from nal_ref_idc and extends the nal_unit_type from five bits to six bits.  If this suggestion is adopted, the available NAL unit type values will be increased.  And since HEVC reuses some of the AVC NAL units, the available NAL unit type values may be sufficient to define new NAL units for HEVC.

- Suggestion

As HEVC standardization is currently at its early stage (Working Draft), it is still not clear how many new NAL units will be created for HEVC.  However, it may be good to start considering if such compatibility to AVC NAL units can be achieved until a point when the number of new HEVC NAL units exceeds the available reserved NAL unit type values of AVC.  Therefore we suggest JCT-VC to consider this aspect in the design of HEVC NAL unit header.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).