| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E376 |
| --- | --- |

| Title: | Asymmetric motion partition with OBMC and Non-Square TU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuan Yuan, Tsinghua Xiaozhen. Zheng, HiSilicon Xiulian Peng, USTC Lingzhi Liu, HiSilicon Yunfei Wang, Tsinghua Xiaoran Cao, Tsinghua Jizheng Xu, Microsoft Changcai Lai, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei | Tel: Email: | y-yuan08@mails.tsinghua.edu.cn zhengxiaozhen@huawei.com |
| Source: | Tsinghua University, Huawei, HiSilicon, Microsoft, University of Science and Technology of China (USTC) |  |  |

_____________________________

# Abstract

This contribution is related to core experiment 2: flexible motion partitioning. In this contribution, non-square Transform Unit is proposed to be used at asymmetric motion partitions. Overlapped block motion compensation (OBMC) is applied at motion partition boundary.

# Introduction

## Asymmetric motion partition

The Asymmetric motion partitioning (AMP) was initially proposed in JCTVC-A124[1]. It allowed rectangular shape PU splitting of a block for inter prediction. Figure 1 shows an example of the AMP where a 64x64 block is asymmetrically partitioned into 64x16, 64x48, 16x64, or 48x16 blocks, instead of two 64x32 or 32x64 blocks. The motion partitions shown in the figure correspond to 2NxnU, 2NxnD, nLx2N, and nRx2N, respectively. AMP improves the coding efficiency, since irregular image patterns, which otherwise would be constrained to being represented by a symmetric partition, can now be more efficiently represented without requiring further splitting.

Figure 1 Example of asymmetric motion partitions for 64x64 blocks

## Overlapped block motion compensation

It is well-known that overlapped block motion compensation (OBMC) used overlapped blocks in motion compensation to reduce blocking artifact along macroblock boundaries [3]. When a coding unit uses asymmetric motion partition, transform blocks of the residual quadtree transform will cross the partition boundaries if the split of residual quadtree transform stops at the first or the second level. Using OBMC can improve coding efficiency by reducing the high frequency energy along the partition boundaries and result in higher coding efficiency and better visual quality. The details of OBMC technique can be found at [5].

## Non-Square Transform Unit

At current HM platform, residual quad transform (RQT) is used at block residual transform. A transform block can be split recursive. When a transform block cross the boundary of motion block, high frequency coefficients are generated which will impose negative influence on coding performance. For the case of AMP, this problem will be more serious because transform block at first and second level will cross motion blocks' boundary. To solve this problem, non-square transform unit (TU) is applied to asymmetric motion partitions.

At non-square TU design, 2Nx2N transform is still used at RQT’s first level, 0.5Nx2N transform or 2Nx0.5N transform is applied to the second level when asymmetric partition is used. For the case of RQT’s third level, the transform block inside non-square TU can be further split to 0.5Nx0.5N. The implementation is illustrated at figure 2.

Figure 2: Illustration of non-square TU design

At current non-square design, no new core transform is introduced. The core transform of 2Nx0.5N and 0.5Nx2N can be reused by 0.5Nx0.5N and 2Nx2N core transform. In general, a nxm block is transformed by the following steps as described in [7].

Cnxm = Tm x Bnxm x TnT

where Bnxm denotes a block with n pixels m rows. Tn and Tm are the transform matrices of size nxn and mxm, respectively. Cnxm denotes the transformed nxm block.

According to the above analysis, 2Nx0.5N core transform can be achieved as follow:

C2Nx0.5N = T0.5N x B2Nx0.5N x T2NT

0.5Nx2N core transform can be achieved as follow:

C0.5Nx2N = Tm x B0.5Nx2N x T0.5NT

The only new added module for non-square TU is the quantization tables for 2Nx0.5N and 0.5Nx2N transform. Based on the implementation of AMP + OBMC at CE2, non-square TU is used at 32x8, 8x32, 16x4 and 4x16 asymmetric partition at luma component, and is used at 16x4, 4x16, 8x2 and 2x8 at chroma component.

# Experimental results

The proposed method is implemented on CE2 software[6]. The coding efficiency is evaluated in randomaccess and lowdelay configurations for both high efficiency and low complexity cases based on the common conditions defined in JCTVC-600 [4]. The coding efficiency is measured by using BD-Rate . The experimental results are shown in Table 1 and Table 2. Table 1a and 1b is the result of AMP + OBMC. Table 2a and 2b is result of AMP + OBMC + Non-Square TU. Table 3a and 3b show the result of AMP + Non-Square TU.

Table 1a: “AMP + OBMC” result (high efficiency)

|  | randomaccess |  |  | lowdelay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.7 | -1.0 | -1.3 |  |  |  |
| Class B | -0.8 | -1.0 | -1.1 | -1.0 | -1.3 | -1.6 |
| Class C | -1.0 | -1.4 | -1.4 | -1.2 | -1.5 | -1.5 |
| Class D | -0.9 | -1.2 | -1.1 | -1.4 | -1.2 | -1.7 |
| Class E |  |  |  | -2.4 | -2.6 | -1.6 |
| All | -0.8 | -1.2 | -1.2 | -1.4 | -1.6 | -1.6 |
| Enc Time | 114% |  |  | 113% |  |  |
| Dec Time | 103% |  |  | 100% |  |  |

Table 1b: “AMP + OBMC” result (low complexity)

|  | Randomaccess_loco |  |  | Lowdelay_loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.6 | -1.0 | -1.0 |  |  |  |
| Class B | -0.7 | -1.0 | -0.9 | -0.7 | -1.0 | -0.6 |
| Class C | -0.9 | -1.3 | -1.3 | -0.8 | -1.0 | -1.2 |
| Class D | -0.6 | -1.1 | -1.0 | -0.7 | -0.9 | -1.0 |
| Class E |  |  |  | -1.9 | -1.8 | -1.5 |
| All | -0.7 | -1.1 | -1.0 | -1.0 | -1.1 | -1.0 |
| Enc Time | 112% |  |  | 112% |  |  |
| Dec Time | 103% |  |  | 110% |  |  |

Table 2a: “AMP + OBMC + Non-Square TU” result (high efficiency)

|  | randomaccess |  |  | lowdelay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.8 | -1.3 | -1.5 |  |  |  |
| Class B | -0.9 | -1.8 | -1.6 | -1.4 | -3.5 | -3.7 |
| Class C | -1.2 | -2.0 | -2.0 | -1.6 | -3.1 | -3.2 |
| Class D | -1.0 | -1.6 | -1.6 | -1.9 | -3.2 | -3.2 |
| Class E |  |  |  | -3.4 | -5.0 | -4.0 |
| All | -1.0 | -1.7 | -1.7 | -2.0 | -3.6 | -3.5 |
| Enc Time | 114% |  |  | 114% |  |  |
| Dec Time | 100% |  |  | 101% |  |  |

Table 2b: “AMP + OBMC + Non-Square TU” result (low complexity)

|  | Randomaccess_loco |  |  | Lowdelay_loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.8 | -1.6 | -1.5 |  |  |  |
| Class B | -1.2 | -2.3 | -2.1 | -2.1 | -3.9 | -3.4 |
| Class C | -1.2 | -2.1 | -2.0 | -1.8 | -2.8 | -2.9 |
| Class D | -0.8 | -1.5 | -1.5 | -1.5 | -2.2 | -2.0 |
| Class E |  |  |  | -3.8 | -3.8 | -3.8 |
| All | -1.0 | -1.9 | -1.8 | -2.2 | -3.2 | -3.0 |
| Enc Time | 113% |  |  | 112% |  |  |
| Dec Time | 102% |  |  | 98% |  |  |

Table 3a: “AMP + Non-Square TU” result (high efficiency)

|  | randomaccess |  |  | lowdelay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.6 | -1.0 | -1.1 |  |  |  |
| Class B | -0.7 | -1.4 | -1.3 | -1.1 | -3.1 | -3.4 |
| Class C | -0.9 | -1.5 | -1.3 | -1.1 | -2.3 | -2.6 |
| Class D | -0.7 | -1.2 | -1.1 | -1.3 | -2.3 | -2.6 |
| Class E |  |  |  | -2.5 | -3.9 | -3.0 |
| All | -0.7 | -1.2 | -1.2 | -1.4 | -2.9 | -2.9 |
| Enc Time | 112% |  |  | 112% |  |  |
| Dec Time | 100% |  |  | 99% |  |  |

Table 3b: “AMP + Non-Square TU” result (low complexity)

|  | randomaccess_loco |  |  | lowdelay_loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BD-rate | Y | U | V | Y | U | V |
| Class A | -0.7 | -1.2 | -1.2 |  |  |  |
| Class B | -1.0 | -1.9 | -1.7 | -1.8 | -3.5 | -3.1 |
| Class C | -0.9 | -1.5 | -1.3 | -1.4 | -2.3 | -2.4 |
| Class D | -0.7 | -1.2 | -1.1 | -1.2 | -1.9 | -1.9 |
| Class E |  |  |  | -3.1 | -3.2 | -3.0 |
| All | -0.8 | -1.5 | -1.4 | -1.8 | -2.8 | -2.6 |
| Enc Time | 109% |  |  | 108% |  |  |
| Dec Time | 99% |  |  | 99% |  |  |

# Conclusion

In this contribution, AMP with OBMC and Non-Square TU is proposed. Compare to HM2.0 anchor, coding performance improvement is 1.0% for randomaccess and 2.0% for lowdelay in high efficiency case and 1.0% for randomacces and 2.2% for lowdelay in lowcomplexity case. The average improvement of coding efficiency against AMP + OBMC is 0.2% for randomaccess and 0.6% for lowdelay in high efficiency case and 0.3% for randomacces and 1.2% for lowdelay in lowcomplexity case. It is recommended to incorporate AMP with OBMC and Non-Square into HM.

# Reference

- JCT-VC, “Samsung’s Response to the Call for Proposals on Video Compression Technology”, JCTVC-A124, JCT-VC Meeting, Dresden, DE, April 2010.

- JCT-VC, “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, JCT-VC Meeting, Dresden, DE, April 2010.

- M. T. Orchard and G. J. Sullivan, “Overlapped block motion compensation: An estimation theoretic approach,” IEEE Trans. Image Processing, vol. 3, pp. 693–699, Sept. 1994.

- Frank Bossen, “Common test conditions and software reference configurations”, Doc. JCTVC-D600 (m19497), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- Min-Su Cheon, Il-Koo Kim (Samsung) , L. Guo, P. Chen, M. Karczewicz (Qualcomm), “CE2: Asymmetric motion partition with overlapped block motion compensation”, Doc. JCTVC-D367 (m19142), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, Korea, January 2011.

- X. Zheng, P. Bordes, P. Chen, I-K Kim, “Description of Core Experiment 2: Flexible Motion Partitioning,” Doc. JCTVC-E022, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, Switzerland, March 2011.

- Mathias Wien and Achim Dahlhoff, “Adaptive Block Transforms”, Doc.VCEG-M62, Q.6/SG16 (VCEG), Thirteenth Meeting: Austin, Texas, USA, 2-4 April, 2001

# Patent rights declaration(s)

Tsinghua University, Huawei Technologies, HiSilicon Technologies, Microsoft and University of Science and Technology of China may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).