| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E377 |
| --- | --- |

| Title: | Limiting Chroma Transform Depth in Residue Quad Tree (RQT) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Xianglin Wang Peisong Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In Dageu JCTVC meeting, it is reported (JCTVC-D060) that significant loss of chroma components is observed with Residue Quad Tree (RQT) coding. We suspected it is because the lack of flexibility of the current RQT structure. In current RQT, chroma is forced to have the the same tranform depth as Luma. Since chroma components are generally much smoother than luma, larger transform size is more desirable. In our contribution, we limit the chroma transform depth to be 0, i.e., a single large chroma transform for U(V) component of the whole CU. Chroma BD bit reduction is observed in most cases. On average for HE coding the BD bit reduction is 1% for U and 1.2% for V. We also observe slight Y improvement and encoding time reduction. It is recommended the current RQT scheme need to be further studied and flexibility should be introduced.

# Introduction

In Dageu JCTVC meeting, it is reported in JCTVC-D060 [1] that significant loss of chroma components is observed with RQT coding. One of the possible reasons is that in RQT structure, chroma transform is performed at the same depth as luma. However, chroma components generally are much smoother than luma components, and unnecessary overhead bits (cbp) may be introdued with small transform size. In this contribution, we limit chroma transform depth to be 0 in RQT. For a CU of size 2Nx2N, the transform size for U and V is fixed to be NxN.

# Solution

We modify RQT coding such that the chroma transform is performed at RQT depth 0. For a CU of size 2Nx2N, the transform size of U and V is always NxN (Table 1) .

| CU Size | 64x64 | 32x32 | 16x16 | 8x8 |
| --- | --- | --- | --- | --- |
| Chroma Transform Size | 32x32 | 16x16 | 8x8 | 4x4 |

Table 1 Chroma Transform Size and CU Size

# Experimental results

Our implementation is based on top of HM 2.0. The coding efficiency is evaluated under Common conditions specified in [2]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time is measured on cluster with the same CPU type, and the decoding time is measured by a single PC.

We observe that obvious bit rate reduction in HE cases (0.2% for Y, 1.0% for U and 1.2% for V). For LoCo case, there is only very minor change in the overall performance. Probably this is because in HE coding, max RQT depth is set to be 3, and chroms has been oversplit since it goes with luma. Limiting chroma transform to be at depth 0 can save unnessecary cbf bit when small transform size is used. In LoCo coding, the max depth is smaller (2 for Inter and 1 for intra), so the advantage from limiting chroma transform depth is not that big.

We also observe slight encoding time reduction (up to 6%). We think this is from the fact that the encoder do not need to test chroma tranform at depth >0.

|  | Y | U | V | Encoding Time | Decoding Time |
| --- | --- | --- | --- | --- | --- |
| Intra | -0.1 | -1.1 | -1.4 | 100% | 98% |
| RA | -0.2 | -0.7 | -1.1 | 94% | 100% |
| LD | -0.2 | -1.2 | -1.2 | 96% | 98% |
| HE Avg. | -0.2 | -1.0 | -1.2 | 97% | 98% |
| Intra LoCo | 0.0 | -0.2 | -0.2 | 99% | 100% |
| RA LoCo | -0.1 | 0.3 | 0.0 | 97% | 99% |
| LD LoCo | -0.3 | 0.3 | 0.3 | 94% | 100% |
| LoCo Avg | -0.1 | 0.1 | 0.0 | 97% | 100% |

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -1.3 | -2.3 | 0.0 | -0.3 | -0.7 |
| Class B | -0.1 | -1.3 | -1.6 | 0.0 | -0.1 | -0.1 |
| Class C | -0.1 | -0.6 | -0.7 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.3 | -0.4 | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -2.5 | -2.0 | 0.0 | -0.5 | -0.5 |
| All | -0.1 | -1.1 | -1.4 | 0.0 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -1.6 | -3.2 | -0.1 | 0.4 | 0.0 |
| Class B | -0.2 | -1.1 | -1.4 | -0.2 | 0.5 | -0.1 |
| Class C | -0.2 | -0.1 | 0.0 | -0.2 | 0.4 | 0.3 |
| Class D | -0.1 | 0.2 | 0.2 | -0.1 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.7 | -1.1 | -0.1 | 0.3 | 0.0 |
| Enc Time[%] | 94% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -1.2 | -2.0 | -0.4 | 1.1 | 0.5 |
| Class C | -0.2 | -0.1 | 0.0 | -0.4 | 1.3 | 0.8 |
| Class D | -0.3 | 0.9 | 0.3 | -0.2 | 0.5 | 0.6 |
| Class E | -0.2 | -5.4 | -3.5 | -0.2 | -2.4 | -0.8 |
| All | -0.2 | -1.2 | -1.2 | -0.3 | 0.3 | 0.3 |
| Enc Time[%] | 96% |  |  | 94% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Conclusion

In this contribution, we limit chroma transform depth to be 0 and test the performance. Coding efficiency is observed in most of the test cases. We recommend the current RQT structure should be further studied and flexibility should be introduced.

# References

- M. Zhou and A. Tabatabai, “Evaluation Results on Residual Quad Tree (RQT)”, JCTVC-D060, JCT-VC Meeting, Daegu, KR, Jan. 2011.

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC Meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).