| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E377 |
| --- | --- |

| Title: | Limiting Chroma Transform Depth in Residue Quad Tree (RQT) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Xianglin Wang Peisong Chen Ying Chen Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In Dageu JCTVC meeting, it is reported (JCTVC-D060) that significant loss of chroma components is observed with Residue Quad Tree (RQT) coding. We suspected it is because the lack of flexibility in the current RQT structure. In this contribution, maximum chroma transform depth is allowed to be different from maximum luma transform depth. Chroma BD bit reduction is observed in most cases. On average for HE coding the BD bit reduction is 1% for U and 1.2% for V. We also observe slight Y improvement and encoding time reduction. It is recommended the current RQT scheme be further studied and more flexibility be introduced.

# Introduction

In Dageu JCTVC meeting, it is reported in JCTVC-D060 [1] that significant loss of chroma components is observed with RQT coding. One of the possible reasons is that in RQT structure, chroma is forced to have the same transform depth as luma. However, chroma components generally are much smoother than luma components, and unnecessary overhead bits (cbp) may be introduced with small transform size. In this contribution, we allow maximum chroma transform depth to be different from maximum luma transform depth such that chroma transform can be performed at a lower depth to save overhead bits.

# Methods

In current HM, both luma and chroma have the same maximum transform depth, which is 3 for HE Inter/Intra, 2 for LC Inter and 1 for LC Intra in the default setting. To avoid oversplit of chroma component in RQT, we limit the maximum chroma transform depth. Let MaxDepthY be the maximum luma transform depth and MaxDepthUV be the maximum chroma transform depth, MaxDepthUV = max ( 0,  MaxDepthY - T), where T is a constant to control the difference between luma and chroma maximum transform depth. As an example, if we choose T=3, MaxDepthUV in HM default settings is 0. In this case, For a CU of size 2Nx2N, the transform size of U and V is always NxN (Table 1) .

| CU Size | 64x64 | 32x32 | 16x16 | 8x8 |
| --- | --- | --- | --- | --- |
| Chroma Transform Size | 32x32 | 16x16 | 8x8 | 4x4 |

Table 1 Chroma Transform Size and CU Size (T=3)

# Experimental results

Our implementation is based on top of HM 2.0, and we test the performance in the case of T = 3. The experiment is performed under common conditions specified in [2]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time is measured on cluster with the same CPU type, and the decoding time is measured on a single PC.

Bit rate reduction in HE cases is observed (0.2% for Y, 1.0% for U and 1.2% for V). For LoCo case, there is only very minor change in the overall performance. Probably this is because in HM2.0 HE coding, maximum RQT depth is set to be 3, and chroma may be well oversplit since it goes with luma. Limiting maximum chroma transform depth can save unnecessary cbf bit associated with small transform size. In LoCo coding, the max depth is smaller (2 for Inter and 1 for intra), so the advantage from limiting chroma transform depth is not that big.

We also observe slight encoding time reduction (up to 6%). We think this is from the fact that the encoder do not need to test chroma transform at depth >0.

|  | Y | U | V | Encoding Time | Decoding Time |
| --- | --- | --- | --- | --- | --- |
| Intra | -0.1 | -1.1 | -1.4 | 100% | 98% |
| RA | -0.2 | -0.7 | -1.1 | 94% | 100% |
| LD | -0.2 | -1.2 | -1.2 | 96% | 98% |
| HE Avg. | -0.2 | -1.0 | -1.2 | 97% | 98% |
| Intra LoCo | 0.0 | -0.2 | -0.2 | 99% | 100% |
| RA LoCo | -0.1 | 0.3 | 0.0 | 97% | 99% |
| LD LoCo | -0.3 | 0.3 | 0.3 | 94% | 100% |
| LoCo Avg. | -0.1 | 0.1 | 0.0 | 97% | 100% |

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -1.3 | -2.3 | 0.0 | -0.3 | -0.7 |
| Class B | -0.1 | -1.3 | -1.6 | 0.0 | -0.1 | -0.1 |
| Class C | -0.1 | -0.6 | -0.7 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.3 | -0.4 | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -2.5 | -2.0 | 0.0 | -0.5 | -0.5 |
| All | -0.1 | -1.1 | -1.4 | 0.0 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | #NUM! |  |  | 100% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -1.6 | -3.2 | -0.1 | 0.4 | 0.0 |
| Class B | -0.2 | -1.1 | -1.4 | -0.2 | 0.5 | -0.1 |
| Class C | -0.2 | -0.1 | 0.0 | -0.2 | 0.4 | 0.3 |
| Class D | -0.1 | 0.2 | 0.2 | -0.1 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.7 | -1.1 | -0.1 | 0.3 | 0.0 |
| Enc Time[%] | 94% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -1.2 | -2.0 | -0.4 | 1.1 | 0.5 |
| Class C | -0.2 | -0.1 | 0.0 | -0.4 | 1.3 | 0.8 |
| Class D | -0.3 | 0.9 | 0.3 | -0.2 | 0.5 | 0.6 |
| Class E | -0.2 | -5.4 | -3.5 | -0.2 | -2.4 | -0.8 |
| All | -0.2 | -1.2 | -1.2 | -0.3 | 0.3 | 0.3 |
| Enc Time[%] | 96% |  |  | 94% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |

# Conclusion

In this contribution, we limit chroma transform depth in RQT scheme. Coding efficiency is observed in most of the test cases. We recommend the current RQT structure should be further studied and flexibility should be introduced.

# References

- M. Zhou and A. Tabatabai, “Evaluation Results on Residual Quad Tree (RQT)”, JCTVC-D060, JCT-VC Meeting, Daegu, KR, Jan. 2011.

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC Meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).