| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E379 |
| --- | --- |

| Title: | CE7: Cross-verification for Panasonic’s test (JCTVC-E074) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina Ankur Saxena Woo-Jin Han | Email: | elena_a.alshina@samsung.com a.saxena1@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This is cross-verification test results for second-step Intra mode dependent transform proposed by Panasonic. In the test, the BD-rates are matched with those in JCTVC-E074. The average gain in all Intra test is 1.1% / 1.7% for HE/LC case correspondently. For random access gain evaluated tool provides 0.5%/0.7% gain (HE/LC). Encoding time overhead was no more than 4%, decoding time overhead is no more than 6% compare to anchor.

# Introduction

In this document, the cross-verification results on second-step Intra mode dependent transform proposed by Panasonic [1]. For this cross-verification, the source code provided by Panasonic was investigated and run.

In JCTVC-D151, Panasonic proposed second-step mode-dependent transform. For smallest transform unit, this technique replaces core transform, but for larger block sizes, it is applied for only low frequency part of core transform coefficients matrix. Second step transform is applied for all block sizes, for Intra residual of Luma component only.

Alternative transform from Panasonic always deal with 4x4 inputs and implemented in matrix multiplication form.

Four different 4x4 transforms are used on second-step according to Intra prediction direction. So no additional signaling about which transform should be used is needed.

# Results

The verification tests were done under test conditions agreed to use in CE7 [2]. Four test cases: AI-HE, AI-LC, RA-HE, RA-LC are investigated. Test results are summarized in Table 1.

The average gain in all Intra test is 1.1% / 1.7% for HE/LC case correspondently. For random access gain evaluated tool provides 0.5%/0.7% gain (HE/LC). Encoding time overhead was no more than 4%, decoding time overhead is no more than 6% compare to anchor. All these numbers are still on noise level because the measurement of running time is rather non-accurate.

Table 1 JCTVC-E074 tool test vs HM2.0.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.3 | -1.2 | -1.6 | -0.5 | -0.4 |
| Class B | -0.9 | -1.0 | -1.1 | -1.5 | -0.7 | -0.7 |
| Class C | -1.1 | -1.2 | -1.1 | -1.7 | -1.0 | -1.0 |
| Class D | -1.1 | -1.0 | -1.0 | -1.6 | -0.8 | -0.8 |
| Class E | -1.7 | -2.2 | -2.0 | -2.4 | -1.0 | -1.3 |
| All | -1.1 | -1.3 | -1.2 | -1.7 | -0.8 | -0.8 |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.3 | -0.5 | -0.7 | -0.2 | 0.1 |
| Class B | -0.4 | -0.4 | -0.4 | -0.7 | -0.2 | -0.2 |
| Class C | -0.6 | -0.6 | -0.5 | -0.8 | -0.5 | -0.4 |
| Class D | -0.5 | -0.4 | -0.3 | -0.7 | 0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.4 | -0.7 | -0.2 | -0.2 |
| Enc Time[%] | 103% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |

# Conclusions

Test results we obtained totally match with proponent’s data. Proposed by Panasonic, this tool shows interesting perspective of extending a mode-dependent transform for larger than 4x4 and 8x8 TU sizes, without significant increase in computational complexity and memory usage.

# References

- JCT-VC, “CE7: Mode Dependent 2-step Transform for Intra Coding”, JCTVC-E074, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011

- JCT-VC, “Description of Core Experiment 7 (CE7): Alternative Transforms”, JCTVC-D607, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011