| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E380 |
| --- | --- |

| Title: | CE7: Experimental results of ROT by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | CE response |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin Felix C. Fernandes Ankur Saxena Vadim Seregin Zhan Ma Woo-Jin Han | Email: | elena_a.alshina@samsung.com alexander_b.alshin@samsung.com felix.f@sta.samsung.com a.saxena1@samsung.com v.seregin@samsung.com zhan.ma@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution provides the simulation results when the Rotational transform (ROT) was ported and enabled in HM2.0 against the HM2.0 anchor results. The ROT is used for Intra residual of Luma component only. For All-Intra (AI) test case, the ROT shows 1.7% (HE) / 2.4% (LC) performance gain. For Random-Access (RA) test case, the ROT shows 1.0% (HE) / 1.4% (LC) performance gain. This contribution also shows that a lifting implementation for ROT provides almost the same performance as the preceding matrix-multiplication implementation.

Additionally, a combined test with both ROT and mode-dependent DCT/DST transform enabled for intra coding was done. For AI test case, this combination shows 1.9% (HE) / 3.0% (LC) improvement, and for RA scenario, 1.0% (HE) / 1.5% (LC) performance gain.

As tradeoff between complexity and performance the combination of mode dependent transform and restricted ROT was tested. This combination shows 2% average gain in AI (with 25% encoding time overhead) and 1% average gain in RA (with 5% encoding time overhead).

- Introduction

It is well known that the DCT basis functions are sub-optimal for some types of residuals. If the predicted signal deviates substantially from the original, then the residual is not smooth and the DCT does not compact energy well. In this case, an alternative transform improves performance.

One group of alternative transforms is the intra-direction dependent transforms. Among them, the DCT/DST mode-dependent combination [1] shows an excellent trade-off between memory usage and performance improvement. Such a mode-dependent transform does not require additional signaling or encoder search complexity.

Another group of alternative transforms is the Rotational Transform (ROT) [2] which is a low-complexity second-step transform adaptively chosen at the encoder.

This proposal is devoted to testing ROT performance and the combination of ROT with DCT/DST.

- Algorithm description

The ROT does not replace the main transform core. It is applied as a secondary transform for only the low frequency part of transform coefficients matrix (Fig. 1).

Fig 1. ROT application on decoder side.

For 4x4 blocks, the 4x4 inverse ROT (invROT4) is applied. In all other cases, the 8x8 inverse ROT (invROT8) is applied to the 8x8 upper-left square of transform coefficients matrix.

So the input of invROT4 is a 4x4 square matrix with transform coefficients after de-quantization. The input of invROT8 is an 8x8 square matrix with low-frequency part transform coefficients after de-quantization

Mathematically, the ROT is 2D separable transform. Therefore, the forward 4x4 ROT (ROT4) is matrix left-hand side multiplication of input matrix with “matrix of horizontal rotation” (RHorizontal) and then right-hand side multiplication with “matrix of vertical rotation” (RVertical).

For the ROT4, these mathematical operations are represented in Fig 2. Shadowed squares indicate non-zero elements of RHorizontal and RVertical.

1

1





=

RHorizontal              input               RVertical         =    rotated output

Fig. 2. The ROT4 process.

Taking into account the a-priori, known, structure of the ROT matrix (some elements of ROT matrices are zeros and one non-zero element is equal to 1) the total number of operations for the ROT is essentially smaller than for two matrix multiplications. The 8x8 ROT process is similar to the 4x4 process and has been described in [3].

The number of operations for ROT process is summarized in Table 1. The dictionary of ROT transforms contains 1 trivial and 4 non-trivial transforms. The trivial ROT transform is the zero matrix, for which no secondary transform is applied. Therefore, no additional operations are needed for the trivial transform. All non-trivial rotational transforms involve matrix multiplication with sparse matrices. In Table 1, we show average number of operations per pixel, assuming that 1 trivial and 4 non-trivial rotational transforms are used with equal probability.

Table 1 Number of operations per pixel for ROT.

|  | Core HM transform |  |  | Additional operations for ROT |  |  | Total |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TU size | “*” | “+” | “>>” | “*” | “+” | “>>” | “*” | “+” | “>>” |
| 4x4 | 0 | 5 | 2 | 3.6 | 3.2 | 1.6 | 3.6 | 8.2 | 3.6 |
| 8x8 | 0 | 10 | 5.5 | 3.6 | 3.2 | 1.6 | 3.6 | 13.2 | 7.1 |
| 16x16 | 5.5 | 10.3 | 6.3 | 0.9 | 0.8 | 0.4 | 5.9 | 11.1 | 6.7 |
| 32x32 | 7.3 | 13.1 | 7.1 | 0.2 | 0.2 | 0.1 | 7.5 | 13.3 | 7.2 |

Number of operations in Table 1 shows that the ROT introduces moderate additional complexity into the transform process. For 4x4 blocks, ROT complexity is comparable with the core transform. For 8x8 case, the number of ROT operations is less than half that of the core transform, and for larger blocks, the ROT complexity is negligible compared to the core transform.

It was agreed among CE7 participants to use special template for number of operations counting. According to this table Core transform in HM2.0 uses 6.00 operations per pixel (averaged across all TU sizes). Additional operations number needed for ROT is 2.08.

The Rotational transform was designed to allow partial exchange of energy between columns/rows of transform coefficients matrix in order to get better energy compaction and minimize quantization error. Partial energy exchange between columns / rows is controlled by angle parameters k (1).

(1)

For example, if we set  1=/2, 2=0, 3=0, then RVertical will  switch the Columns 1 and 2 of the input matrix  with change of sign of the 2nd column. By setting 1=, 2=0, 3=0, one gets RVertical  which will change the sign of all elements. If 1=2=3=0, then the ROT does not change the input.

Because we assume that the main transform is close to optimal and only a small modification of transform coefficients matrix is needed, the parameters k in (1) were chosen to be near 0 or . This corresponds to a small absolute value change with possible sign change. From (1) it is clear that the ROT is an orthogonal transform. Therefore Parseval’s theorem guarantees that total transform-coefficients energy after the ROT is preserved.

Exact values of k are not important because in hardware and software implementations, both RVertical and RHorizontal are pre-calculated, scaled and rounded to integer. The set of k was not changed during this CE. Four sets of non-trivial k provide 4 non-trivial ROT transforms. We also define the “zero” ROT, in which trivially, k=0, so that the ROT is effectively skipped and no transform-coefficients are changed after the DCT. These 5 ROT transforms constitute the dictionary of alternative transforms for this CE. One of these possibilities is chosen at the encoder and signaled to the decoder explicitly by encoding indexROT = 0,…,4 for each TU.

If the ROT is applied, then no modification of main transform core or quantization part is needed because the ROT re-uses the same quantization matrices that are already implemented in HM.

The entire ROT dictionary with 4 non-trivial alternative transforms requires storage of 72 additional integers for invROT4 and 72 additional integers for invROT8. So in total, 144 integers with absolute values less than 256 should be stored. Total additional memory usage for ROT is 144 bytes only. This is sufficient for 4 alternative transforms applied for arbitrary TU sizes (from 44 up to TUsizeMax TUsizeMax) and all QP. For comparison, the de-quantization matrix for only 44 and 8x8 core transforms in HM contains 480 elements. Therefore, the memory usage by ROT compared to other alternative transforms and even the main transform core in HM is relatively small.

The ROT is applied for intra residuals only.

At the encoder, all members of the ROT dictionary are tried and the best transform for the current TU is chosen. In an initial realization of ROT [2], the entire TU encoding process was repeated inside the ROT search loop: prediction, forward main transform, forward ROT, quantization, de-quantization, inverse ROT, and inverse main transform (7 steps in total). The best ROT transform was decided based on TU Rate-Distortion (RD) cost.

Because the ROT is an orthogonal transform, the distortion can be estimated in frequency domain using Parseval's identity. We exploit this advantage in RDOQ to reduce computational complexity: the best quantization is decided using distortion estimation in frequency domain.  For the test in this CE, the ROT encoder search process was re-designed and merged with RDOQ. In the resulting fast ROT version, only 2 steps are repeated within the ROT search loop: forward ROT and quantization.

The ROT is independent of the core transform, so no changes of core transform or quantization are needed.

- Test 1: ROT (matrix multiplication form) performance in HM2.0.

The ROT was implemented in HM2.0 and tested using encoder and decoder configurations as described in JCTVC-D600 [3]. No additional change (for example, scan modification) which typically has synergy with alternative transforms was enabled during this test.

Test results are summarized in Table 2. In the Intra-Only scenario, the ROT shows about 1.7% gain for HE and about 2.4% gain for LC configuration. For the Random-Access scenario, the ROT gain is over 1% on average. Note that for this scenario, the ROT is applied for intra residuals and for Luma component only.

We would like to thank Qualcomm for verification of these test results. We show encoding/decoding time measurement provided by cross-checker on Table 2.

Table 2. ROT performance test results.

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | -1.7 |  | 0.1 |  | 0.1 | -2.7 |  | 1.2 | 1.1 |
| Class B | -1.7 |  | 1.1 |  | 1.0 | -2.3 |  | 1.8 | 1.7 |
| Class C | -1.7 |  | 1.6 |  | 1.5 | -2.3 |  | 1.8 | 1.8 |
| Class D | -1.8 |  | 1.6 |  | 1.7 | -2.5 |  | 1.9 | 1.9 |
| Class E | -1.5 |  | 0.3 |  | 0.3 | -2.2 |  | 1.1 | 1.1 |
| All | -1.7 |  | 1.0 |  | 0.9 | -2.4 |  | 1.6 | 1.5 |
| Enc Time[%] | 147% |  |  |  |  | 184% |  |  |  |
| Dec Time[%] | 105% |  |  |  |  | 104% |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0.7 | 0.2 |  | 0.0 |  | -1.3 | 1.3 |  | 1.2 |
| Class B | -1.0 | 0.8 |  | 1.0 |  | -1.5 | 1.0 |  | 1.1 |
| Class C | -1.2 | 0.9 |  | 0.7 |  | -1.4 | 1.0 |  | 0.9 |
| Class D | -1.1 | 0.7 |  | 0.7 |  | -1.3 | 0.9 |  | 0.7 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -1.0 | 0.7 |  | 0.6 |  | -1.4 | 1.0 |  | 1.0 |
| Enc Time[%] | 104% |  |  |  |  | 104% |  |  |  |
| Dec Time[%] | 103% |  |  |  |  | 100% |  |  |  |

Application of the ROT typically improves Luma PSNR by 0.15dB. There is no Chroma PSNR change due to the ROT. Total bit usage increases slightly because of indexROT signaling. Because total bits and individual Chroma PSNR are used to compute the Chroma-component BD rate, this measure formally shows Chroma performance drop. But this is a known problem of the chosen performance measure that  is not fairly adequate for Chroma components.

Decoding time increment due to ROT is on a noise level (about 5%). Encoding time increment in RA test scenario is negligible. Encoder complexity in Intra only test is 1.5 and 1.8 for HE and LC cases, respectively.

Each of 5 passes of ROT search loop is independent and can be performed in parallel. So encoder complexity of ROT can be significantly reduced by parallel implementation. For parallelized ROT loop we expect encoding time overhead to be x1.1/x1.2 instead 1.6 and 2.0, as described in [3].

- Test 2: ROT (lifting form) performance in HM2.0.

Additional ROT complexity reduction is possible if lifting [3] is applied. In this CE, the ROT lifting implementation was tested. With lifting, significant hardware complexity reduction is possible, as explained here briefly. In a lifting factorization of the ROT, each ROT matrix in the dictionary is first factored into Givens rotation matrices. These Givens rotation matrices are then each replaced by a triplet of integer lifting matrices. While retaining overall ROT transform invertibility, the multipliers in each lifting matrix can be approximated so that fewer hardware elemental adders are required in a ROT implementation. With almost no performance drop, the lifting factorization obtained 46% and 21% reduction in elemental-adder count of forward and inverse ROT implementations, respectively.

In [3], the lifting implementation was applied to 8x8 and larger block sizes. In this CE, the lifting implementation is applied to all block sizes. However, to demonstrate that performance gains are possible with reduced storage costs, we use the same lifting multipliers for all block sizes. Specifically, for the lifting implementation, the non-zero 3x3 submatrix for invROT4 in (1) is re-used twice for each of the larger block sizes. Note that in Test 1, a particular non-zero 3x3 submatrix is used for the 4x4 ROT and a different submatrix is re-used twice for each of the larger block sizes. Therefore, this lifting implementation requires additional storage of 108 bytes compared to the 144 bytes required in Test 2.

Test results are summarized in Table 3.

Table 3. ROT (with lifting) performance test results.

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | -1.6 |  | 0.1 |  | 0.2 | -2.5 |  | 1.1 | 1.0 |
| Class B | -1.6 |  | 1.1 |  | 0.9 | -2.2 |  | 1.7 | 1.6 |
| Class C | -1.6 |  | 1.4 |  | 1.5 | -2.2 |  | 1.7 | 1.7 |
| Class D | -1.7 |  | 1.6 |  | 1.6 | -2.3 |  | 1.8 | 1.8 |
| Class E | -1.4 |  | 0.3 |  | 0.3 | -2.1 |  | 1.0 | 1.1 |
| All | -1.6 |  | 0.9 |  | 0.9 | -2.3 |  | 1.5 | 1.5 |
| Enc Time[%] | 163% |  |  |  |  | 204% |  |  |  |
| Dec Time[%] | 106% |  |  |  |  | 110% |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0.7 | -0.1 |  | 0.1 |  | -1.1 | 1.4 |  | 1.4 |
| Class B | -0.9 | 0.7 |  | 0.7 |  | -1.4 | 1.1 |  | 1.1 |
| Class C | -1.1 | 0.8 |  | 0.8 |  | -1.4 | 0.8 |  | 1.0 |
| Class D | -0.9 | 0.5 |  | 0.5 |  | -1.2 | 0.9 |  | 0.7 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.9 | 0.5 |  | 0.6 |  | -1.3 | 1.1 |  | 1.0 |
| Enc Time[%] | 109% |  |  |  |  | 106% |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 101% |  |  |  |

We would like to thank Panasonic for verification of these test results (we show time measurement provide by the cross-checker here).

Compared to matrix-multiplication form (Test 1), the ROT lifting implementation shows almost the same performance. The small difference occurs due to the reduced storage overhead of the lifting implementation. This proves that matrix multiplication and lifting implementation are almost equivalent in terms of performance.

- Test 3: Joint-test ROT and DCT/DST (extreme performance)

In the preceding tests, the ROT does not depend on main transform core. Here we combine the ROT with the DCT/DST [4]. The DCT/DST is a mode-dependent transform modification. The Main transform core in HM is a 2D separable DCT-like transform. In DCT/DST algorithm one of the 1D transforms is replaced by DST according to the intra prediction direction. The DCT/DST was applied for 4x4 and 8x8 intra-residual blocks in this test. Performance data for this test is summarized in Table 4.

Table 4. ROT (matrix multiplication) combined with DCT/DST performance test results.

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | -2.3 |  | -1.0 |  | -0.8 | -3.8 |  | 0.7 | 0.7 |
| Class B | -1.7 |  | 0.4 |  | 0.2 | -2.6 |  | 1.0 | 0.9 |
| Class C | -1.8 |  | 0.7 |  | 0.7 | -2.8 |  | 1.0 | 1.0 |
| Class D | -2.0 |  | 0.9 |  | 0.9 | -3.0 |  | 1.2 | 1.2 |
| Class E | -1.6 |  | -0.8 |  | -0.6 | -2.9 |  | 0.3 | 0.3 |
| All | -1.9 |  | 0.1 |  | 0.1 | -3.0 |  | 0.9 | 0.9 |
| Enc Time[%] | 157% |  |  |  |  | 198% |  |  |  |
| Dec Time[%] | 105% |  |  |  |  | 110% |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | -0.8 | -0.5 |  | -0.4 |  | -1.5 | 1.1 |  | 1.1 |
| Class B | -0.9 | 0.4 |  | 0.6 |  | -1.4 | 0.9 |  | 0.8 |
| Class C | -1.3 | 0.5 |  | 0.5 |  | -1.6 | 0.6 |  | 0.8 |
| Class D | -1.1 | 0.3 |  | 0.4 |  | -1.4 | 0.7 |  | 0.5 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -1.0 | 0.2 |  | 0.3 |  | -1.5 | 0.8 |  | 0.8 |
| Enc Time[%] | 108% |  |  |  |  | 106% |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 101% |  |  |  |

We would like to thank Panasonic for verification of these test results (we show time measurement provide by the cross-checker here).

Together, DCT/DST and ROT provide almost 2% gain in AI-HE and 3% in AI-LC. Performance improvement in RA test case is 1%(HE) and 1.5% (LC). Decoding time overhead is small. Only in the AI_LC case this overhead reaches 10%, but taking into account at least 5% error in time measurement, this is not critical.

- Test 4: Joint test ROT and DCT/DST (trade-off).

We’ve mentioned above that encoding time in the All Intra test case can be significantly reduced by parallelization. Because the HM doesn’t support parallelization, in this test, we demonstrate an attractive ROT performance/complexity tradeoff by restricting the search space. In HM, intra coding the most probable intra-prediction direction is determined from left and above blocks. Only 1 bit signals intra direction in the current block is equal to the most probable direction (MPM). If MPM is chosen, there is a higher chance that the mode was selected because signaling overhead is small not because the chosen mode is optimal for current block.  In this case it is reasonable to expect improved performance using an alternative transform. In this test, at the encoder, the ROT was tried only for intra-prediction direction  equal to most probable mode. Otherwise, a ROT search loop was performed at the encoder and there is no need to signal the ROTindex. The DCT/DST was applied for 4x4 and 8x8 blocks without restriction.

Performance data for this test is summarized in Table 5.

Table 5. ROT (restricted for MPM only) combined with DCT/DST performance test results

|  | Intra |  |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -1.7 |  |  | -1.3 |  | -1.1 | -2.9 |  | 0.0 |  | 0,1 |
| Class B | -1.3 |  |  | -0.4 |  | -0.4 | -2.0 |  | -0.2 |  | -0,1 |
| Class C | -1.3 |  |  | -0.3 |  | -0.3 | -2.1 |  | -0.4 |  | -0,4 |
| Class D | -1.4 |  |  | -0.2 |  | -0.2 | -2.1 |  | -0.3 |  | -0,3 |
| Class E | -1.3 |  |  | -1.4 |  | -1.2 | -2.5 |  | -0.3 |  | -0,5 |
| All | -1.4 |  |  | -0.7 |  | -0.6 | -2.3 |  | -0.2 |  | -0,2 |
| Enc Time[%] | 123% |  |  |  |  |  | 130% |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |  | 105% |  |  |  |  |
|  | Random access |  |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate |  |  | U BD-rate | V BD-rate |
| Class A | -0.7 |  | -0.3 |  | -0.3 |  | -1.1 |  |  | 0.6 | 0,6 |
| Class B | -0.6 |  | 0.2 |  | 0.3 |  | -1.0 |  |  | 0.3 | -0,2 |
| Class C | -0.9 |  | -0.1 |  | -0.1 |  | -1.2 |  |  | -0.1 | 0,2 |
| Class D | -0.7 |  | -0.1 |  | 0.1 |  | -1.0 |  |  | -0.1 | -0,1 |
| Class E |  |  |  |  |  |  |  |  |  |  | 0,1 |
| All | -0.7 |  | 0.0 |  | 0.0 |  | -1.0 |  |  | 0.2 | 0,6 |
| Enc Time[%] | 104% |  |  |  |  |  | 102% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  | 100% |  |  |  |  |

We would like to thank Panasonic for verification of these test results (we show time measurement provide by the cross-checker here).

This version shows only 20-30% encoding time increment compared to the anchor. The same tool combination but with non-restricted ROT has 55% (HE)/95%(LC) encoding time overhead.

The performance gain provided by this tool combination is 1.9% (in average for All Intra test) and 0.9% (in average for Random access test case)..

- Test 5: Joint test ROT and DCT/DST and 2nd step Mode Dependent Transform (our recommendation).

Performance of this tool combination can be further improved by extension of mode dependent transform for large block sizes using second-step mode dependent transform from Panasonic [5]. DCT/DST in [4] implemented for small block sizes 4x4 and 8x8. Further extension of full-size mode dependent transform will lead to noticeable (~10%) increase of decoding complexity. Second order mode dependent transform [5] modifies only small (4x4) part of transform coefficients matrix(similar to ROT) according to Intra prediction direction (similar to DCT/DST).

With permission of Panasonic we use second step mode dependent transform for 16x16 and 32x32 intra TU additionally to DCT/DST in 4x4 and 8x8 blocks in our last test. ROT is applied for all block sizes but restricted to most probable prediction only (the same with Test 4).

In order to avoid cascade application of several tools we use Mode Dependent Transforms for indexROT=0 only (this means no ROT). If indexROT>0 then no Mode Dependent transform modifications are used. Mode dependent transforms in this test are separated by block size: DCT/DST for 4x4 and 8x8 and 2nd step transform for 16x6 and 32x32.

For each TU only one alternative transform tool is applied.

Performance data for this test is summarized in Table 6.

Table 6. ROT (restricted for MPM only) combined with Mode dependent transforms (DCT/DST  and 2nd step transform) performance test results

|  | Intra |  |  |  |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | -1,8 |  | -1,5 |  | -1,3 |  | -3,0 |  | -0,2 | -0,1 |
| Class B | -1,5 |  | -0,4 |  | -0,5 |  | -2,2 |  | -0,3 | -0,3 |
| Class C | -1,5 |  | -0,3 |  | -0,3 |  | -2,3 |  | -0,4 | -0,4 |
| Class D | -1,5 |  | -0,2 |  | -0,2 |  | -2,2 |  | -0,3 | -0,3 |
| Class E | -1,7 |  | -1,8 |  | -1,6 |  | -2,8 |  | -0,7 | -0,9 |
| All | -1,6 |  | -0,8 |  | -0,7 |  | -2,5 |  | -0,4 | -0,4 |
| Enc Time[%] | 125% |  |  |  |  |  | 134% |  |  |  |
| Dec Time[%] | 104% |  |  |  |  |  | 107% |  |  |  |
|  | Random access |  |  |  |  |  | Random access LoCo |  |  |  |
|  | Y BD-rate |  | U BD-rate |  |  | V BD-rate | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | #VALUE! |  | #VALUE! |  |  | #VALUE! | -1,2 | 0,3 |  | 0,3 |
| Class B | #VALUE! |  | #VALUE! |  |  | #VALUE! | -1,2 | 0,1 |  | 0,0 |
| Class C | -1,0 |  | -0,1 |  |  | -0,1 | -1,3 | -0,2 |  | 0,0 |
| Class D | -0,8 |  | -0,1 |  |  | 0,0 | -1,0 | 0,1 |  | -0,1 |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | #VALUE! |  | #VALUE! |  |  | #VALUE! | -1,2 | 0,1 |  | 0,0 |
| Enc Time[%] | #NUM! |  |  |  |  |  | 104% |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |  |  | 101% |  |  |  |

We would like to thank Panasonic again for verification of these test results (we show time measurement provide by the cross-checker here).

This version shows 30% encoding time increment compared to the anchor in All Intra test. About only 4% encoding time increase in RA case. Decoding time overhead is no more than 7%.

The performance gain provided by this tool combination is 2% (in average for All Intra test) and 1% (in average for Random access test case).

- Conclusions

Based on these test results

2% (in average for All Intra test) and 1% (in average for Random access test) performance improvement

we recommend HM adoption of following alternative transform design:

- DCT/DST for 4x4 and 8x8

- 2nd step mode dependent transform for 16x16 and 32x32

- ROT for all block sizes restricted for MPM and no mode dependent transforms.

Only one alternative transform tool is applied for each Intra TU in this design.

- References

- A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST for intra prediction in video coding," JCTVC-D033, Daegu, Korea, January 2011.

- Ken McCann, Woo-Jin Han and Il-Koo Kim, “Samsung’s Response to the Call for Proposals on Video Compression Technology,” JCTVC-A124, Dresden, Germany, April 2010.

- F. C. Fernandes, “Low-Complexity Rotational Transform”, JCTVC-C096, Guangzhou, October 2010.

- A. Saxena and F. C. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction”, JCTVC-E074, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011.

- Youji Shibahara , Takahiro Nishi “CE7: Mode Dependent 2-step Transform for Intra Coding”, JCTVC-E074, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).