| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E386 |
| --- | --- |

| Title: | IDCT pruning |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Email: | madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

High frequency region of large transforms is typically zero due to quantization and energy compaction properties of transform. This contribution presents non-zero low frequency sub-block statistics of large transform blocks in anchor bitstreams of HM 2.0. The non-zero sub-block information can be used to carry out IDCT pruning where in the IDCT computations that have zero input and zero output can be eliminated. This contribution asserts that IDCT pruning is a useful technique that can be used to reduce SIMD computational complexity based on source statistics in decoder. It is also asserted to result in corresponding power savings in hardware transform engines. Complexity of pruned IDCT for HM 2.0 transform and transform implemented using partial butterfly (JCTVC-E243) are presented to demonstrate complexity reduction from IDCT pruning. This contribution recommends that the pruning behavior of large transforms be considered in design of HEVC transforms.

# Introduction

High frequency region of large transforms is typically zero due to quantization and energy compaction properties of transform. This contribution presents non-zero low frequency sub-block statistics of large transform blocks in anchor bitstreams of HM 2.0. The non-zero sub-block information can be used to carry out IDCT pruning where in the IDCT computations that have zero input and zero output can be eliminated. This was highlighted during discussions in core transform break-out activity in Daegu [1].

In this contribution, IDCT pruning is asserted to be a useful technique that can be used to reduce SIMD computational complexity based on source statistics. It is also asserted to result in corresponding power savings in hardware transform engines. Complexity of pruned IDCT for HM 2.0 transform and transform implemented using partial butterfly (JCTVC-E243) are presented to demonstrate complexity reduction from IDCT pruning. This contribution recommends that the pruning behavior of large transforms be considered in design of HEVC transforms.

# Non-zero block statistics

Due to quantization and energy compaction properties of transform, the high frequency region of large transforms is typically zero. The occurrence of following non-zero sub-blocks (See Figure 1) was measured. Let LK denote a block that has only the lowest frequency KxK sub-block to be non-zero.

- 16x16 Transform: L4, L8, L16

- 32x32 Transform: L4, L8, L16, L32

Table below shows the distribution of non-zero sub-blocks. It can be seen from the table that majority of large transform blocks have non-zero coefficients in quadrant around DC value (which is Number of L4+L8 for 16x16 and Number of L4+L8+L16 for 32x32 transform blocks in Table 1).

Table 1: Distribution of non-zero sub-blocks.

| 16x16 Transform |  |  |
| --- | --- | --- |
|  | High efficiency | Low complexity |
| Number of L4 | 59% | 64% |
| Number of L8 | 18% | 29% |
| Number of L16 | 23% | 7% |
| Number of L4+L8 | 77% | 93% |
| 32x32 Transform |  |  |
| Number of L4 | 44% | 61% |
| Number of L8 | 15% | 29% |
| Number of L16 | 17% | 10% |
| Number of L32 | 24% | 0% |
| Number of L4+L8+L16 | 76% | 100% |

# Complexity implications

IDCT pruning can be successfully used to reduce complexity for blocks that have a majority of block region to be zero. 2D IDCT of an NxN block requires 2N 1D IDCTs. Figure 2 shows an example where N/2 x N/2 region is non-zero. For this example, only N/2 transforms need to be carried out along rows. Total number of 1D DCTs for this example is N/2+N = 1.5N, a 25% savings in complexity.

A further savings in complexity can be achieved by pruning within a 1D transform as shown in Figure 3. In Figure 3, matrix multiplication implementation of IDCT is used as an example to illustrate pruning within a 1D transform. For the example shown in Figure 3, only ¼ the number of multiplication are required if multiplication by 0 is eliminated.

The overall number of multiplications from Figure 2 and Figure 3 would be:

Horizontal transform: N/2 * N2/4

Vertical transform: N * N2/4

Total: (3/8)* N3

which corresponds to about 62% savings in multiplications compared to the case where no pruning is used. There is a similar savings in additions.

# Inverse transform complexity for HM 2.0 and partial butterfly pruned transforms

The number of multiplications and additions required for pruned transforms were counted by hand for the HM 2.0 transform and partial butterfly of JCTVC-E243. An example calculation for L4 case is shown in Appendix. Table 2 summarizes the computational complexity.

Table 2: Pruned transform computational complexity for 1D transform.

| Mults are 16x8 mults |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16x16 |  | 8x8 non-zero in 16x16 |  | 4x4 non-zero in 16x16 |  |  |  |
|  | Mult | Add | Mult | Add | Mult | Add | Mult | Add |
| HM 2.0 - 16x16 | 58 | 66 | 44 | 58 | 36 | 42 |  |  |
| Partial butterfly - 16x16 | 82 | 100 | 44 | 46 | 22 | 32 |  |  |
|  | 32x32 |  | 16x16 non-zero in 32x32 |  | 8x8 non-zero in 32x32 |  | 4x4 non-zero in 32x32 |  |
|  | Mult | Add | Mult | Add | Mult | Add | Mult | Add |
| HM 2.0 - 32x32 | 170 | 194 | 140 | 162 | 124 | 130 | 84 | 88 |
| Partial butterfly - 32x32 | 342 | 372 | 172 | 200 | 86 | 112 | 42 | 64 |

It can be seen from Table 2 that in terms of number of multiplications and additions, the complexity characteristics of HM 2.0 butterfly structure and partial butterfly structures are different depending on amount of pruning done.

# Conclusions

This contribution presents non-zero sub-block statistics of large transforms in anchor bitstreams of HM 2.0. The non-zero sub-block information can be used to carry out IDCT pruning where in the IDCT computations that have zero input and zero output can be eliminated. IDCT pruning is a useful technique that can be used to reduce SIMD computational complexity and power consumption. Complexity of pruned IDCT for HM 2.0 transform and partial butterfly transform (e.g. JCTVC-E243) are presented. This contribution recommends that the pruning behavior of large transforms be considered in design of HEVC transforms.

# References

[1] D. Flynn, BBC, JCTVC Daegu meeting.

# Patent rights declaration(s)

Texas Instruments Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

## Number of computations for L4 case (only lowest 4x4 is non-zero) for HM 2.0 16x16 transform:

Non-colored operations below are pruned operations. Yellow colored multiplication are 16-bit multiplications and pink colored multiplication are 32-bit multiplications

| Stage 1 | Stage 2 | Stage 3 |
| --- | --- | --- |
| F8 = ( 6 * X1 - 63 * X15 ) >> 6 F9 = ( 49 * X9 - 40 * X7 ) >> 6 F10 = ( 30 * X5 - 56 * X11 ) >> 6 F11 = ( 61 * X13 - 18 * X3 ) >> 6 F12 = ( 61 * X3 + 18 * X13 ) >> 6 F13 = ( 30 * X11 + 56 * X5 ) >> 6 F14 = ( 49 * X7 + 40 * X9 ) >> 6 F15 = ( 6 * X15 + 63 * X1 ) >> 6 | E4 = ( 12 * X2 - 62 * X14 ) >> 6 E5 = ( 53 * X10 - 35 * X6 ) >> 6 E6 = ( 53 * X6 + 35 * X10 ) >> 6 E7 = ( 12 * X14 + 62 * X2 ) >> 6 E8 = F8 + F9 E9 = F8 - F9 E10 = F11 - F10 E11 = F11 + F10 E12 = F12 + F13 E13 = F12 - F13 E14 = F15 - F14 E15 = F15 + F14 | D0 = ( 45 * ( X0 + X8 ) ) >> 6 D1 = ( 45 * ( X0 - X8 ) ) >> 6 D2 = ( 24 * X4 - 59 * X12 ) >> 6 D3 = ( 59 * X4 + 24 * X12 ) >> 6 D4 = E4 + E5 D5 = E4 - E5 D6 = E7 - E6 D7 = E7 + E6 D9 = ( 24 * E14 - 59 * E9 ) >> 6 D10 = ( - 59 * E13 - 24 * E10 ) >> 6 D13 = ( 24 * E13 - 59 * E10 ) >> 6 D14 = ( 59 * E14 + 24 * E9 ) >> 6 |

| Stage 4 | Stage 5 | Stage 6 |
| --- | --- | --- |
| C0 = D0 + D3 C3 = D0 - D3, C8 = E8 + E11 C11 = E8 - E11 C12 = E15 - E12 C15 = E15 + E12 C1 = D1 + D2 C2 = D1 - D2 C9 = D9 + D10 C10 = D9 - D10 C13 = D14 - D13 C14 = D14 + D13 C5 = ( 45 * ( D6 - D5 ) ) >> 6 C6 = ( 45 * ( D6 + D5 ) ) >> 6 | B0 = C0 + D7 B7 = C0 - D7 B1 = C1 + C6 B6 = C1 - C6 B2 = C2 + C5 B5 = C2 - C5 B3 = C3 + D4 B4 = C3 - D4 B10 = ( 45 * ( C13 - C10 ) ) >> 6 B13 = ( 45 * ( C13 + C10 ) ) >> 6 B11 = ( 45 * ( C12 - C11 ) ) >> 6 B12 = ( 45 * ( C12 + C11 ) ) >> 6 | Y0 = B0 + C15 Y15 = B0 - C15 Y1 = B1 + C14 Y14 = B1 - C14 Y2 = B2 + B13 Y13 = B2 - B13 Y3 = B3 + B12 Y12 = B3 - B12 Y4 = B4 + B11 Y11 = B4 - B11 Y5 = B5 + B10 Y10 = B5 - B10 Y6 = B6 + C9 Y9 = B6 - C9 Y7 = B7 + C8 Y8 = B7 - C8 |

Number of operations:

- Number of 16-bit mults: Eight 16-bit mults and fourteen 32-bit mults = 8 + 14*2 = 36

- Adds: 42

## Number of computations for L4 case (only lowest 4x4 is non-zero) for Partial butterfly of JCTVC-E243 16x16 transform:

O[0] = T[1][0]*X[1] + T[3][0]*X[3] + T[5][0]*X[5] + T[7][0]*X[7] + T[9][0]*X[9] + T[11][0]*X[11] + T[13][0]*X[13] + T[15][0]*X[15]

O[1] = T[1][1]*X[1] + T[3][1]*X[3] + T[5][1]*X[5] + T[7][1]*X[7] + T[9][1]*X[9] + T[11][1]*X[11] + T[13][1]*X[13] + T[15][1]*X[15]

O[2] = T[1][2]*X[1] + T[3][2]*X[3] + T[5][2]*X[5] + T[7][2]*X[7] + T[9][2]*X[9] + T[11][2]*X[11] + T[13][2]*X[13] + T[15][2]*X[15]

O[3] = T[1][3]*X[1] + T[3][3]*X[3] + T[5][3]*X[5] + T[7][3]*X[7] + T[9][3]*X[9] + T[11][3]*X[11] + T[13][3]*X[13] + T[15][3]*X[15]

O[4] = T[1][4]*X[1] + T[3][4]*X[3] + T[5][4]*X[5] + T[7][4]*X[7] + T[9][4]*X[9] + T[11][4]*X[11] + T[13][4]*X[13] + T[15][4]*X[15]

O[5] = T[1][5]*X[1] + T[3][5]*X[3] + T[5][5]*X[5] + T[7][5]*X[7] + T[9][5]*X[9] + T[11][5]*X[11] + T[13][5]*X[13] + T[15][5]*X[15]

O[6] = T[1][6]*X[1] + T[3][6]*X[3] + T[5][6]*X[5] + T[7][6]*X[7] + T[9][6]*X[9] + T[11][6]*X[11] + T[13][6]*X[13] + T[15][6]*X[15]

O[7] = T[1][7]*X[1] + T[3][7]*X[3] + T[5][7]*X[5] + T[7][7]*X[7] + T[9][7]*X[9] + T[11][7]*X[11] + T[13][7]*X[13] + T[15][7]*X[15]

EO[0] = T[2][0]*X[2] + T[6][0]*X[6] + T[10][0]*X[10] + T[14][0]*X[14]

EO[1] = T[2][1]*X[2] + T[6][1]*X[6] + T[10][1]*X[10] + T[14][1]*X[14]

EO[2] = T[2][2]*X[2] + T[6][2]*X[6] + T[10][2]*X[10] + T[14][2]*X[14]

EO[3] = T[2][3]*X[2] + T[6][3]*X[6] + T[10][3]*X[10] + T[14][3]*X[14]

EEE[0] = T[0][0]*X[0] + T[ 8][0]*X[ 8]

EEO[0] = T[4][0]*X[4] + T[12][0]*X[12]

EEE[1] = T[0][1]*X[0] + T[ 8][1]*X[ 8]

EEO[1] = T[4][1]*X[4] + T[12][1]*X[12]

EE[0] = EEE[0] + EEO[0]

EE[1] = EEE[1] + EEO[1]

EE[2] = EEE[1] - EEO[1]

EE[3] = EEE[0] - EEO[0]

E[0] = EE[0] + EO[0]

E[1] = EE[1] + EO[1]

E[2] = EE[2] + EO[2]

E[3] = EE[3] + EO[3]

E[4] = EE[3] - EO[3]

E[5] = EE[2] - EO[2]

E[6] = EE[1] - EO[1]

E[7] = EE[0] - EO[0]

x[ 0] = E[0] + O[0]

x[ 1] = E[1] + O[1]

x[ 2] = E[2] + O[2]

x[ 3] = E[3] + O[3]

x[ 4] = E[4] + O[4]

x[ 5] = E[5] + O[5]

x[ 6] = E[6] + O[6]

x[ 7] = E[7] + O[7]

x[ 8] = E[7] - O[7]

x[ 9] = E[6] - O[6]

x[10] = E[5] - O[5]

x[11] = E[4] - O[4]

x[12] = E[3] - O[3]

x[13] = E[2] - O[2]

x[14] = E[1] - O[1]

x[15] = E[0] - O[0]

Non-colored operations above are pruned operations. Yellow colored multiplication are 16-bit multiplications. There are no 32-bit multiplications.

Number of operations:

- Number of 16-bit mults: 22

- Adds: 32