| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E388 |
| --- | --- |

| Title: | Cross-verification for Motorola Mobility’s test by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Elena Alshina Jiane Chen Woo-Jin Han | Email: | elena_a.alshina@samsung.com jianle.chen@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This is cross-verification for test contributed by Motorola Mobility in CE3. In average proposed interpolation filter show only 0.01% performance drop compare to HM2.0. According to our measurement encoding time varies on 95% to 100% compare to anchor. Decoding time varies from 99% to 105% compare to anchor.

# Introduction

For this cross-verification, the source code provided by Motorola was investigated and run.

Motorola proposes solid (non switchable) interpolation filter solution for HEVC. In Motorola’s solution the same filter is applied for both High Efficiency and Low Complexity test cases. So proposed solution use the minimum interpolation filter set for 1/4 pel accuracy MC. Compare to HM2.0 only 1/4-pel position interpolation filter was changed by Motorola. For 1/2 pel position proponent uses the same interpolation filter with HM2.0.

Table 1. Difference in 1/4 pel interpolation filter between verified tool and HM2.0.

| HM2.0 | {-1, 4, -10, 57, 19, -7, 3, -1 }, | 12 “+” 4”>>” |
| --- | --- | --- |
| Motorola Mobility | {-1, 3, -9, 57, 18, -6, 2, 0 } | 12 “+” 4”>>” |

As it shown in Table 1 proposed algorithm uses shorter filter for quarter position compare to HM2.0. If interpolation filter would be realized with multiplication than proposed by Motorola filter will use 1 less multiplication and one less addition. But in HM2.0 interpolation filter was implemented in multiplication-free form (using shifts and adds only). This multiplication free representation simulates h/w implementation. The number operations in multiplication-free form is the same for proposed filter and filter used in HM2.0.

Only luma interpolation filter was modified in HM2.0 during this test. Precision of filter coefficients representation was 6 bits (the same with HM2.0 Luma filter).

# Results

The verification tests were done under test conditions agreed to use in CE3 [2]. Four test cases: RA-HE, RA-LC, LD-HE, LD-LC are investigated. Test results are summarized in Table 2.

Table 1 Motorola’s interpolation filter vs HM2.0.

|  | Random access |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class A | 0.0 | -0.1 | -0.2 | -0.3 | -0.2 | -0.2 |  |
| Class B | -0.1 | 0.0 | 0.0 | -0.3 | 0.0 | 0.0 |  |
| Class C | 0.1 | 0.1 | 0.1 | 0.5 | 0.1 | 0.1 |  |
| Class D | 0.3 | 0.3 | 0.4 | 1.3 | 0.3 | 0.4 |  |
| Class E |  |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.1 | 0.3 | 0.1 | 0.1 |  |
| Enc Time[%] | 100% |  |  | 99% |  |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.1 | -0.1 | -0.9 | -0.1 | 0.2 |
| Class C | 0.1 | 0.3 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class D | 0.1 | 0.4 | 0.2 | 0.8 | 0.5 | 0.6 |
| Class E | -0.4 | -0.7 | 0.6 | -1.3 | -0.5 | 0.0 |
| All | -0.1 | 0.0 | 0.2 | -0.3 | 0.0 | 0.2 |
| Enc Time[%] | 99% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |

The average performance drop shown in this test is 0.01%. According to our measurement encoding time varies on 95% to 100% compare to anchor. Decoding time varies from 99% to 105% compare to anchor. Writing reconstructed yuv file on disk was enabled during our test (both for anchor and modification). It should be taken into account that the measurement of running time is rather non-accurate.

The number of operations counting has not been provided before this document uploading. So memory band with usage and computational complexity can’t be confirmed by this cross-check. As soon as proponent will provide complexity analysis this document will be up-dated.

# Conclusions

Test results and complexity estimation have not been provided by proponent before this document uploading. Samsung’s estimation for number of operations in multiplication-free form shows no complexity reduction from proposed filter compare to HM2.0.

The average performance drop shown in this test is 0.01%. According to our measurement encoding time varies on 95% to 100% compare to anchor. Decoding time varies from 99% to 105% compare to anchor.

# References

- JCT-VC, “CE3: Report on Motorola Mobility's interpolation filter for HEVC”, JCTVC-E358, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011

- JCT-VC, “Description of Core Experiment 3 (CE3): Interpolation for MC (Luma)”, JCTVC-D603, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011