| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E391 |
| --- | --- |

| Title: | CU-Level QP Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Wei-Jung Chien Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

Perceptual quantization and rate control algorithms might require QP change at the sub-LCU level. This proposal presents a spatial neighbor based quantization parameter (QP) prediction scheme for derivation of delta QP at CU level. In the current HEVC design, QP of the previous LCU in coding order is used as the predictor of QP instead of the neighboring CU’s QPs. In this contribution, as an alternative to coding order based QP prediction method, spatial neighbor based prediction for delta QP calculation is presented.

# Introduction

Perceptual quantization or adaptive quantization targets visual quality improvement [1][2][3]. Generally it performs finer quantization on flat areas and coarser quantization on busy areas. Spatial correlations among neighboring blocks are reflected to the QP selection. If delta QP signaling is used at sub LCU level, following the CU coding order and selecting the previously coded CUs QP as a predictor for the current CU’s QP doesn’t exploit the spatial correlation between neighboring blocks.

In this proposal QP prediction from neighboring CUs is used. Two methods for QP prediction are described.

# Proposed Scheme

## Method 1

We propose to use left top most neighbor (L) CU’s QP as the predictor for coding of the current CU’s QP. If the left neighbor is not available then the top left most (T) neighboring CU’s QP is used as the predictor, as show in Figure 1.

Figure 1: CU level QP assignment

Hence delta QP calculations becomes

dQPC0 = QPC0 –  QPA5

dQPC1 = QPC1 –  QPC0

dQPC5 = QPC5 –  QPA12

dQPA0 = QPA0 –  QPB8

…………

If left CU is not available then the top CU’s QP is used as a predictor. If the neighboring predictor CU is a skipped CU, then the QP value of the predictor is the QP value of its predictor QP. Basically skip CU’s QP is assumed to of its predictor QP. Top QP predictor gets used at the beginning of each LCU row.

## Method 2

We propose to use immediate neighbors within a quadtree split CU block for QP prediction as shown in Figure 2.

Figure 2: Quadtree QP prediction direction

Arrows in Figure 2 shows the prediction direction for QPs. In this case dQPs are computed as follows

dQP1 = QP1 – QP0

dQP2 = QP2 – QP0

dQP3= QP3 – QP2

dQP0 computation depends on where this quadtree lies in the hierarchical quadtree structure. It can be based on the QP of the top or left neighboring CU depending on its position.

Across quadtrees, higher level quadtree’s prediction pattern is followed. The left and top neighbor definition follows that of method 1.

Across LCUs rules set in method 1 above are followed.

Figure 2 illustrates the QP prediction at higher CU levels and across quadtrees.

In this case delta QPs are computed as follows

dQPC0 = QPC0 –  QPA5

dQPC1 = QPC1 –  QPC0

dQPC5 = QPC5 –  QPC0

dQPC6 = QPC6 –  QPC5

…………

# Conclusion

This contribution proposes use of spatial neighbor based quantization parameter (QP) prediction scheme for derivation of delta QP at CU level instead of using coding order based QP prediction for delta QP calculation.The proposed method exploits the spatial correlation of QPs of neighboring CUs when perceptual quantization is used.

# Reference

- Madhukar Budagavi, and Minhua Zhou “Delta QP signaling at sub-LCU level”, JCT-VC, 4th Meeting, Daegu, KR, Jan. 2011, Doc. JCTVC-D038.

- Kazushi Sato “On LBS and Quantization”, JCT-VC, 4th Meeting, Daegu, KR, Jan. 2011, Doc. JCTVC-D308.

- L. Dong, W. Liu, K. Sato “CU Adaptive Quantization Syntax Change for Better Decoding pipelining”, JCT-VC, 4th Meeting, Daegu, KR, Jan. 2011, Doc. JCTVC-D258.

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).