| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E392 |
| --- | --- |

| Title: | CE11: Adaptive Coefficient Scanning for LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang Liwei Guo Muhammed Coban Marta Karczewicz | Email: | xianglin@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports the results of Qualcomm’s CE11 adaptive coefficient scanning method for LCEC.   Counter based adaptation is used for coefficient scanning in intra block coding. The average bit savings are 1.0%, 0.4% and 0.0% for AI_LC, RA_LC and LD_LC cases respectively. On average there is 2% running time increase at the encoder and the decoder.

- Introduction

In the current HM-2.0, fixed scanning order, vertical, horizontal or zig-zag, is used for coefficients coding in intra block. In this proposal, a counter based adaptation scheme is introduced for coefficient scanning of intra block in LCEC mode. The average bit savings are 1.0%, 0.4% and 0.0% for AI_LC, RA_LC and LD_LC cases, respectively.

- Solution

The proposed method is applied to intra block coding. For each intra prediction mode, an initial scan (horizontal, vertical or zigzag) is assigned. During coefficient coding process, each scan is adjusted using a counter-based scheme described as follows. The counter at a scan position is increased by 1 whenever a non-zero coefficient is encountered. The incremented scan position’s counter is compared with the counter for the previous position in the scan order. If the current position’s counter is greater than the previous position’s counter, the scan order of the neighboring positions is swapped.

Several methods have been applied to reduce the computation complexity and the storage complexity:

- We only maintain the counter for the first M coefficients in the scanning order. The number of M for different block size is provided in Table 1.

| TU Size | 32x32 | 16x16 | 8x8 | 4x4 |
| --- | --- | --- | --- | --- |
| M | 64 | 64 | 64 | 16 |

Table 1 TU size and number of adapted locations M

- The horizontal scan and the vertical scan share the same set of counters to reduce the total number of counters. This implies for a TU size, its vertical scan and horizontal scan are the transpose of each other.

- TU 32x32 and TU16x16 share the same set of counters.

- Whenever a counter reaches 255, all the counters in that scan are divided by 2. As a result, the bit-depth of each counter is limited to be 8.

Table 2 summarizes the total number of counters involved.

| TU Size | 32x32/16x16 | 8x8 | 4x4 | SUM |
| --- | --- | --- | --- | --- |
| # of Zigzag counters | 64 | 64 | 16 | 144 |
| # of H/V counters | 64 | 64 | 16 | 144 |
| Sum | 128 | 128 | 32 | 288 |

Table 2 Number of Counters

- Experimental results

The proposed algorithm was implemented on HM-2.0. The experiments were performed under common CE test condition [1]. The anchor is the HM-2.0 default anchor. The coding efficiency is measured by using 4 point BD-Rate. The summary result is shown in Table 1.

| BD-rate | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -2.0 | -1.4 | -1.3 | -1.0 | -0.7 | -0.4 |  |  |  |
| Class B | -1.0 | -0.8 | -0.8 | -0.4 | -0.3 | -0.4 | -0.1 | -0.1 | -0.2 |
| Class C | -0.6 | -0.3 | -0.3 | -0.3 | -0.2 | -0.1 | -0.1 | 0.0 | -0.1 |
| Class D | -0.2 | -0.1 | -0.1 | 0.0 | 0.2 | 0.1 | 0.1 | 0.2 | -0.1 |
| Class E | -1.2 | -0.9 | -0.9 |  |  |  | 0.1 | -0.5 | 0.3 |
| All | -1.0 | -0.7 | -0.7 | -0.4 | -0.3 | -0.2 | 0 | -0.1 | -0.1 |
| Enc Time[%] | 104% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  | 101% |  |  |

Table 1 Summary result of adaptive coefficients scanning for LCEC (Y BD-rate)

- Conclusions

This contribution describes a counter based adaption scheme for coefficient coding in LCEC mode. The average bit savings are 1.0%, 0.4% and 0.0% for AI_LC, RA_LC and LD_LC cases respectively. The encoding and the decoding time are basically kept the same. It is recommended this adaptive coefficients scanning method be integrated into HM software.

- Reference

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC Meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).