| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E394 |
| --- | --- |

| Title: | CE4: Cross-verification of MediaTek proposal on slice granularity and slice boundary processing (JCTVC-E043, JCTVC-E044) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peisong Chen Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-845-1347 peisongc@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-E043 and JCTVC-E044 proposed by MediaTek. Proposed tools have been evaluated on the common test condition, which is defined in JCTVC-D600 and the test condition required by JCTVC-D604. The source code was provided by MediaTek to cross verify. It is confirmed that the results generated by the cross-checkers match ones provided by MediaTek.

# Cross verification

MediaTek proposal is included in CE4 which is techniques related to slice granularity and slice boundary processing. The proposed method for slice granularity includes 2-step addressing of slice boundary and slice boundary filtering. In all of the following tests, the anchor data are generated by HM-2.0 under the common test condition defined in JCTVC-D600.

CE4/subset1/case4

LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 2-step addressing, use end_of_slice

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.3 | 3.8 | 4.5 | 5.1 | 5.1 | 5.5 |
| Class B | 5.3 | 4.6 | 5.0 | 5.8 | 5.8 | 5.8 |
| Class C | 5.8 | 5.2 | 5.2 | 6.2 | 6.1 | 5.9 |
| Class D | 4.1 | 3.8 | 4.0 | 4.3 | 3.8 | 4.1 |
| Class E | 7.8 | 8.1 | 6.9 | 8.0 | 10.9 | 10.0 |
| All | 5.3 | 4.9 | 5.0 | 5.8 | 6.1 | 6.1 |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.0 | 9.7 | 8.6 | 5.4 | 6.0 | 6.9 |
| Class B | 5.0 | 6.2 | 5.5 | 5.0 | 5.3 | 5.1 |
| Class C | 4.4 | 5.4 | 5.3 | 4.4 | 4.4 | 4.2 |
| Class D | 2.9 | 3.0 | 3.5 | 2.7 | 2.5 | 2.7 |
| Class E |  |  |  |  |  |  |
| All | 4.6 | 6.1 | 5.7 | 4.4 | 4.6 | 4.7 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.7 | 7.3 | 5.5 | 3.7 | 3.8 | 3.6 |
| Class C | 3.0 | 5.0 | 5.1 | 2.8 | 2.6 | 2.4 |
| Class D | 1.8 | 3.4 | 2.7 | 1.8 | 1.8 | 2.0 |
| Class E | 1.9 | 1.9 | 2.3 | 1.3 | 3.8 | 3.1 |
| All | 2.7 | 4.8 | 4.1 | 2.5 | 3.0 | 2.8 |

CE4/subset1/case5

LCU=64x64, leaf-CU-aligned, 1500 bytes per slice, 2-step addressing, remove end_of_slice

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.2 | 3.8 | 4.5 | 5.1 | 5.1 | 5.5 |
| Class B | 5.2 | 4.5 | 5.0 | 5.8 | 5.8 | 5.8 |
| Class C | 5.8 | 5.1 | 5.1 | 6.2 | 6.1 | 5.9 |
| Class D | 4.1 | 3.8 | 3.9 | 4.3 | 3.8 | 4.1 |
| Class E | 7.7 | 8.0 | 6.8 | 8.0 | 10.9 | 10.0 |
| All | 5.3 | 4.9 | 4.9 | 5.8 | 6.1 | 6.1 |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.9 | 9.6 | 8.5 | 5.4 | 6.0 | 6.9 |
| Class B | 4.9 | 6.1 | 5.4 | 5.0 | 5.3 | 5.1 |
| Class C | 4.3 | 5.3 | 5.2 | 4.4 | 4.4 | 4.2 |
| Class D | 2.7 | 2.9 | 3.4 | 2.7 | 2.5 | 2.7 |
| Class E |  |  |  |  |  |  |
| All | 4.5 | 6.0 | 5.6 | 4.4 | 4.6 | 4.7 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.2 | 5.4 | 3.7 | 3.8 | 3.6 |
| Class C | 2.9 | 4.9 | 5.1 | 2.8 | 2.6 | 2.4 |
| Class D | 1.7 | 3.3 | 2.6 | 1.8 | 1.8 | 2.0 |
| Class E | 1.7 | 1.8 | 2.1 | 1.3 | 3.8 | 3.1 |
| All | 2.6 | 4.7 | 4.0 | 2.5 | 3.0 | 2.8 |

CE4/subset2/case3

LCU=64x64, LCU-aligned, two slices per picture, slice-independent DF and ALF,

slice boundary filter off

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.4 | 0.2 | 0.3 | 0.3 |
| Class B | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 | 0.3 |
| Class C | 0.6 | 0.7 | 0.7 | 0.7 | 0.8 | 0.8 |
| Class D | 1.0 | 1.2 | 1.2 | 1.0 | 1.2 | 1.3 |
| Class E | 1.0 | 1.1 | 0.9 | 0.9 | 1.3 | 1.2 |
| All | 0.6 | 0.6 | 0.7 | 0.6 | 0.7 | 0.8 |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.1 | 0.9 | 0.7 | 0.5 | 0.9 |
| Class B | 0.7 | 0.7 | 0.7 | 0.7 | 0.7 | 0.5 |
| Class C | 1.3 | 1.6 | 1.6 | 1.2 | 1.2 | 1.2 |
| Class D | 2.6 | 2.6 | 2.8 | 2.2 | 2.0 | 2.0 |
| Class E |  |  |  |  |  |  |
| All | 1.3 | 1.5 | 1.4 | 1.2 | 1.1 | 1.1 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.2 | 0.7 | 0.6 | 0.4 | 0.9 |
| Class C | 1.2 | 1.8 | 1.8 | 1.1 | 0.8 | 0.6 |
| Class D | 2.5 | 2.9 | 2.4 | 2.1 | 2.3 | 2.1 |
| Class E | 2.5 | 3.2 | 2.5 | 2.1 | 2.2 | 2.1 |
| All | 1.6 | 2.1 | 1.7 | 1.4 | 1.3 | 1.3 |

CE4/subset2/case4

LCU=64x64, LCU-aligned, two slices per picture, slice-independent DF and ALF,

slice boundary filter on

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.4 | 0.2 | 0.3 | 0.3 |
| Class B | 0.2 | 0.2 | 0.2 | 0.3 | 0.2 | 0.3 |
| Class C | 0.6 | 0.5 | 0.5 | 0.7 | 0.7 | 0.6 |
| Class D | 1.0 | 0.9 | 0.9 | 1.0 | 0.9 | 1.0 |
| Class E | 0.9 | 0.8 | 0.7 | 0.8 | 1.0 | 1.0 |
| All | 0.5 | 0.5 | 0.5 | 0.6 | 0.6 | 0.6 |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 1.0 | 0.8 | 0.6 | 0.6 | 0.9 |
| Class B | 0.7 | 0.7 | 0.6 | 0.6 | 0.6 | 0.4 |
| Class C | 1.3 | 1.5 | 1.5 | 1.2 | 1.1 | 1.1 |
| Class D | 2.6 | 2.4 | 2.6 | 2.2 | 1.9 | 1.9 |
| Class E |  |  |  |  |  |  |
| All | 1.3 | 1.4 | 1.3 | 1.1 | 1.0 | 1.1 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.5 | 0.7 | 0.6 | 0.4 | 0.7 |
| Class C | 1.2 | 1.7 | 1.7 | 1.0 | 0.8 | 0.6 |
| Class D | 2.4 | 2.7 | 2.7 | 2.1 | 2.1 | 1.9 |
| Class E | 2.6 | 3.0 | 2.6 | 2.1 | 2.2 | 2.2 |
| All | 1.6 | 2.1 | 1.8 | 1.3 | 1.3 | 1.2 |

# References

[1]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, January 2011.

[2] JCT-VC, “Description of Core Experiment 4: Slice Boundary Processing and Fine Granularity”, JCTVC-D604, 4th. JCT-VC Meeting, Daegu, KR, January 2011.