| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E397 |
| --- | --- |

| Title: | CE6.c: Cross-Verification of Differential Coding of Intra Modes (JCTVC-E318) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | G. Van der Auwera, M. Coban, and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: geertv@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cross-verification of the CE6.c contribution JCTVC-E318 on differential coding of intra modes (DCIM) confirms the rate-distortion results. The decoding times match the measurements of the DCIM proponents with respect to HM2.0.

# Introduction

Differential coding of intra modes (DCIM) is an edge-based intra prediction method. This contribution reports the objective rate-distortion results and the processing times.

# Objective Verification Results

## Rate-Distortion Results

Table 1 enumerates the rate-distortion results as BD-rate differences. The common test conditions are those specified in JCTVC-D600 for intra coding in the high-efficiency and low-complexity configurations. The conclusion is that the BD-rate differences match those reported by the proponents.

Table 1 Rate-distortion results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -0.5 | -0.3 | -1.0 | -0.2 | -0.3 |
| Class B | -1.3 | -0.9 | -1.0 | -1.2 | -1.3 | -1.5 |
| Class C | -1.8 | -1.5 | -1.8 | -2.2 | -1.9 | -2.1 |
| Class D | -1.3 | -0.9 | -1.0 | -1.7 | -1.3 | -1.4 |
| Class E | -1.8 | -1.7 | -1.6 | -2.3 | -0.5 | -0.9 |
| All | -1.4 | -1.1 | -1.1 | -1.6 | -1.1 | -1.3 |
| Enc Time[%] | 103% |  |  | 106% |  |  |
| Dec Time[%] | 105% |  |  | 110% |  |  |

## Encoding and Decoding Times

Table 1 reports the encoding and decoding times for DCIM.

The encodings were executed on a Linux computing cluster consisting of machines with various computing performance and memory configurations. The encoder executable was built using GCC version 4.1.2. The differences in encoding times between our measurements and the proponents’ measurements are likely attributable to the variation in execution times on our computing cluster.

The decodings were executed on a PC with the following configuration: Windows 7, 6 GB memory, Intel Xeon 2.79GHz (W3530), Microsoft Visual Studio 2008. The decoding times are similar to the measurements reported by the proponents.

# Conclusion

This cross-verification matches the rate-distortion results and decoding times reported by the DCIM proponents.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).