| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E399 |
| --- | --- |

| Title: | Sliding Window for Temporal Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen and Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1 858 845 6589 cheny@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The current sliding window reference picture marking mechanism is not useful, especially when temporal scalability is supported with hierarchical coding structure.  To support temporal scalability, reference picture list construction processes are constrained such that a picture with a higher temporal level will not be used as an inter prediction reference for the current picture.  In this contribution, it is proposed that similar constraints to be given to the sliding window, such that a picture will not implicitly mark a reference picture with a lower temporal level in the DPB as “unused for reference”.

# Introduction

## DPB Management in H.264/AVC

Reference pictures after decoded, are put in the Decoded Picture Buffer (DPB).  A reference picture can only be removed from the DPB after it has been marked as “unused for reference”.

There are two types of operations for the reference picture marking: sliding window and adaptive memory control.  Sliding window operation, as an implicit marking mechanism, works as a first-in-first-out queue with a fixed number of short-term reference pictures. In other words, short-term reference pictures with earliest decoding time is firstly to be marked as “unused for reference”.

The adaptive memory control however marks reference pictures explicitly, e.g., by using the frame_num difference to indicate which short-term reference picture is to be marked.

Note that after each picture is decoded, either the implicit marking or explicit marking has to be invoked.

## Problems

In a hierarchical B prediction structure, sliding window is not useful.

For example, if the GOP size is equal to 16, typically the num_ref_frames must be set at least to 8, when the decoding order is as shown in Table 1. Otherwise, the sliding window mechanism will mark picture 0 (POC equal to 0) and picture 16 (POC equal to 16) as “unused for reference” when coding the reference pictures with POC values between 0 and 16.

Note that in Table 1, the highlighted pictures are reference pictures thus by default activate sliding window operations.

Table 1 Decoding order for the pictures in the first GOPs when GOP size is 16

| Decoding Order | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| POC | 0 | 16 | 8 | 4 | 2 | 1 | 3 | 6 | 5 | 7 | 12 | 10 | 9 | 11 | 14 | 13 | 15 |
| TID | 0 | 0 | 1 | 2 | 3 | 4 | 4 | 3 | 4 | 4 | 2 | 3 | 4 | 4 | 3 | 4 | 4 |

However, such a case may only need to set the num_ref_frames to 5. This requires the adaptive memory control operations.  For example, after the picture 6 (POC equal to 6) is coded, picture 2 is marked as “unused for reference”.

Almost every reference picture needs to have the explicit reference picture marking syntax signaled.  This also means additional encoder intelligence is needed to generate those syntax elements correctly.  This is more difficult when adaptive GOP size is used.

# Proposal

Similar to the constraints that have been introduced for reference picture list construction, to solve the above mentioned problems, it is proposed that sliding window shall only mark a reference picture, with temporal_id value not lower than the current picture, as “unused for reference”.

With the proposed modification, the explicit reference picture marking mechanism (adaptive memory control) is not needed to optically manage the short-term references in terms of memory consumption.

The following modification is needed in the specification text (only the red text is newly added).

#### 8.2.5.3 Sliding window decoded reference picture marking process

This process is invoked when adaptive_ref_pic_marking_mode_flag is equal to 0.

Depending on the current field, the following applies.

- If the current picture is a coded field that is the second field in decoding order of a complementary reference field pair, and the first field has been marked as “used for short-term reference”, the current picture is also marked as “used for short-term reference”.

- Otherwise, the following applies.

-	Let numShortTerm be the total number of reference frames, complementary reference field pairs and non-paired reference fields for which at least one field is marked as “used for short-term reference”. Let numLongTerm be the total number of reference frames, complementary reference field pairs and non-paired reference fields for which at least one field is marked as “used for long-term reference”.

-	Let temporal_id of the current picture be tId.

-	When numShortTerm + numLongTerm is equal to num_ref_frames, the condition that numShortTerm is greater than 0 shall be fulfilled, and the short-term reference frame, complementary reference field pair or non-paired reference field with a temporal_id equal to or higher than tID that has the smallest value of FrameNumWrap is marked as “unused for reference”. When it is a frame or a complementary field pair, both of its fields are also marked as “unused for reference”.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).