| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E400 |
| --- | --- |

| Title: | Comments on Clean Decoding Refresh Pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen, Coban Muhammed, Peisong Chen and Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1 858 845 6589 cheny@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

The clean decoding refresh (CDR) pictures as a new type of random access points besides the IDR pictures were introduced to HEVC.  This document includes discussions and proposals on four aspects related to the CDR pictures: the definition of a CDR picture, decoding of some unnecessary pictures following the CDR picture, implicit reference picture marking after decoding of a CDR picture, and the detection of whether a CDR is being used for random access or just for normal playback.

# Introduction

## Current status of CDR design

In the latest working draft of HEVC, a new NAL unit type is assigned to a CDR picture.  The current definition of CDR picture contains the following aspects:

- A CDR picture is an Intra picture

- As a random access point, pictures following a CDR picture in the output order can be decoded without inter prediction from any picture that precedes this CDR in the output order

- An implicit marking process is specified before the decoding of the first picture following the CDR in the output order

# Discussions and Proposals

In this section, discussions on the definition of the CDR picture are firstly given.  Proposals on decoding of some unnecessary pictures, implicit reference picture marking and detection of random access behavior are then provided.

## Definition of the CDR picture

This part focuses on the second aspect of the current definition of CDR, as listed in Section 1.1.  An example of random access point picture is shown in Fig 1, wherein pictures (with POC 41 through 47) in the GOP containing a CDR may not be successfully decoded and may or may not be needed for inter prediction by pictures (POC>48) following the CDR picture in output order.  Just for simplicity, those pictures that follow the CDR in the decoding order and precede the CDR in display order are called potentially unnecessary pictures.  Note that the CDR picture, as shown in Fig 1, in this context, is not exactly the same as the CDR defined in the current HEVC Working Draft (WD), however, it is an Intra (I) picture with the corresponding prediction relations illustrated in Fig 1.

Fig 1:  Inter prediction from Intra coded potentially unnecessary picture.

In this example, there is one potentially unnecessary picture, I46 that can be decoded successfully if CDR48 is used for random access.  Moreover a picture, e.g., B56 following CDR48 in output order may use I46 for inter prediction.  Such a case is supported in H.264/AVC and the CDR48 can still be used as a random access point, if the decoding of I46 is guaranteed.

Fig 2:  Inter prediction from Inter coded potentially unnecessary picture.

Such a potentially unnecessary picture that is useful for the inter prediction of the pictures following the CDR in the output order can also be an Inter picture, as shown in Fig 2, as P46 or B46.  Based on the current definition of CDR in HEVC, such a situation is disallowed, thus any potentially unnecessary picture is really not needed for the decoding of the pictures following CDR in the output order.

We have two alternative solutions

- Keep the current definition of CDR, thus any potentially unnecessary picture is not needed (for decoding or buffering) when the CDR is used for random access

- Modify the current definition of CDR as follows:

As a random access point, pictures following a CDR picture in the output order can be decoded without inter prediction from any picture that precedes this CDR in output decoding order. Inter prediction from the pictures that precede this CDR in output order, follow this CDR in decoding order and can be successfully decoded when the CDR is used for random access, may happen for pictures following the CDR picture in the output order.

If the CDR definition is modified accordingly, there are still a lot of pictures e.g., all the other potentially unnecessary pictures except I46/P46/B46 that are not needed when the CDR is used for random access.

In this case, a flag is proposed to be added into NAL unit header or slice header to indicate this.  Suppose the closest CDR that proceeds the current picture in the decoding order is just decoded for random access, this flag indicates whether the current picture is successfully decodable and may be used for inter prediction by pictures following the CDR in the output order.

## Decoding of the potentially unnecessary pictures

The current HEVC WD doesn’t explicitly mention if the potentially unnecessary pictures need to be decoded when the CDR is used for random access.

However decoding and displaying those pictures will not improve the user experience and may complicate the system level implementations.

Moreover, decoding and buffering those sequences may cause the following problems:

- Unnecessary computations

- More memory consumption, even to a level that is larger than the maximum Decoded Picture Buffer (DPB) size.

So, it is proposed that the decoding of potentially unnecessary pictures is skipped when the CDR is used for random access.

## Implicit reference picture marking

Currently, an implicit reference picture marking process is invoked after the CDR picture is stored in the DPB.  Such a design has two drawbacks:

- It anyways stores one additional picture (the CDR) in the current DPB.  When random access happens, the decoder doesn’t know the current status of the DPB, it might be full already.

- It is not consistent with the reference picture marking design principle in H.264/AVC.

  - In H.264/AVC, reference picture marking happens always right after a reference picture is decoded and before the picture is not stored into the DPB.

  - The traditional random access point, IDR pictures also require a decoding process to clear the DPB after an IDR is decoded and before it is stored into the DPB.

Similar to the implicit reference picture marking for IDR picture or a picture with MMCO equal to 5, it is proposed that after decoding a CDR and before storing it to DPB, all reference pictures are marked as “unused for reference”, when the CDR is used for random access.

## Detection of a CDR being used for random access

In the above mentioned scenarios, to make the decoder more optimal and clean, in terms of decoder complexity and memory management, it is necessary to detect whether a CDR is used for random access.

It is proposed that a picture is signaled in the slice header of a CDR picture.  When decoding a CDR, if the signaled picture is not in the DPB, it is detected that random access is invoked. Otherwise, it is detected that the decoder is doing normal sequential decoding.

Such a picture, namely brother picture can be e.g., the closest picture that precedes the CDR in decoding order and has a temporal level equal to 0.  The POC or frame_num of the brother picture can be signaled in the slice header of the CDR picture.  In the example as shown in Fig 1, B48 is signaled as a brother picture.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).