| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E401 |
| --- | --- |

| Title: | Slice Design with Fine Granularity |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Peisong Chen, Ying Chen, Xianglin Wang and Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1 858 845 1347 peisongc@qualcomm.com cheny@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This is a follow-up proposal of JCTVC-D396.  In the previous proposal, it was proposed that the boundary of two consecutive slices can be put inside a largest coding unit (LCU), and since the needed slice granularity may vary from picture to picture, the slice granularity for a group of pictures is signaled in Picture Parameter Sets (PPSs).  In this proposal, more analysis results are given based on the previous proposal.

# Introduction

In applications e.g., video conferencing, the coded video sequence can be packetized into RTP packets and the payload of each RTP packet (with the packet header) should not exceed the Maximum Transmission Unit (MTU) size, especially in an error-prone environment.

- If the size of a slice exceeds the MTU size, it will be spitted into more than one packet in the transport level so that if any of the packets is lost, the whole slice is useless even if the other packets of the slice are successfully received.

- If the size of a slice is smaller than the MTU size, the RTP packet might be smaller than the MTU size, thus the overhead of the packet is larger.

When encoding a picture into multiple slices, it is better to prevent the above two cases from happening. However, statistically speaking, if most of the slices have a size close to the MTU size, the packetization is considered efficient and error resilient.

# Problems

If LCU is large, e.g., 64x64, the coded bits of a LCU may be larger than the MTU size, so it might be desirable to at least increase the slice granularity for some pictures.

It is reported in JCTVC-D396, the bit consumption for CUs with the same size might be different for different pictures.

To enable error resilient decoding, the location of beginning block of a slice is to be signaled.  When the slice granularity increases, e.g., from 32x32 to 8x8, more bits may be required to signal that location.

# Proposal

It is shown that a large number of the pictures may consume 1% of the MTU size in average if the slice granularity is set to 32x32 or less and the required slice granularity is picture dependent.

As proposed in JCTVC-D396, the slice granularity can be signaled in the Picture Parameter Sets.

Signaling the slice granularity in PPS, can avoid always using more bits e.g., to signal the 8x8 block locations in the slice header.

# Simulation results

In the experiment, different slice granularity is set for pictures simply based on temporal level and spatial resolution. The tested software is HM-2.0-dev-slices (rev 609) provided by the AhG on slice granularity. The target number of bytes per slice was set to a typical MTU size of 1500.

The anchor data is generated by using HM-2.0 with the default configuration defined in JCTVC-D600 without slice partitioning.

Only results for HE Random access case are shown here. In the Random access case, temporal level dependent slice granularity is realized.

The following table shows temporal level dependent slice granularity for different classes of test sequences. Multiple PPSs are signaled in our implementation. Note that it is possible that class C and class D can just use 16x16 as the slice granularity, although we don’t have time to run the simulations.

|  | Temporal level |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| Class A | 16x16 | 16x16 | 32x32 | 64x64 |
| Class B | 16x16 | 16x16 | 32x32 | 64x64 |
| Class C | 8x8 | 16x16 | 16x16 | 16x16 |
| Class D | 8x8 | 16x16 | 16x16 | 16x16 |

The BD rate increase is shown in the following table:

|  |  | Random access |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.0 | 9.7 | 9.0 |
| Class B | 5.0 | 6.2 | 5.6 |
| Class C | 4.4 | 5.3 | 5.2 |
| Class D | 2.8 | 3.0 | 3.5 |
| All | 4.6 | 6.1 | 5.8 |

It is noticed that the proposed solution never performs worse than the other relevant solutions and may have some gain in some specific cases.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).