| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E403 |
| --- | --- |

| Title: | CE11: Cross-check of Panasonic’s context size reduction for the significance map (JCTVC-E227) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | J. Sole 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: +1-858-845-2429 Email: joels@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This is the cross-verification report of Panasonic’s context size reduction for the significance map contribution in CE11 (document JCTVC-E227). The verification includes 3 cases: simplification of the significance flag context, simplification of the last flag context and the combination of both. Rate-distortion results by Panasonic are confirmed. Bin count results are also confirmed. Encoding and decoding times are very similar to those of HM2.0.

# Experimental Conditions

Panasonic’s context size reduction method described in JCTVC-E227 [1] is cross-verified. The software to be tested was provided by Panasonic and its performance relative to HM2.0 software has been checked under the common test condition described in JCTVC-D600 [2].
Three cases have been tested within the scope of CE11 [3]:

- Context size reduction for the significant flag (n1)

- Context size reduction for the last flag (n2)

- Combination of both (n1+n2)

# Results

## BD-rates

Table 1 shows the BD-rate results for n1, table 2 shows the results for n2 and table 3 shows the results for the case n1+n2. All the BD-rates match with the ones provided by Panasonic.

Table 1: BD-rate for n1 (high efficiency)

| BD-rate | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.12 | -0.22 | -0.20 | 0.03 | 0.05 | -0.23 |  |  |  |
| Class B | 0.01 | -0.14 | -0.13 | -0.02 | -0.23 | -0.27 | -0.06 | -0.20 | -0.14 |
| Class C | -0.12 | -0.11 | -0.08 | -0.19 | -0.31 | -0.21 | -0.26 | -0.47 | -0.40 |
| Class D | -0.20 | -0.12 | -0.08 | -0.14 | -0.37 | -0.14 | -0.31 | -0.66 | -1.24 |
| Class E | -0.05 | -0.13 | -0.31 |  |  |  | -0.29 | -0.28 | 0.29 |
| All | -0.04 | -0.14 | -0.15 | -0.08 | -0.22 | -0.21 | -0.22 | -0.40 | -0.40 |
| Enc T[%] | 100% |  |  | 99% |  |  | 99% |  |  |
| Dec T[%] | 101% |  |  | 101% |  |  | 101% |  |  |

Table 2: BD-rate for n2 (high efficiency)

| BD-rate | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.13 | 0.16 | 0.17 | 0.17 | 0.25 | -0.25 |  |  |  |
| Class B | 0.12 | 0.14 | 0.08 | 0.07 | -0.07 | -0.12 | 0.06 | -0.15 | -0.27 |
| Class C | 0.06 | 0.00 | -0.04 | 0.04 | -0.18 | -0.25 | 0.09 | -0.12 | -0.51 |
| Class D | 0.07 | -0.02 | -0.11 | 0.05 | -0.56 | -0.54 | 0.00 | -0.28 | -1.13 |
| Class E | 0.01 | 0.02 | 0.23 |  |  |  | -0.11 | -0.41 | 0.80 |
| All | 0.08 | 0.07 | 0.06 | 0.08 | -0.14 | -0.28 | 0.02 | -0.22 | -0.34 |
| Enc T[%] | 99% |  |  | 97% |  |  | 99% |  |  |
| Dec T[%] | 102% |  |  | 105% |  |  | 104% |  |  |

Table 3: BD-rate for n1+n2 (high efficiency)

| BD-rate | Intra |  |  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.25 | -0.04 | 0.02 | 0.20 | 0.18 | -0.21 |  |  |  |
| Class B | 0.13 | -0.01 | -0.03 | 0.05 | -0.24 | -0.24 | 0.00 | -0.30 | -0.22 |
| Class C | -0.07 | -0.07 | -0.10 | -0.13 | -0.43 | -0.40 | -0.15 | -0.65 | -0.55 |
| Class D | -0.14 | -0.07 | -0.10 | -0.13 | -0.48 | -0.43 | -0.21 | -1.13 | -1.52 |
| Class E | -0.05 | -0.11 | -0.14 |  |  |  | -0.28 | -0.86 | 0.20 |
| All | 0.03 | -0.05 | -0.06 | 0.00 | -0.24 | -0.32 | -0.14 | -0.70 | -0.55 |
| Enc T[%] | 101% |  |  | 99% |  |  | 98% |  |  |
| Dec T[%] | 102% |  |  | 102% |  |  | 101% |  |  |

## Encoding and Decoding Times

The encodings were executed on a Linux computing cluster consisting of machines with various computing performance and memory configurations. The encoder executable was built using GCC version 4.1.2. The decoding processes were executed on a PC with the following configuration: Windows 7, 6 GB memory, Intel Xeon 2.79GHz (W3530), Microsoft Visual Studio 2008.

As shown in tables 1, 2 and 3, encoding and decoding times are very similar to the encoding and decoding times of the anchors.

## Bin Counts

As per the CE11 requirements, the following bin counts are provided:

- The total number of bins and bin reduction/increase due to proposed technique.

- A breakdown of the number of bins coded in regular and bypass mode for CABAC.

- A breakdown of the number of bins for sig flag, last flag and absolute coefficient level.

Tables 4, 5 and 6 show the bin counts for case n1, n2 and n1+n2, respectively. The counts match with the ones provided by Panasonic.

Table 4: Bin counts for n1

|  | Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 30% | 14% | 28% | 85% | 15% |
| Class B | 0% | 32% | 14% | 23% | 86% | 14% |
| Class C | 0% | 28% | 14% | 23% | 85% | 14% |
| Class D | 0% | 27% | 15% | 24% | 85% | 15% |
| Class E | 0% | 27% | 13% | 23% | 87% | 13% |
| All | 0% | 29% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 29% | 11% | 17% | 86% | 14% |
| Class B | 0% | 28% | 10% | 16% | 87% | 13% |
| Class C | 0% | 24% | 10% | 15% | 86% | 13% |
| Class D | 1% | 24% | 10% | 15% | 87% | 13% |
| Class E |  |  |  |  |  |  |
| All | 0% | 26% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 27% | 9% | 13% | 87% | 12% |
| Class D | 1% | 27% | 9% | 12% | 88% | 12% |
| Class E | 1% | 24% | 7% | 8% | 89% | 9% |
| All | 0% | 27% | 9% | 12% | 88% | 11% |

Table 5: Bin counts for n2

|  | Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 30% | 14% | 28% | 85% | 15% |
| Class B | 0% | 32% | 14% | 23% | 86% | 14% |
| Class C | 0% | 28% | 14% | 23% | 85% | 15% |
| Class D | 0% | 27% | 15% | 24% | 85% | 15% |
| Class E | 0% | 27% | 13% | 23% | 87% | 13% |
| All | 0% | 29% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 28% | 11% | 17% | 86% | 14% |
| Class B | 0% | 28% | 10% | 16% | 87% | 13% |
| Class C | 0% | 23% | 10% | 15% | 86% | 13% |
| Class D | 0% | 23% | 10% | 15% | 87% | 13% |
| Class E |  |  |  |  |  |  |
| All | 0% | 26% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 26% | 9% | 13% | 87% | 12% |
| Class D | 0% | 26% | 9% | 12% | 88% | 12% |
| Class E | 0% | 23% | 7% | 8% | 89% | 9% |
| All | 0% | 27% | 9% | 12% | 88% | 11% |

Table 6: Bin counts for n1+n2

|  | Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 30% | 14% | 28% | 85% | 15% |
| Class B | 0% | 32% | 14% | 23% | 86% | 14% |
| Class C | 0% | 28% | 14% | 23% | 85% | 14% |
| Class D | 0% | 27% | 15% | 24% | 85% | 15% |
| Class E | 0% | 27% | 13% | 23% | 87% | 13% |
| All | 0% | 29% | 14% | 24% | 86% | 14% |
|  | Random access |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A | 0% | 28% | 11% | 17% | 86% | 14% |
| Class B | 0% | 28% | 10% | 16% | 87% | 13% |
| Class C | 0% | 24% | 10% | 15% | 86% | 13% |
| Class D | 1% | 24% | 10% | 15% | 87% | 13% |
| Class E |  |  |  |  |  |  |
| All | 0% | 26% | 10% | 16% | 86% | 13% |
|  | Low delay |  |  |  |  |  |
|  | increase in bin % | % sig | % last | % coeff | % reg | % bypass |
| Class A |  |  |  |  |  |  |
| Class B | 0% | 29% | 10% | 13% | 88% | 11% |
| Class C | 0% | 26% | 9% | 13% | 87% | 12% |
| Class D | 1% | 27% | 9% | 12% | 88% | 12% |
| Class E | 1% | 24% | 7% | 8% | 89% | 9% |
| All | 0% | 27% | 9% | 12% | 88% | 11% |

## Number of Contexts

Currently, HM2.0 uses 107 contexts for the significance map (15/16 contexts per block size and component). This amount is reduced to 83 contexts in the n1 case.

HM2.0 uses 104 contexts for the last flag (15/16 contexts per block size and component). This amount is reduced to 28 contexts in the n2 case.

# Conclusion

The results provided by Panasonic for the context size reduction contribution in CE11 are confirmed.

# References

- H. Sasai and T. Nishi, "CE11: Context size reduction for the significance map", JCTVC-E227, Daegu, KR, January 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Daegu, KR, January 2011.

- V. Sze et. al., “CE11: Coefficient Scanning and Coding”, JCTVC-D611, Daegu, KR, January 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).