| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E404 |
| --- | --- |

| Title: | CE5: LCEC coded block flag coding under residual quadtree |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Peisong Chen Xianglin Wang, Wei-Jung Chien, Liwei Guo, Marta Karczewicz, Bin Li, Ji-Zheng Xu | Tel: Email: | +1-858-651-5135 xianglin@qualcomm.com jzxu@microsoft.com |
| Source: | Qualcomm Inc., Microsoft Research |  |  |

_____________________________

# Abstract

In HM2.0, coded block flags of Y, U, and V component at CU root level are jointly coded. To improve coding efficiency, this proposal extends such joint coding in two ways: 1) It further includes transform split flag of the same block as the same level if it needs to be explicitly signaled; 2) The joint coding is not only performed at CU root level, but also extended to higher transform depth level so that the benefit of such joint coding can be maximized. Simulation results show that using default LC configurations and with a maximum TU depth level of 3, BD rate savings with the proposed scheme are 0.3% for all intra, 0.5% for random access and 0.7% for low delay case respectively.

- Introduction

According to the current HM low complexity entropy coding (LCEC), coded block flags (or cbf) are coded both at coding unit root level and at higher transform depth level. At coding unit root level, coded block flag of Y, U and V components are coded jointly. A coded block flag at this level indicates if a corresponding video component of the coding unit contains any non-zero coefficient. If a coded block flag at coding unit root level is not zero and the transform block size is smaller than the coding unit size, coded block flags at higher transform depth level are further coded. In this case, four coded block flags at the same transform depth level are grouped and coded jointly.

The following two tables show transform tree syntax and transform coefficient syntax extracted from the current working draft. For simplicity, the associated conditions are omitted.

### Transform tree syntax

| CABAC | LCEC |
| --- | --- |
| transform_tree { | transform_tree { |
| no_residual_data_flag(at root level) | cbp_yuv_root(at root level) |
| split_transform_flag(trafoDepth ) | split_transform_flag( trafoDepth ) |
| coded_block_flag_cb(trafoDepth) |  |
| coded_block_flag_cr(trafoDepth ) |  |
| coded_block_flag_luma (when no further split) |  |
| … … … | … … … |
| } |  |

### Transform coefficient syntax

| CABAC | LCEC |
| --- | --- |
| transform_coeff (trafoDepth) { | transform_coeff (trafoDepth) { |
|  | cbp_luma (trafoDepth+1) |
|  | cbp_cb (trafoDepth+1) |
|  | cbp_cr (trafoDepth+1) |
| … … … | … … … |
| } |  |

# Proposed Approach

Based on this proposal, to further improve coding efficiency the joint coding of Y, U and V coded block flag at CU root level is further extended in two ways: 1) transform split flag, when available, is also jointly coded together with the three coded block flags;  2) the joint coding is not only performed at CU root level, but also used at higher transform depth level. If there is only one luma coded block flag left for a block, which always happen for a 4x4 block, the flag is combined with three other luma coded block flags from neighboring blocks and coded jointly.

When only luma cbf and transform split flag are jointly coded (Flagpattern=9), for four blocks at the same depth level, if the luma cbf is zero for the first three blocks and the luma cbf of parent block is one, the luma cbf for the fourth block is inferred as 1. In this case for the fourth block, only the transform split flag is coded.

To limit the number of codewords for the joint coding of Y, U, V coded block flags and split flag, U and V are combined such that a one-bit flag is used to indicate whether both U and V cbf are zero or not.

Different VLC tables are used depending on what flags are to be jointly coded. In the following table 1, a 4-bit value “flag pattern” is used to indicate which flags need to be signaled. The first three bits correspond to the Y, U, V cbf and the lsb transform split flag. It a corresonding bit is 1, it indicates that the associated flag needs to be signaled. Otherwise, it means the associated flag can be inferred.

Table 1: number of codewords needed for joint coding

| Flag pattern | No. of codeword | Relative to current scheme |
| --- | --- | --- |
| 8 | 15 | Same |
| 9 | 4(intra)/3(inter) | Additional |
| 10, 12 | 4 | Additional |
| 11, 13, 15 | 8(intra)/7(inter) | Additional |
| 14 | 8 | Same |

From Table 1, it can be seen that to limit the number of VLC tables needed, the same VLC table may be shared under different cbf and transform split flag combinations. Compared with the current scheme in HM2.0, additional VLC tables are introduced. For flag pattern 9, 10 and 12, unary code is used. When flag pattern is one of 11, 13 and 15, for inter blocks unary code is used,  while for intra blocks the following VLC codewords are used:

| In addition, with the proposed method, the syntax on CBP and transform split flag signaling becomes more consistent between LCEC and CABAC. Codeword | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code length | 1 | 2 | 4 | 4 | 4 | 5 | 6 | 6 |
| Code value | 0 | 3 | 9 | 11 | 8 | 20 | 42 | 43 |

### Transform tree syntax

| CABAC | LCEC |
| --- | --- |
| transform_tree { | transform_tree { |
| no_residual_data_flag(when at root level) | cbp_yuv_split(when at root level) |
| split_transform_flag( trafoDepth ) |  |
| coded_block_flag_cb(trafoDepth) | cbp_yuv_split (trafoDepth) |
| coded_block_flag_cr(trafoDepth ) |  |
| coded_block_flag_luma (when no further split) |  |
| … … … | … … … |
| } |  |

# Simulation Results

To test the proposed scheme, we used the three low complexity configurations.  In addition to the default low complexity configurations, we also tested the proposed scheme using a maximum TU depth of 3 in the low complexity configurations. The implementation was on HM-2.0.

In the first test, the current HM-2.0 is used as anchor. Both the anchor and our proposed method were using default low complexity configurations. In this case, the maximum TU depth was 1 for intra and 2 for inter.

In the second test, still the current HM-2.0 was used as anchor, but a maximum TU depth of 3 was used for both intra and inter blocks in both the anchor and our proposed method.

Based on the results, it can be seen that the proposed method outperforms the current HM-2.0, especially at the TU depth of 3.

# Conclusion

This contribution presents a joint coding scheme for coded block flags and split flag. According to the simulation results, the proposed method outperforms the current HM-2.0. Furthermore, with the proposed method the CBF and transform split flag syntax are more consistent between LCEC and CABAC. We recommend that the proposed method be adopted into the next HM.

# References

[1] X. Wang, M. Karczewicz, W.-J. Chien, “TE4: Report of VLC for coded block flag”, JCTVC-C262, Guangzhou, China, Oct. 2010.

[2] T. Wiegand, H. Schwarz, B. Bross, A. Fuldseth, X. Wang, W.-J. Han, “BoG report: residual quadtree structure”, JCTVC-C319, Guangzhou, China, Oct. 2010.

# Patent rights declaration(s)

Qualcomm Inc and Microsoft may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Annex: proposed text

### Transform tree syntax

| transform_tree( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if (entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0){ |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | ¦ ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if ( residualDataPresentFlag) { |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if(!entropy_coding_mode_flag) { |  |
| xBase = x0 − ( x0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| yBase = y0 − ( y0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| codeLuma = (trafoDepth = = 0 ¦ ¦ cbf_luma[xBase][yBase][trafoDepth − 1 ]) |  |
| codeCb = (trafoDepth = = 0 ¦ ¦ (cbf_cb[xBase][yBase][trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize) ) |  |
| codeCr = (trafoDepth = = 0 ¦ ¦ (cbf_cr[xBase][yBase][trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize) ) |  |
| codeSplitTrans = (log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag) |  |
| if(!codeLuma ) { |  |
| if(codeCb) |  |
| cbf_cb[ x0 ][ y0 ] [trafoDepth ] | u(1) |
| if(codeCr) |  |
| cbf_cr[ x0 ][ y0 ] [trafoDepth ] | u(1) |
| if(codeSplitTrans) |  |
| split_transform_flag[ x0 ][ y0 ] [trafoDepth ] | u(1) |
| } |  |
| else |  |
| cbp_yuv_split_trans | vlc(n,v) |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag) |  |
| split_transform_flag[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( PredMode != MODE_INTRA && log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| xBase = x0 − ( x0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| yBase = y0 − ( y0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | ae(v) |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_tree( x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } |  |
| else if( entropy_coding_mode_flag ){ |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| if( PredMode = = MODE_INTRA ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| } else if( blkIdx = = 0 ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | u(1) ¦ ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } |  |

cbp_yuv_split_trans contains a number of coded block flags and/or a split_transform_flag. It is decoded using one of five VLC tables Table 7-1 ~ Table 7-5. Table selection is based on a 4-bit value flagPattern which is derived as follows.

flagPattern = (codeLuma<<3) + (codeCb<<2) + (codeCr << 1) + codeSplitTrans

codeLuma, codeCb, codeCr and codeSplitTrans are derived according to 6.1.1 and specify respectivley if associated flag cbf_luma, cbf_cb, cbf_cr, and split_transform_flag need to be signaled.

- Table 7-1 – cbp_yuv_split_trans codeword index mapping (flagPattern=8)

| Block0 | Block1 | Block2 | Block3 | Intra (vlc0, vlc2, vlc11) | Inter (vlc11) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | NA | NA |
| 0 | 0 | 0 | 1 | 14 | 0 |
| 0 | 0 | 1 | 0 | 13 | 1 |
| 0 | 0 | 1 | 1 | 10 | 4 |
| 0 | 1 | 0 | 0 | 12 | 2 |
| 0 | 1 | 0 | 1 | 9 | 5 |
| 0 | 1 | 1 | 0 | 8 | 6 |
| 0 | 1 | 1 | 1 | 4 | 10 |
| 1 | 0 | 0 | 0 | 11 | 3 |
| 1 | 0 | 0 | 1 | 7 | 7 |
| 1 | 0 | 1 | 0 | 6 | 8 |
| 1 | 0 | 1 | 1 | 3 | 11 |
| 1 | 1 | 0 | 0 | 5 | 9 |
| 1 | 1 | 0 | 1 | 2 | 12 |
| 1 | 1 | 1 | 0 | 1 | 13 |
| 1 | 1 | 1 | 1 | 0 | 14 |

- Table 7-2 – cbp_yuv_split_trans codeword index mapping (flagPattern=9)

| Luma | Split | Intra (unary vlc) | Inter (unary vlc) |
| --- | --- | --- | --- |
| 0 | 0 | 3 | 2 |
| 0 | 1 | 2 | NA |
| 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 |

- Table 7-3 – cbp_yuv_split_trans codeword index mapping (flagPattern=10, 12)

| Luma | CbCr | Intra (unary vlc) | Inter (unary vlc) |
| --- | --- | --- | --- |
| 0 | 0 | 3 | 0 |
| 0 | 1 | 1 | 2 |
| 1 | 0 | 2 | 1 |
| 1 | 1 | 0 | 3 |

- Table 7-4 – cbp_yuv_split_trans codeword index mapping (flagPattern=11, 13,15)

| Luma | CbCr | Split | Intra (vlc14) | Inter (unary vlc) |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 4 | 0 |
| 0 | 0 | 1 | 5 | NA |
| 0 | 1 | 0 | 6 | 5 |
| 0 | 1 | 1 | 7 | 6 |
| 1 | 0 | 0 | 1 | 2 |
| 1 | 0 | 1 | 2 | 1 |
| 1 | 1 | 0 | 0 | 4 |
| 1 | 1 | 1 | 3 | 3 |

- Table 7-5 – cbp_yuv_split_trans codeword index mapping (flagPattern=14)

| Luma | Cb | Cr | Intra trafoDepth = 0 (unary vlc) | Inter trafoDepth = 0 (unary vlc) | Intra trafoDepth > 0 (unary vlc) | Inter trafoDepth > 0 (unary vlc) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 2 | 0 | 4 | 0 |
| 0 | 0 | 1 | 0 | 1 | 6 | 6 |
| 0 | 1 | 0 | 6 | 6 | 5 | 5 |
| 0 | 1 | 1 | 4 | 3 | 7 | 7 |
| 1 | 0 | 0 | 5 | 4 | 0 | 1 |
| 1 | 0 | 1 | 3 | 2 | 3 | 4 |
| 1 | 1 | 0 | 7 | 7 | 2 | 2 |
| 1 | 1 | 1 | 1 | 5 | 1 | 3 |