| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E405 |
| --- | --- |

| Title: | Cross Verification of JCTVC-E042 Proposed by MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Qiu Shen Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | shenqiu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the crosschecking procedures and results for MediaTek’s proposal in JCTVC-E042.

# Test condition

The proposed fine granularity slices implemented into HM2.0. The implementation conforms to the description in JCTVC-E042.

The test conditions specified in JCTVC-E042 are used for the simulation. VS2005 x32 compiler is used to generate the executable file from the provided source code. Simulations were done on computers running Windows Server 2008 x64 OS.

However, due to the limited computational resources, the encoding tasks are dispatched to machines of different type. So encoding and decoding time are not provided in the test results.

# Test results

The executables generated from the provided source code can be used for the encoding and decoding test. The decoded sequences match the reconstructed sequences.

The RD data provided by MediaTek match the results we obtained.

The results are described as follows.

Table 1  Results of the proposed method for LCU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.0 | 3.6 | 4.3 | 4.9 | 4.9 | 5.1 |
| Class B | 4.9 | 4.1 | 4.6 | 5.5 | 5.4 | 5.4 |
| Class C | 5.3 | 4.6 | 4.6 | 5.8 | 5.7 | 5.5 |
| Class D | 3.7 | 3.4 | 3.5 | 4.0 | 3.4 | 3.6 |
| Class E | 7.4 | 7.7 | 6.5 | 7.7 | 10.4 | 9.6 |
| All | 4.9 | 4.5 | 4.6 | 5.5 | 5.7 | 5.7 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.8 | 9.7 | 9.0 | 5.4 | 5.9 | 6.7 |
| Class B | 4.7 | 5.8 | 5.5 | 4.8 | 5.0 | 4.7 |
| Class C | 4.2 | 4.9 | 4.9 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.9 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.4 | 5.8 | 5.6 | 4.3 | 4.3 | 4.5 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.1 | 5.5 | 3.6 | 3.7 | 3.5 |
| Class C | 2.8 | 5.0 | 5.0 | 2.8 | 2.6 | 2.3 |
| Class D | 1.7 | 2.5 | 2.4 | 1.8 | 1.6 | 2.0 |
| Class E | 1.7 | 2.4 | 1.5 | 1.2 | 3.1 | 3.0 |
| All | 2.6 | 4.6 | 3.9 | 2.5 | 2.8 | 2.7 |

Table 2  Results of the current HM method for LCU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.3 | 3.8 | 4.5 | 5.1 | 5.1 | 5.5 |
| Class B | 5.3 | 4.6 | 5.0 | 5.8 | 5.8 | 5.8 |
| Class C | 5.8 | 5.2 | 5.2 | 6.2 | 6.1 | 5.9 |
| Class D | 4.1 | 3.8 | 4.0 | 4.3 | 3.8 | 4.1 |
| Class E | 7.8 | 8.1 | 6.9 | 8.0 | 10.9 | 10.0 |
| All | 5.3 | 4.9 | 5.0 | 5.8 | 6.1 | 6.1 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.8 | 9.7 | 9.0 | 5.4 | 5.9 | 6.7 |
| Class B | 4.8 | 5.8 | 5.5 | 4.8 | 5.0 | 4.7 |
| Class C | 4.2 | 4.9 | 4.9 | 4.2 | 4.2 | 4.1 |
| Class D | 2.6 | 2.9 | 3.0 | 2.5 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 4.4 | 5.8 | 5.6 | 4.3 | 4.3 | 4.5 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.2 | 5.5 | 3.6 | 3.7 | 3.5 |
| Class C | 2.8 | 5.0 | 5.0 | 2.8 | 2.6 | 2.3 |
| Class D | 1.7 | 2.5 | 2.4 | 1.8 | 1.6 | 2.0 |
| Class E | 1.7 | 2.5 | 1.5 | 1.2 | 3.1 | 3.0 |
| All | 2.6 | 4.6 | 3.9 | 2.5 | 2.8 | 2.7 |

Table 3  Results of the proposed method for leaf-CU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.2 | 3.8 | 4.5 | 5.1 | 5.1 | 5.5 |
| Class B | 5.2 | 4.5 | 5.0 | 5.8 | 5.8 | 5.8 |
| Class C | 5.8 | 5.1 | 5.1 | 6.2 | 6.1 | 5.9 |
| Class D | 4.1 | 3.8 | 3.9 | 4.3 | 3.8 | 4.1 |
| Class E | 7.7 | 8.0 | 6.8 | 8.0 | 10.9 | 10.0 |
| All | 5.3 | 4.9 | 4.9 | 5.8 | 6.1 | 6.1 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 5.9 | 9.6 | 8.5 | 5.4 | 6.0 | 6.9 |
| Class B | 4.9 | 6.1 | 5.4 | 5.0 | 5.3 | 5.1 |
| Class C | 4.3 | 5.3 | 5.2 | 4.4 | 4.4 | 4.2 |
| Class D | 2.7 | 2.9 | 3.4 | 2.7 | 2.5 | 2.7 |
| Class E |  |  |  |  |  |  |
| All | 4.5 | 6.0 | 5.6 | 4.4 | 4.6 | 4.7 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.6 | 7.2 | 5.4 | 3.7 | 3.8 | 3.6 |
| Class C | 2.9 | 4.9 | 5.1 | 2.8 | 2.6 | 2.4 |
| Class D | 1.7 | 3.3 | 2.6 | 1.8 | 1.8 | 2.0 |
| Class E | 1.7 | 1.8 | 2.1 | 1.3 | 3.8 | 3.1 |
| All | 2.6 | 4.7 | 4.0 | 2.5 | 3.0 | 2.8 |

Table 4  Results of 2-level end-of-slice flags in JCTVC-E043 for leaf-CU-aligned 1500-byte slices

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.3 | 3.8 | 4.5 | 5.1 | 5.1 | 5.5 |
| Class B | 5.3 | 4.6 | 5.0 | 5.8 | 5.8 | 5.8 |
| Class C | 5.8 | 5.2 | 5.2 | 6.2 | 6.1 | 5.9 |
| Class D | 4.1 | 3.8 | 4.0 | 4.3 | 3.8 | 4.1 |
| Class E | 7.8 | 8.1 | 6.9 | 8.0 | 10.9 | 10.0 |
| All | 5.3 | 4.9 | 5.0 | 5.8 | 6.1 | 6.1 |

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 6.0 | 9.7 | 8.6 | 5.4 | 6.0 | 6.9 |
| Class B | 5.0 | 6.2 | 5.5 | 5.0 | 5.3 | 5.1 |
| Class C | 4.4 | 5.4 | 5.3 | 4.4 | 4.4 | 4.2 |
| Class D | 2.9 | 3.0 | 3.5 | 2.7 | 2.5 | 2.7 |
| Class E |  |  |  |  |  |  |
| All | 4.6 | 6.1 | 5.7 | 4.4 | 4.6 | 4.7 |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 3.7 | 7.3 | 5.5 | 3.7 | 3.8 | 3.6 |
| Class C | 3.0 | 5.0 | 5.1 | 2.8 | 2.6 | 2.4 |
| Class D | 1.8 | 3.4 | 2.7 | 1.8 | 1.8 | 2.0 |
| Class E | 1.9 | 1.9 | 2.3 | 1.3 | 3.8 | 3.1 |
| All | 2.7 | 4.8 | 4.1 | 2.5 | 3.0 | 2.8 |

# Conclusion

Huawei has crosschecked the proposal of MediaTek in JCTVC-E042, and the RD performance provided by MediaTek match the results we obtained.

- References

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).