| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E407 |
| --- | --- |

| Title: | HM2.0 Chroma Intra Coding Description for WD2 |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Xun Guo1 Ali Tabatabai2 Oscar Au3 | Email: | xun.guo@mediatek.com ali.tabatabai@am.sony.com ee.au@ust.hk |  |
| Source: | MediaTek Inc. 1, Sony2, HKUST3 |  |  |  |

_____________________________

# Abstract

This document describes the chroma intra prediction mode coding in HM2.0 for WD2 based on the adopted technologies in JCTVC-D255, JCTVC-D166 and JCTVC-D278.

# Introduction

In this document, the proposed descriptions of WD2 based on the adopted chroma intra mode coding techniques in JCTVC-D255, JCTVC-D166 and JCTVC-D278 are listed. The corresponding subclauses are highlighted with yellow color.

# Proposed Descriptions

7.3.7 Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( !prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( chroma_format_idc != 0 && (PartMode == 2Nx2N ¦¦ PartIdx == 3)) |  |
| Intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_MERGE, MODE_INTER */ |  |
| …… |  |
| } |  |
| } |  |

7.4.6 Coding unit semantics

intra_chroma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode for chroma samples of the current block. The value shall be in the range from 0 to 4 (0: vertical prediction, 1: horizontal prediction, 2: DC prediction, 3: diagonal down right prediction and 4: the same prediction mode with luma samples).

#### 8.3.3.1 Intra chroma sample predictions

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable intraPredMode specifying the luma intra prediction mode,

–	a variable intraPredModeC specifying the chroma intra prediction mode.

Output of this process is:

–	the predicted chroma samples.

Both chroma components (Cb and Cr) of the current block shall use the same prediction mode.

At most 5 prediction modes are used for chroma samples including 4 fixed prediction modes and 1 adaptive prediction mode. If intraPredModeC is equal to one of the values from 0 to 3, the intra prediction processes specified in subclause 8.3.2.1.1, 8.3.2.1.2, 8.3.2.1.3 and 8.3.2.1.4 shall be invoked with intraPredModeC  as input.

Otherwise, if intraPredModeC  is equal to 4, the intra prediction processes specified in subclause 8.3.2.1.1, 8.3.2.1.2, 8.3.2.1.3 and 8.3.2.1.4 shall be invoked with the intraPredMode as input.

Table 8-7 specifies the value for intraPredModeC and the associated names.

Table 8-7 Specifications of intra prediction modes for chroma samples

| - intraPredModeC | - Name of intraPredModeC |
| --- | --- |
| - 0 | - vertical prediction |
| - 1 | - horizontal prediction |
| - 2 | - DC prediction |
| - 3 | - diagonal prediction (down right) |
| - 4 | - same mode with luma samples |

9.1.2 Mapping process for chroma intra prediction mode

Inputs to this process are:

- bits for intra_chroma_pred_mode[ xB ][ yB ]

- intraPredMode specifying the corresponding luma intra prediction mode of the current block.

Output of this process is the value of intra_chroma_pred_mode[ xB ][ yB ]

Table 9-4 shows the coding tables of intra_chroma_pred_mode[ xB ][ yB ] depending on the value of intraPredMode.

According to different intra luma prediction modes, different coding tables shall be used in this process.

Table 9-4 Binarization for intra_chroma_pred_mode[ xB ][ yB ]

| 9.3 CABAC parsing process for slice data - intra_chroma_pred_mode | - intra_luma_pred_mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | - 0 | - 1 | - 2 | - 3 | - Others (4..33) |
| - 0 (vertical) | - n/a | - 10 | - 10 | - 10 | - 10 |
| - 1(horizontal) | - 10 | - n/a | - 111 | - 111 | - 110 |
| - 2 (DC) | - 111 | - 111 | - n/a | - 110 | - 1111 |
| - 3 (diagonal down right) | - 110 | - 110 | - 110 | - n/a | - 1110 |
| - 4 (same as luma) | - 0 | - 0 | - 0 | - 0 | - 0 |

9.3.1.3 Initialization process for context variables

This process is invoked before decoding the first LCU of a slice.

Outputs of this process are the initialised CABAC context variables for  intra_chroma_pred_mode[ xB ][ yB ].

In table 9-5, the initialisation values of variables m and n for ctxIdx of intra_chroma_pred_mode[ xB ][ yB ].

Table 9-5 Values of variables m and n for ctxIdx of intra_chroma_pred_mode[ xB ][ yB ]

| Value of cabac_init_idc | Initialisation variables | ctxIdx |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 |
| 0 | m | 0 | 0 | 0 | 0 |
|  | n | 64 | 64 | 64 | 64 |
| 1 | m | 0 | 0 | 0 | 0 |
|  | n | 64 | 64 | 64 | 64 |
| 2 | m | 0 | 0 | 0 | 0 |
|  | n | 64 | 64 | 64 | 64 |

9.3.2.2 Binarization process for the syntax element intra_chroma_pred_mode[ xB ][ yB ]

Inputs to this process are

- a request for a binarization for the syntax element intra_chroma_pred_mode[ xB ][ yB ]

- intraPredMode specifying corresponding luma intra prediction mode.

Output of this process is the binarization of the syntax element.

The binarization for intra_chroma_pred_mode is given by table 9-4 depending on intraPredMode.

9.3.2.2.1 Decoding process for intra_chroma_pred_mode[ xB ][ yB ]

Inputs to this process are:

- A binarization of the syntax element intra_chroma_pred_mode[ xB ][ yB ].

- intraPredMode specifying the corresponding luma prediction mode of the current block.

Output of this process is the value of intra_chroma_pred_mode[ xB ][ yB ].

Table 9-4 shows the coding tables of intra_chroma_pred_mode[ xB ][ yB ] depending on the value of intraPredMode.

According to different intra prediction modes of luma samples, different coding tables shall be used in this process.

9.3.3.1.1 Derivation process of ctxIdxInc for intra_chroma_pred_mode[ xB ][ yB ]

Inputs to this process are:

- A binIdx specifying the number of the current bin.

- Variable arrays IntraPredModeN (if available) that are previously derived of adjacent coding units.

Output of this process is ctxIdxInc.

Let the variable condTermFlagN (with N being either A or B) be derived as follows.

- If A is coded in intra prediction mode and intraPredModeA is equal to 4, condTermFlagA is set equal to 1; otherwise, condTermFlagA is set equal to 0.

- If B is coded in intra prediction mode and intraPredModeB is equal to 4, condTermFlagB is set equal to 1; otherwise, condTermFlagB is set equal to 0.

The variable ctxIdxInc of the current block is derived by

ctxIdxInc = ( binIdx == 0 ) ? ( condTermFlagA + condTermFlagB ) : 3

# Harmonization of a minor difference in JCTVC-D255/D278/D166

In the fourth JCTVC meeting at Daegu, the proposals JCTVC-D255/D278/D166 on chroma intra mode coding were identified to be almost identical and were adopted in HM2 (or WD2). However, there was a minor difference among JCTVC-D255/D278/D166 that was found prior to the software integration to HM2.0. The difference was that the last two VLC codewords for intra_chroma_pred_mode (for different intra_chroma_pred_mode) in JCTVC-D255 and JCTVC-D278 were swapped in comparison to those in JCTVC-D166. The VLC table for intra_chroma_pred_mode in JCTVC-D255 and JCTVC-D278 is shown in Table 4-1. The VLC table for intra_chroma_pred_mode in JCTVC-D166 is shown in Table 4-2.

Table 4-1 The VLC table for intra_chroma_pred_mode (for different intra_chroma_pred_mode) in JCTVC-D255 and JCTVC-D278

| intra_chroma_pred_mode | intra_luma_pred_mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | others |
| 0 (vertical) | n/a | 10 | 10 | 10 | 10 |
| 1(horizontal) | 10 | n/a | 110 | 110 | 110 |
| 2 (DC) | 110 | 110 | n/a | 111 | 1110 |
| 3 (diagonal down right) | 111 | 111 | 111 | n/a | 1111 |
| 4 (same as luma) | 0 | 0 | 0 | 0 | 0 |

Table 4-2 The VLC table for intra_chroma_pred_mode (for different intra_chroma_pred_mode) in JCTVC-D166

| intra_chroma_pred_mode | Intra_luma_pred_mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | others |
| 0 (vertical) | n/a | 10 | 10 | 10 | 10 |
| 1(horizontal) | 10 | n/a | 111 | 111 | 110 |
| 2 (DC) | 111 | 111 | n/a | 110 | 1111 |
| 3 (diagonal down right) | 110 | 110 | 110 | n/a | 1110 |
| 4 (same as luma) | 0 | 0 | 0 | 0 | 0 |

The experimental results using Table 4-1 are shown in Table 4-3 and those using Table 4-2 are shown in Table 4-4. Since the code lengths are the same in Table 4-1 and Table 4-2, there is no effect to the coding results for low-complexity settings. The change does affect the high-efficiency cases because it affects the context statistics in CABAC. For the high-efficiency all-intra cases, the results using Table 4-2 shows 0.1% of BD-rate gain for luma and 0.2% of BD-rate gain for chroma when compared to the results using Table 4-1. Thus, Table 4-2 was used for the HM2.0 software integration of the adopted chroma intra mode coding.

Table 4-3 The experimental results using VLC table in Table 4-1 for intra_chroma_pred_mode

|  | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A | -0.8 | 0.0 | 0.3 | -0.6 | -0.3 | -0.1 |
| Class B | -0.7 | 0.1 | 0.1 | -0.6 | -0.5 | -0.4 |
| Class C | -0.7 | 0.1 | 0.2 | -0.7 | -0.5 | -0.4 |
| Class D | -0.6 | 0.2 | 0.3 | -0.5 | -0.3 | -0.3 |
| Class E | -1.1 | -0.5 | -0.2 | -0.5 | -0.1 | -0.2 |
| All | -0.7 | 0.0 | 0.1 | -0.6 | -0.4 | -0.3 |

Table 4-4 The experimental results using VLC table in Table 4-2 for intra_chroma_pred_mode

|  | Intra High Efficiency |  |  | Intra Low Complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) | Y BD-rate (%) | U BD-rate (%) | V BD-rate (%) |
| Class A | -0.9 | -0.5 | -0.3 | -0.6 | -0.3 | -0.1 |
| Class B | -0.7 | -0.1 | -0.1 | -0.6 | -0.5 | -0.4 |
| Class C | -0.7 | -0.1 | 0.0 | -0.7 | -0.5 | -0.4 |
| Class D | -0.6 | 0.0 | 0.0 | -0.5 | -0.3 | -0.3 |
| Class E | -1.0 | -0.6 | -0.4 | -0.5 | -0.1 | -0.2 |
| All | -0.8 | -0.2 | -0.1 | -0.6 | -0.4 | -0.3 |

# Patent rights declaration(s)

MediaTek Inc., Sony Corp. and HKUST may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The VLC table 4 in JCTVC-D255 actually has 0 and 1 swapped when compared to the Table 4-1 here.

The Fig. 1 in JCTVC-D278 is equivalent to the Table 4-1 here.