| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E408_r1 |
| --- | --- |

| Title: | Tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arild Fuldseth Michael Horowitz Shilin Xu Minhua Zhou | Tel: Email: Tel: Email: Tel: Email: | +47 92414082 arild.fuldseth@cisco.com +1 778.331.3432 x323 michael@ebriskvideo.com +1 214.480.3816 zhou@ti.com |
| Source: | Cisco Systems, eBrisk Video, and Texas Instruments |  |  |

_____________________________

# Abstract

This contribution introduces a coding technique called Tiles that partition a picture into rectangular segments. The technique combines ideas from the generalized slices concept introduced in JCTVC-D378 with tiles introduced in JCTVC-D227 and sub-pictures introduced in JCTVC-B062. Tiles comprises vertical and horizontal boundaries that partition a picture into columns and rows respectively. Column and row boundaries break prediction mechanisms (e.g., intra prediction and motion vector prediction) in the same way as slice boundaries unless indicated otherwise. Results from three experiments are presented comparing coding efficiency for Tiles against an identically configured encoder using slices with LCUs processed in raster scan-order within a picture. Results for MTU size matching, Tiles is shown to provide 0.7%, 0.4%, and 0.3% coding gains for Intra, Random Access, and Low Delay respectively relative to the anchor. Results for high-level parallel processing, show that Tiles provide 1.6%, 3.0%, and 4.4% coding gains for Intra, Random Access, and Low Delay respectively relative to the anchor. Finally, for memory constrained motion estimation using four tiles provided an average of 12.5%, and 3.4% Y BD-rate advantage for the Random Access and Low Delay cases respectively compared against a memory constrained anchor.

# Introduction

Partitioning a picture into slices as part of the encoding process is known to negatively impact coding efficiency particularly when the slices are designed to be independently decodable. Many applications and implementations, however, require the partitioning of a picture. Two important examples, network maximum transmission unit (MTU) size matching and high-level parallel processing, are described in detail in [1]. A third important example is described in [2]. Tiles proposed in this contribution provide picture-partitioning functionality while minimizing the coding efficiency loss resulting from those partitions.

Tiles brings together the generalized slices concept introduced in [1] with tiles introduced in [3] and sub-pictures from [2]. The resulting technique preserves the simplicity of the three underlying methods while facilitating MTU size matching, high-level parallel processing, and memory-efficient motion estimation over a broad range of application scenarios. Tiles comprises vertical and horizontal boundaries that partition a picture into columns and rows respectively. These boundaries break prediction mechanisms (e.g., intra prediction and motion vector prediction) in the same way as slice boundaries unless otherwise indicated. The rectangular regions resulting from the intersecting column and row boundaries are called tiles (hence the name for the technique as a whole). LCUs are processed in raster scan order within each tile and the tiles themselves are processed in raster scan order within the picture (see Figures 1 and 2). Slice boundaries are introduced by the encoder and need not be coincident with tile boundaries.  For example, a tile may contain more than one slice and slices may contain more than one tile. When a slice contains LCUs in more than one tile, the tiles containing the LCUs shall be contiguous. Two tiles, tiles T and T+1, are said to be contiguous if the first LCU in tile T+1 is one greater than the last LCU in tile T.

Tiles allows the column and row boundaries to be specified with and without uniform spacing. Uniform spacing of column and row boundaries is useful for simple picture partitioning to facilitate processor load balancing for high-level parallelization and for improving video quality under the constraint of limited on-chip memory size. Non-uniform spacing of column and row boundaries provides greater partition flexibility that may be useful for error resilience applications (e.g., to isolate more important areas in a picture) and to provide more precise processor load balancing (equal picture area does not always equate to an equal processing load).

This contribution is organized as follows. Section 2 contains a description of the Tiles syntax, semantics, and decoding process.  In Section 3, three experiments are described.  Section 4 contains a summary of the experiment results. The conclusion is contained in Section 5 and references in Section 6.

Figure 1. An example of Tiles partitioning using three columns and three rows.

Figure 2. An example of Tiles partitioning using three columns and zero rows.

# Test model modifications for Tiles

To implement Tiles, the following decoder modifications were made to the HM 2.0-dev-slices code base:

- modification of the LCU decoding order from raster scan within a picture to raster scan within a tile and raster scan of tiles within a picture,

- modification of CU availability to recognize that a CU in a different tile is not available (unless indicated otherwise),

- addition of the syntax and semantics to decode column widths, row heights, and compute column and row boundary locations including an option to indicate uniform spacing, and the

- addition of  a syntax element indicating  whether row and tile boundaries break prediction mechanisms or not.

Each of these modifications is discussed in the sub-sections that follow. Analogous changes were made to the encoder source code.

## Decoding order

The decoding order for Tiles was modified so that LCUs are decoded in raster scan order within tiles and tiles are themselves decoded in raster scan order within the picture as described in the introduction.

## CU availability

CU availability for Tiles is the same as that for HM 2.0-dev-slices except that CUs in a different tile are not available.   Possible generalizations to CU availability such as allowing availability from one side of a column or row boundary and not the other as proposed in [4] are possible.

## Coding of column widths and row heights

The Sequence and Picture Parameter Sets syntax in the reference software was modified to convey column boundaries, row heights in two distinct ways.  In addition, syntax to indicate that tile boundaries shall not break prediction mechanisms has been added. The syntax is shown explicitly in the following sub-section. Semantics are presented in sub-section 2.3.2. All syntax and semantics changes to the HM related to Tiles are described in this section.

### Tiles related syntax

Tiles related syntax is located in both the Sequence and Picture Parameter Sets as described in the following two sub-sections.   When present, parameters in the Picture Parameter Set take precedence over those in the Sequence Parameter Set.

#### Sequence Parameter Set

Syntax added to the Sequence Parameter Set of the reference software to support Tiles is highlighted in yellow in Table 1. It may be desirable to have the code length be level dependent.  Further, we envisage a normative level-dependent minimum column width specification to prevent long, narrow vertical tiles.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| bit_depth_luma_increment | ue(v) |
| bit_depth_chroma_increment | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| interpolation_filter_flag | u(1) |
| tile_boundary_independence_idr | u(1) |
| uniform_spacing_idr | u(1) |
| if (uniform_spacing_idr == 1) { |  |
| column_width_minus1 | ue(v) |
| row_height_minus1 | ue(v) |
| } |  |
| else { |  |
| num_columns_minus1 | ue(v) |
| num_rows_minus1 | ue(v) |
| for (i=0; i < num_columns_minus1; i++) |  |
| column_width[i] | ue(v) |
| for (i=0; i < num_rows_minus1; i++) |  |
| row_height[i] | ue(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 1. Tiles related syntax in the sequence parameter set highlighted in yellow.

#### Picture Parameter Set

Syntax added to the Picture Parameter Set of the reference software to support Tiles is highlighted in yellow in Table 2. It may be desirable to have the code length will be level dependent.  Further, we envisage a normative level-dependent minimum column width specification to prevent vertically elongated tiles.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| column_row_info_present | u(1) |
| if (column_row_info_present == 1) { |  |
| tile_boundary_independence_idr | u(1) |
| uniform_spacing_idr | u(1) |
| if (uniform_spacing_idr == 1) { |  |
| column_width_minus1 | ue(v) |
| row_height_minus1 | ue(v) |
| } |  |
| else { |  |
| num_columns_minus1 | ue(v) |
| num_rows_minus1 | ue(v) |
| for (i=0; i < num_columns_minus1; i++) |  |
| column_width[i] | ue(v) |
| for (i=0; i < num_rows_minus1; i++) |  |
| row_height[i] | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Table 2. Tiles related syntax in the picture parameter set highlighted in yellow.

### Tiles related semantics

The semantics associated with syntax elements introduced in section 2.1.1 are presented in this sub-section.

num_columns_minus1 indicates the number of column boundaries in the picture (i.e., the number of columns minus one).

num_rows_minus1 indicates the number of row boundaries in the picture (i.e., the number of rows minus one).

uniform_spacing_idr when uniform_spacing_idr equals 1, the distance between every column boundary shall be uniform, except possibly at the right picture boundary. The distance between column boundaries is indicated by column_width_minus1. In addition, the distance between every row boundary shall be uniform except possibly at the bottom picture boundary. The distance between row boundaries is indicated by row_height_minus1. Otherwise when uniform_spacing_idr equals 0, the distance between column boundaries is indicated by column_width[] (when num_columns_minus1 > 0) and the distance between row boundaries is indicated by row_height[] (when num_rows_minus1 > 0). column_width_minus1 plus 1 specifies the column width in units of LCUs. The variable for column width in units of LCUs is derived as follows

ColumnWidth = column_width_minus1 + 1

row_height_minus1 plus 1 specifies the row height in units of LCUs. The variable for row height in units of LCUs is derived as follows

RowHeight = row_height_minus1 + 1

tile_boundary_independence_idr when tile_boundary_independence_idr equals 1, all column and row boundaries break prediction mechanisms (e.g., intra prediction and motion vector prediction). Otherwise, when tile_boundary_independence_idr equals 0, row and column boundaries in pictures associated with the picture parameter set do not break prediction mechanisms.

column_width[i] specifies the width of the ith column in units of LCUs.  The last column width equals the number of LCUs between the right-most column boundary and the right picture edge.

row_height[i] specifies the height of the ith row in units of LCUs. The last row height equals the number of LCUs between the bottom-most row boundary and the bottom picture edge.

### Tiles column and row boundary decoding process

The derivation of column and row boundaries for the uniform and more general non-uniform columns and row spacing cases are described in this sub-section.

#### Uniform column and row spacing

When uniform_spacing_idr equals 1, the decoder uses the ColumnWidth and RowHeight values to derive column and row boundaries respectively as shown below. The variable TbSize (=64) is used in [5] for the LCU (=treeblock) size.

// column boundary derivation

columnBoundary[0] = ColumnWidth;

for (i=1; i < pic_width_in_luma_samples/TbSize; i+=ColumnWidth) {

columnBoundary[i] = columnBoundary[i-1] + ColumnWidth;

}

// row boundary derivation

rowBoundary[0] = RowHeight;

for (i=1; i < pic_height_in_luma_samples/TbSize; i+=RowHeight) {

rowBoundary[i] = rowBoundary[i-1] + RowHeight;

}

#### Non-uniform column and row spacing

When uniform_spacing_idr equals 0, the decoder uses the column_width[] and row_height[] values to derive the column and row boundaries respectively as shown below.

// column boundary derivation

if (num_columns_minus1 > 0) {

columnBoundary[1] = column_width[0];

for (i=1; i < num_columns_minus1; i++) {

columnBoundary[i+1] = columnBoundary[i] + column_width[i];

}

}

// row boundary derivation

if (num_rows_minus1 > 0) {

rowBoundary[1] = row_height[0];

for (i=1; i < num_rows_minus1; i++) {

rowBoundary[i+1] = rowBoundary[i] + row_height[i];

}

}

# Description of the experiments

Experiments exploring the effects of tiles on coding efficiency for MTU-size matching, high-level parallel processing and on-chip memory constrained motion estimation applications are described in this section.

## Common conditions

The reference software HM 2.0-dev-slices was used to produce results for the reference in the experiment.  In addition, the encoder and decoder source code from the reference codebase was modified to introduce Tiles functionality. The common conditions and encoding configurations described in [6] were used except where indicated otherwise.

## Experiment 1: MTU size matching

In this experiment, MTU size matching is considered. The HM2.0-dev-slices software [7] version 609  using the common conditions served as reference. The reference was compared against Tiles, also using the common conditions.  In both cases, a maximum number of bytes per slice were specified in the encoder configuration file.  The maximum number of bytes was set to 1500 to be consistent with the MTU size of an Ethernet v2 network. For Tiles, column boundaries and no row boundaries were introduced.  The encoder introduced slices as required to meet the 1500 byte per slice constraint. The number of rows was set to zero.  The column widths were selected according to simulation type (e.g., Intra), sequence class, and QP value to achieve approximately square slice shapes.  The column widths used for the Tiles simulation are shown in Table 3.

|  | Intra | Intra LoCo | Random Access | Random Access Loco | Low Delay | Low Delay LoCo |
| --- | --- | --- | --- | --- | --- | --- |
| Class A |  |  |  |  |  |  |
| QP 22 | 5 | 2 | 5 | 4 | xxx | xxx |
| QP 27 | 5 | 2 | 7 | 6 | xxx | xxx |
| QP 32 | 7 | 3 | 10 | 8 | xxx | xxx |
| QP 37 | 7 | 4 | 10 | 10 | xxx | xxx |
| Class B |  |  |  |  |  |  |
| QP 22 | 5 | 5 | 6 | 6 | 4 | 6 |
| QP 27 | 5 | 5 | 6 | 6 | 8 | 8 |
| QP 32 | 6 | 6 | 8 | 8 | 10 | 10 |
| QP 37 | 8 | 8 | 10 | 10 | 15 | 15 |
| Class C |  |  |  |  |  |  |
| QP 22 | 1 | 1 | 4 | 3 | 3 | 3 |
| QP 27 | 2 | 2 | 7 | 4 | 4 | 4 |
| QP 32 | 3 | 3 | 7 | 7 | 7 | 7 |
| QP 37 | 4 | 4 | 13 | 13 | 13 | 13 |
| Class D |  |  |  |  |  |  |
| QP 22 | 1 | 1 | 4 | 4 | 4 | 2 |
| QP 27 | 2 | 2 | 7 | 7 | 4 | 4 |
| QP 32 | 2 | 2 | 7 | 7 | 7 | 7 |
| QP 37 | 4 | 4 | 7 | 7 | 7 | 7 |
| Class E |  |  |  |  |  |  |
| QP 22 | 4 | 4 | xxx | xxx | 7 | 7 |
| QP 27 | 5 | 5 | xxx | xxx | 20 | 20 |
| QP 32 | 5 | 5 | xxx | xxx | 20 | 20 |
| QP 37 | 7 | 7 | xxx | xxx | 20 | 20 |

Table 3. Column widths in units of LCUs used in Experiment #1.

## Experiment 2: High-level parallel processing

In this experiment, high-level parallel processing is considered without MTU size matching. The HM 2.0-dev-slices software using the common conditions served as reference. The reference was compared against Tiles, also using the common conditions.  In the Tiles case, a fixed number of LCUs were configured for each tile (except where picture boundaries prevented it).  The tile sizes for each sequence class were chosen so as to contain roughly the same number of pixels as one picture in a Class D sequence. In the reference case, LCUs were assigned to each slice in raster scan order within the picture and with an integer number of LCU rows per slice. The slice and tile partitioning for each class is shown in Table 4 for the reference case and the test case respectively.

|  | Reference |  | Test |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Number of slices | Slice sizes (in units of LCUs) | Number of tiles |  | Tile dimensions (in units of LCUs) |  |
|  |  |  | horizontally | vertically | horizontally | vertically |
| A | 25 | 40 | 5 | 5 | 8 | 5 |
| B | 17 | 30 | 6 | 3 | 5 | 6 |
| C | 4 | 26 | 2 | 2 | 7 | 4 |
| D | 1 | 28 | 1 | 1 | 7 | 4 |
| E | 12 | 20 | 4 | 3 | 5 | 4 |

Table 4. Slice and tile partitioning for Experiment 2.

## Experiment 3: Motion estimation with an on-chip memory constraint

This experiment explores the effect of using uniformly spaced tiles to improve motion estimation under the constraint of limited on-chip memory size. Specifically, partitioning a picture into uniformly-sized tiles and processing LCUs in raster-scan order within tiles allows for an increased motion estimation search range compared with processing LCUs in raster scan order within a picture when the amount of memory used for motion search is constrained. In this experiment, a picture is divided into N x 1 tiles where N differs among the sequence classes as shown in Figure 3.  In both the reference and the test cases, the memory size for motion search is constrained to be equal, but the search ranges differ as a result of the difference in LCU scan order. One slice per picture is used in the reference and test cases.

Figure 3. Growing search window is applied to each tile. SRx and SRy represent the search range relative to the current LCU row of tiles.

In motion estimation with uniformly spaced tiles, a growing window search is assumed, motion vector candidates are eliminated for merge, skip mode, and for the initial motion vector predictor selection of regular motion search when they are outside the growing search window (see Figure 3). The refinement search range for regular motion search is also adjusted on the fly for each PU based on the growing search window size and the PU location.

### Test sequences

Two sets of test sequences were selected, one with high vertical motion, the other with high horizontal motion. The test sequences are described in Table 5. The test sequences in the common test conditions were not used for this experiment because they lack vertical motion.

| Sequence name | Picture resolution | Frame-rate | Number of frames | Motion characteristics |
| --- | --- | --- | --- | --- |
| FV_seq1 | 1920 x 816 | 24 | 60 | High vertical |
| FV_seq2 | 1920 x 816 | 24 | 60 | High vertical |
| FV_seq3 | 1920 x 816 | 24 | 59 | High vertical |
| FH_seq1 | 1920 x 1080 | 30 | 60 | High horizontal |
| FH_seq2 | 1920 x 1080 | 24 | 58 | High horizontal |
| FH_seq3 | 1920 x 1080 | 24 | 60 | High horizontal |
| FH_seq4 | 1920 x 1080 | 30 | 60 | High horizontal |
| FH_seq5 | 1920 x 1080 | 30 | 60 | High horizontal |
| FH_seq6 | 1920 x 1080 | 24 | 30 | High horizontal |

Table 5. Test sequences used in experiment #3.

### Test conditions

Except for the test sequences, the common testing conditions defined in [6] were used. A single slice without restricted search range was used as the anchor. The simulation was carried for tile number N   {1, 2 and 4}. The simulation platform used in this experiment was LSF equipped with Intel Xeon CPU X5570 @ 2.93GHz with a 64 bit Linux operating system.

In the simulation, the on-chip memory budget available for motion estimation is set to 2 MBytes. Assuming four reference pictures in each direction, and the motion estimation performed in parallel for all the reference pictures and both directions (L0/L2), the growing widow search ranges used in the simulation are listed in Table 6. The horizontal search range is fixed to 256 samples. Note that the search ranges in Table 6 are different from the refinement search ranges specified in [6]. The search ranges defined in Table 6 specify the search window around the current LCU row in a tile as illustrated in Figure 3.

| Tile number (N) | Horizontal search range (SRx) | Vertical search range (SRy) | Relative ME memory bandwidth |
| --- | --- | --- | --- |
| 1 (anchor) | unlimited | unlimited |  |
| 1 | unlimited | 30 | 1 |
| 2 | 256 | 67 | 1.27 |
| 4 | 256 | 148 | 1.80 |

Table 6. Growing window search ranges and relative ME memory bandwidth requirements used for simulations with tile numbers (N = 1, 2, 4).

# Results

Results from the three experiments described in Section 3 are presented below.

## Experiment 1

The simulation results for MTU size matching are provided in Table 7.  The results show that Tiles provides modest gains on average.  The coding gains due to Tiles is diminished significantly as a result of the fact that in several cases, particularly for larger QP values, the number of slices per picture is small because most (or all) of the picture may be coded within the 1500 byte MTU size limit. In these cases, tile partitioning provides little or no advantage.  At the other extreme, some sequences had very few LCUs per slice.  When the number of LCU’s is less than three per slice, Tiles provide little or no gain.  Finally, ideally column widths would be specified so that a 1500 byte slice would have a square shape (for reasons detailed in [1]).  Consequently, the ideal column width would be a function of bit rate.  As a result of the QP scaling used in the Random Access and Low Delay simulations, the picture-to-picture bit rate and hence ideal column widths would vary. The column widths used in this experiment remained fixed for a given test type (e.g. Intra), sequence class, and QP value thereby introducing some inefficiency.

The results shown in Table 7 are partial and the averages include only results that were completed by the upload deadline.  In addition, this experiment was performed on Amazon’s EC2 cloud so the time measurements reported by the encoder an decoder were completely unreliable and are not reported here.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.6 | 0.1 | 0.1 | 0.1 |
| Class B | -0.6 | -0.7 | -0.6 | -0.7 | -0.9 | -0.8 |
| Class C | -0.7 | -0.7 | -0.7 | -0.7 | -0.9 | -0.8 |
| Class D | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class E | -2.4 | -2.5 | -2.8 | -2.4 | -4.2 | -3.5 |
| All | -0.7 | -0.7 | -0.6 | -0.7 | -1.0 | -0.9 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.2 | -0.3 | -0.4 | -0.5 |
| Class B | -0.8 | -0.8 | -0.7 | -0.8 | -1.0 | -0.9 |
| Class C | -0.3 | -0.2 | -0.4 | -0.4 | -0.4 | -0.4 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.3 | -0.4 | -0.4 | -0.5 | -0.5 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -0.6 | -0.7 | -0.7 | -0.9 | -0.8 |
| Class C | -0.4 | -0.6 | -0.5 | -0.5 | -0.3 | -0.6 |
| Class D | 0.0 | 0.0 | 0.3 | -0.1 | -0.3 | -0.2 |
| Class E | 0.0 | -0.2 | -0.3 | 0.0 | -0.1 | -0.1 |
| All | -0.3 | -0.4 | -0.3 | -0.3 | -0.4 | -0.5 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |

Table 7. MTU size matching results comparing a tile partitioned test case with a slice partitioned anchor.

## Experiment 2

The simulation results for high-level parallel processing are provided in Table 8.  The results show that tiles provide significant coding gains compared with partitioning a picture with slices containing LCUs in raster-scan order. In addition, the results show an improvement compared with those presented in Generalized Slices [1]. Note that in this contribution results for sequence class D reduce the overall average coding gains since the gains for those sequences are zero due to the fact that there is always one tile per picture.  Like Generalized Slices, Tiles provides gains compared with slices with raster scan LCUs because there are fewer boundaries to break prediction mechanisms. On top of this gain, tiles have the added benefit that they have no header and hence no header bit overhead. The reduced bit overhead is particularly significant for the lower bit rate Random Access and Low Delay cases. Consequently, some of the gains seen in Table 8 are due to the fact that the test case used one slice (and hence one slice header) per picture and the reference used the number of slices shown in Figure 4.  In addition, this experiment was performed on Amazon’s EC2 cloud so the time measurements reported by the encoder an decoder were completely unreliable and are not reported here.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -1.3 | -1.4 | -1.4 | -1.7 | -1.8 |
| Class B | -2.1 | -2.1 | -2.1 | -2.2 | -2.5 | -2.5 |
| Class C | -0.8 | -0.7 | -0.8 | -0.8 | -0.9 | -0.9 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | -4.7 | -5.2 | -4.9 | -5.0 | -7.3 | -6.4 |
| All | -1.6 | -1.7 | -1.7 | -1.7 | -2.2 | -2.1 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -4.2 | -4.9 | -5.4 | -4.5 | -4.7 | -5.6 |
| Class B | -5.2 | -5.1 | -4.9 | -5.2 | -5.1 | -4.9 |
| Class C | -2.1 | -2.1 | -2.4 | -2.1 | -2.1 | -2.3 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -3.0 | -3.1 | -3.3 | -3.1 | -3.1 | -3.3 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -4.4 | -4.7 | -4.3 | -4.9 | -4.7 | -4.3 |
| Class C | -1.7 | -1.8 | -1.7 | -1.7 | -1.6 | -1.5 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | -13.7 | -14.3 | -14.4 | -13.6 | -13.9 | -14.0 |
| All | -4.4 | -4.6 | -4.5 | -4.5 | -4.5 | -4.3 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |

Table 8.  Parallel processing results for a tile-partitioned (single slice) test case compared with a slice-partitioned anchor.

## Experiment 3

The simulation results for motion estimation with an on-chip memory size constraint are provided in Tables 9, 10, 11. As shown in Table 9, there is a significant quality loss (12.8% in Random Access and Random Access LoCo, 3.5% in Low Delay and 3.3% in Low Delay LoCo) for single tile coding under the on-chip memory size (search range) constraints. The loss can be almost completely mitigated when a picture is divided into four tiles (see Table 11) during encoding. Compared to single tile coding with constrained on-chip memory, the proposed Tiles scheme with four uniformly spaced tiles provided an average of 12.6%, 12.4%, and 3.5% and 3.3% Y BD-rate advantage for Random Access, Random Access LoCo, Low Delay, and Low Delay LoCo, respectively.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 12.8 | 15.5 | 15.6 | 12.8 | 15.3 | 15.2 |
| Class FH | 0.3 | 0.3 | 0.1 | 0.3 | 0.5 | 0.5 |
| All | 4.5 | 5.4 | 5.3 | 4.4 | 5.5 | 5.4 |
| Enc Time[%] | 92% |  |  | 92% |  |  |
| Dec Time[%] | 93% |  |  | 88% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 3.5 | 3.2 | 2.8 | 3.3 | 2.4 | 2.5 |
| Class FH | 0.1 | 0.3 | 0.1 | 0.0 | 0.1 | 0.1 |
| All | 1.2 | 1.2 | 1.0 | 1.1 | 0.9 | 0.9 |
| Enc Time[%] | 85% |  |  | 96% |  |  |
| Dec Time[%] | 81% |  |  | 100% |  |  |

Table 9. Results for one tile (N = 1) vs. a single slice, no restricted search range anchor.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 3.1 | 4.9 | 4.5 | 3.4 | 4.3 | 4.0 |
| Class FH | -0.1 | 0.1 | -0.4 | 0.0 | 0.0 | 0.0 |
| All | 1.0 | 1.7 | 1.3 | 1.2 | 1.5 | 1.3 |
| Enc Time[%] | 99% |  |  | 93% |  |  |
| Dec Time[%] | 100% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 0.2 | 0.2 | -0.1 | 0.2 | 0.3 | 0.0 |
| Class FH | 0.0 | 0.5 | 0.5 | -0.1 | -0.3 | -0.2 |
| All | 0.0 | 0.4 | 0.3 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 88% |  |  | 100% |  |  |
| Dec Time[%] | 83% |  |  | 108% |  |  |

Table 10. Results for two tiles (N = 2) vs. a single slice, no restricted search range anchor.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 0.2 | 1.7 | 0.9 | 0.4 | 0.7 | 0.6 |
| Class FH | 0.0 | 0.2 | 0.0 | 0.2 | 0.3 | 0.1 |
| All | 0.0 | 0.7 | 0.3 | 0.3 | 0.5 | 0.2 |
| Enc Time[%] | 95% |  |  | 98% |  |  |
| Dec Time[%] | 93% |  |  | 96% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class FV | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class FH | 0.1 | 0.5 | 0.3 | 0.0 | -0.2 | -0.2 |
| All | 0.1 | 0.3 | 0.2 | 0.0 | -0.2 | -0.2 |
| Enc Time[%] | 92% |  |  | 99% |  |  |
| Dec Time[%] | 88% |  |  | 102% |  |  |

Table 11. Results for four tiles (N = 4) vs. a single slice, no restricted search range anchor.

# Conclusion and recommendation

Tiles has been shown to reduce loss in coding efficiency relative to LCU’s processed in raster scan-order within a picture in situations where picture partitioning is required. Results for MTU size matching, Tiles is shown to provide 0.7%, 0.4%, and 0.3% coding gains for Intra, Random Access, and Low Delay respectively relative to the anchor. Results for high-level parallel processing, show that Tiles provide 1.6%, 3.0%, and 4.4% coding gains for Intra, Random Access, and Low Delay respectively relative to the anchor. Finally, for memory constrained motion estimation using four tiles provided an average of 12.5%, and 3.4% Y BD-rate advantage for the Random Access and Low Delay cases respectively compared against a memory constrained anchor. Further the losses when using four tiles relative to a non motion constrained reference are essentially zero.

In addition, Tiles adds no meaningful computational complexity. Further, Tiles is complementary with fine granularity slices [8], [9], entropy slices [10], and wavefront coding [11] that have been proposed in earlier meetings and the combination of tools might prove advantageous for some applications. Moreover, Tiles can be extended to support decoder parallelization, as is proposed in JCTVC-E412.  Tiles is endorsed by Cisco Systems, eBrisk Video, and Texas Instruments.  Experiments 1 and 3 were cross-verified by Sharp.  Experiment 2 was cross-verified by Qualcomm and Sharp. We recommend that Tiles be adopted to the HM.

# References

[1] M. Horowitz and S. Xu, “Generalized slices,” JCT-VC Document JCTVC-D378, Daegu, January 2011.

[2] M. Zhou, “Sub-picture based raster scanning coding order for HEVC UHD video coding”, JCTVC-B062, Geneva, July, 2010.

[3] A. Fuldseth, “Replacing slices with tiles for high level parallelism,” JCT-VC Document JCTVC-D227, Daegu, January 2011.

[4] K. Kazui, J. Koyama, A. Nakagawa, “Benefit of the new syntax and semantics for very low delay coding in HEVC”, JCTVC-D054, Daegu, January 2011.

[5] T. Wiegand, W.-J. Han, B. Bross, J.-R. Ohm, G. J. Sullivan, “Working draft 2 of high-efficiency video coding”, JCTVC-D503, Daegu, January, 2011.

[6]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-D600, Daegu, January 2011.

[7] JCT-VC, “Test Model under Consideration Software HM 2.0-dev-slices”,

http://hevc.kw.bbc.co.uk/svn/jctvc-a124/branches/HM-2.0-dev-slices/

[8] R. Sjöberg, “Fine Granularity slices”, JCTVC-C154, Guangzhou, October 2010.

[9] Q. Shen, Q. Xie, H. Yu, “Fine granularity slice partition”, JCTVC-D312, Daegu, January 2011.

[10] K. Misra, A. Segall, “Lightweight slicing for entropy coding”, JCTVC-D070, Daegu, January 2011.

[11] K. Misra, A. Segall, “New results for periodic inits for wavefront coding functionality”, JCTVC-E409, Geneva, March 2011.

# Patent rights declaration(s)

Cisco Systems, Inc., eBrisk Video, Inc. and Texas Instruments, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).