| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E409 |
| --- | --- |

| Title: | New results for periodic inits for wavefront coding functionality |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 Pacific Rim Blvd. Camas WA 98607 | Tel: Email: | +1-360-817-8487 misrak@sharpwall.com asegall@sharpwall.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document reports new results for re-initializing the CABAC engine at the start of each largest coding unit (LCU) row and transmitting the location of these re-initialization points to the decoder. The asserted benefit is wavefront processing at both the encoder and decoder, and the technique was previously proposed in JCTVC-D073. Here, results are presented with the current HM2.0 software.

# Introduction

As outlined in JCTVC-D073 [1], the initialization of the context engine at the start of every LCU row provides explicit support for wavefront processing of the bitstream. The location of LCU row starts within the bitstream is signaled in the slice header. Decoding as soon as neighbor dependencies are satisfied in conjunction with the raster-scan encoding order gives the visual appearance of a wavefront at the decoder. Note, for the additional “start of LCU row” context initializations in forward predicted B-slice we use the P-slice tables.

# Syntax and Semantics

Wavefront processing is enabled using the following syntax and semantics:

Table 1 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| regular sequence parameter set …. |  |  |
| if (entropy_coding_mode_flag) { |  |  |
| lcu_row_cabac_init_flag | 1 | u(1) |
| if( lcu_row_cabac_init_flag){ |  |  |
| lcu_row_cabac_init_idc_flag | 1 | u(1) |
| } |  |  |
| } |  |  |
| } |  |  |

“lcu_row_cabac_init_flag” flag signals if entropy decoding is initialized at the start of the largest coding unit (LCU) row.  A value 0 indicates no context initialization is carried out at the start of a LCU row. A value of 1 indicates context initialization is carried out at the start of LCU.

“lcu_row_cabac_init_idc_flag” flag signals if cabac_init_idc values will be transmitted at the start of each LCU row.   When the value of the “lcu_row_cabac_init_idc_flag” flag is “1” values will be transmitted at the start of each LCU row.  When the flag is not sent the “lcu_row_cabac_init_idc_flag” flag shall be inferred to be “0.”

Table 2 Slice header

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| a regular slice header …….. |  |  |
| if( lcu_row_cabac_init_flag ){ |  |  |
| lcu_row_locations_flag | 1 | u(1) |
| if (lcu_row_locations_flag) { |  |  |
| lcu_row_ locations () |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

- “lcu_row_locations_flag” signals if LCU row location information is being transmitted or not.  If the value of “lcu_row_locations_flag” is “1”, then the LCU row location information is transmitted, otherwise it is not transmitted. The default value for “lcu_row_locations_flag” is “0”.

Table 3 LCU row location information

| lcu_row_ locations() | C | Descriptor |
| --- | --- | --- |
| { |  |  |
| num_of_lcu_rows_minus1_slice | 2 | u(5) |
| num_of_diff_bits_used_minus1_offset | 2 | u(5) |
| for (i=0; i<num_of_lcu_rows_minus1_slice; i++) { |  |  |
| lcu_row_offset[i] | 2 | ue(v) |
| } |  |  |
| } |  |  |

- “num_of_lcu_rows_minus1_slice” specifies the number of LCU rows in the slice

- “num_of_diff_bits_used_minus1_offset” specifies the “number of bits used minus one” by each lcu_row_offset [i] entry.

- “lcu_row_offset [i]” indicates the offset of the ith  LCU row from the previous LCU row. The lcu_row_offset [i] is differentially encoded as the difference in the size of previous LCU row  from the LCU row prior to the previous LCU row. For i=0 since no LCU row prior to the previous LCU row exists the size of the previous LCU row is transmitted.

Table 4 coding unit

| coding_unit( x0, y0, currCodingUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if (x0==0 && currCodingUnitSize==MaxCodingUnitSize && mb_row_cabac_init_idc_flag==true && mb_id!=first_mb_in_slice ) { |  |  |
| cabac_init_idc_present_flag | 1 | u(1) |
| if( cabac_init_idc_present_flag) |  |  |
| cabac_init_idc | 2 | ue(v) |
| } |  |  |
| a regular coding unit … |  |  |
| } |  |  |

“cabac_init_idc_present_flag” flag may signal if a cabac_init_idc value is transmitted for the LCU.  When a cabac_init_idc value is not transmitted for the LCU then the entropy coding context is reset using the preceding value for cabac_init_idc in the bit-stream.

# Results

We implemented the proposed method into the HM2.0 software. All sequences in the high-performance configuration described in JCTVC-D600 [2] were evaluated. Full results are available in the attached XLS sheets.

# Conclusion

The context initialization method in JCTVC-D073 to enable wavefront processing with explicit location signaling within the slice header was evaluated for the new common test conditions. The system allows for parallel wavefront processing with LCUs written to the bit-stream in raster scan order.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Kiran Misra, Andrew Segall, “Periodic inits for wavefront coding functionality,” JCTVC-D073/ m18823, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[2] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-D600/m19497, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[3] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.