| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E414 |
| --- | --- |

| Title: | Cross verification of Tiles |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Andrew Segall 5750 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8487 misrak@sharplabs.com, asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution reports the cross-check of Tiles experiment 1 corresponding to MTU size matching and outlined in document JCTVC-E408. The tile dimensions used was as specified for experiment 1 (Table 3) in JCTVC-E408. For reference HM-2.0-dev-slices software version 609 with common conditions and 1500 bytes slice limit was used. The software implementation was verified and the R-D results were identical to those provided by the proponent.

# Cross-verification

The software implementation was verified to be consistent with document JCTVC-E408 [2]. The source code provided by the proponents is a modified version of the source in branch HM-2.0-dev-slices version 609 [1]. The performance of the proposed Tile experiment 1 with 1500 bytes limit on a slice (MTU size matching) was tested using Tile dimensions specified in JCTVC-E408 on conditions specified in JCTVC-D600 [3].

The encoder and decoder were run on a cluster of machines. The computing platform is summarized in Table 1 below.

Table 1: Computing platforms

| OS | Linux (64bit) |
| --- | --- |
| Some CPUs | Intel(R) Core(TM)2 CPU 6300 @ 1.86GHz cache size : 2048 KB |
| Some CPUs | Intel(R) Xeon(R) CPU X3360 @ 2.83GHz cache size : 6144 KB |
| Some CPUs | Intel(R) Core(TM)2 Extreme CPU Q6800 @ 2.93GHz cache size : 4096 KB |
| Compiler | gcc 4.3.2 |

# Results

A summary of BD Bitrate [4][5] results is presented below. The timing information is not provided due to large variability observed in timing data. The detailed R-D results can be found in attached excel sheet.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.6 | 0.1 | 0.1 | 0.1 |
| Class B | -0.6 | -0.7 | -0.6 | -0.7 | -0.9 | -0.8 |
| Class C | -0.7 | -0.7 | -0.7 | -0.7 | -0.9 | -0.8 |
| Class D | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class E | -2.4 | -2.5 | -2.8 | -2.4 | -4.2 | -3.5 |
| All | -0.7 | -0.7 | -0.6 | -0.7 | -1.0 | -0.9 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.2 | -0.3 | -0.4 | -0.5 |
| Class B | -0.8 | -0.8 | -0.7 | -0.8 | -1.0 | -0.9 |
| Class C | -0.3 | -0.2 | -0.4 | -0.4 | -0.4 | -0.4 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.3 | -0.4 | -0.4 | -0.5 | -0.5 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -0.6 | -0.7 | -0.7 | -0.9 | -0.8 |
| Class C | -0.4 | -0.6 | -0.5 | -0.5 | -0.3 | -0.6 |
| Class D | 0.0 | 0.0 | 0.3 | -0.1 | -0.3 | -0.2 |
| Class E | 0.0 | -0.2 | -0.3 | 0.0 | -0.1 | -0.1 |
| All | -0.3 | -0.4 | -0.3 | -0.3 | -0.4 | -0.5 |
| Enc Time[%] | --- |  |  | --- |  |  |
| Dec Time[%] | --- |  |  | --- |  |  |

The R-D performance observed is exactly the same as those provided by the proponent.

# Conclusion

The cross-check and software verification of “CE10: core transform proposal 1” is reported. The experimental results yield same R-D results as stated in the proponent’s result sheet.

# References

[1]JCT-VC, “Test Model under Consideration Software HM 2.0-dev-slices”,

http://hevc.kw.bbc.co.uk/svn/jctvc-a124/branches/HM-2.0-dev-slices/

[2] Arild Fuldseth, Michael Horowitz, Shilin Xu, Minhua Zhou, “Tiles,” JCTVC-E408/m19946 5th Meeting: Geneva, CH, 16-23 March, 2011.

[3] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-D600/m19497, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[4] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[5] Stéphane Pateux, Joel Jung, " An excel add-in for computing Bjontegaard metric and its evolution," 31st VCEG Meeting, Marrakech, MA, 15-16 January, 2007, Doc. VCEG-AE07.