| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E416 |
| --- | --- |

| Title: | CE10: Cross check of FastVDO/Samsung's fast integer transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kiran Misra, Louis Kerofsky, Andrew Segall SHARP 5750 NW Pacific Rim Blvd, Camas, WA, 98607, USA | Tel: Email: | +1 360 817 8487 misrak@sharplabs.com, lkerofsky@sharplabs.com, asegall@sharplabs.com, |
|  | Dzung Hoang Zenverge, Inc. | Tel: Email: | dzung.hoang@zenverge.com |
| Source: | SHARP, Zenverge, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check of the CE10: core transform proposal 1 outlined in document JCTVC-E353. The software implementation was verified and encoding-decoding were run for low and high QP configurations specified in JCTVC-D610. The R-D results were identical to those provided by the proponent for all cases but BQTerrace low-complexity, low QP settings. The magnitude of the mismatch was 0.1% BD bitrate. The mismatch was caused due to a small discrepancy in length of sequence tested.

# Cross-verification

The software implementation was verified to be consistent with document JCTVC-E353 [1]. The source code provided by the proponents is a modified version of HM-2.0. The performance of the proposed core transform was tested on conditions specified in JCTVC-D610 [2].

The encoder and decoder were run on a cluster of machines. The computing platform is summarized in Table 1 below.

Table 1: Computing platforms

| OS | Linux (64bit) |
| --- | --- |
| Some CPUs | Intel(R) Core(TM)2 CPU 6300 @ 1.86GHz cache size : 2048 KB |
| Some CPUs | Intel(R) Xeon(R) CPU X3360 @ 2.83GHz cache size : 6144 KB |
| Some CPUs | Intel(R) Core(TM)2 Extreme CPU Q6800 @ 2.93GHz cache size : 4096 KB |
| Compiler | gcc 4.3.2 |

# Results

A summary of BD Bitrate [3] results is presented below. The timing information is not provided due to large variability observed in timing data. The detailed R-D results can be found in attached excel sheet.

Low QP (QP=1,5,9,13)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.2 | -0.5 | -0.6 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class E | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

High QP (QP=36,42,47,51)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.2 | 0.0 | -0.2 | 0.2 |
| Class B | 0.0 | -0.2 | 0.0 | -0.1 | -0.2 | -0.5 |
| Class C | 0.0 | -0.3 | -0.2 | 0.0 | -0.4 | -0.1 |
| Class D | 0.0 | 0.2 | -0.2 | 0.0 | 0.0 | -0.2 |
| Class E | 0.0 | 0.1 | -0.4 | -0.1 | 0.1 | -0.4 |
| All | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.5 | 1.2 | 0.0 | 0.7 | 0.7 |
| Class B | 0.0 | -1.0 | -0.7 | 0.0 | 0.0 | -0.4 |
| Class C | -0.2 | 0.7 | -0.2 | 0.0 | -0.1 | -0.1 |
| Class D | 0.2 | 0.6 | -2.4 | -0.1 | 0.5 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | -0.8 | 0.0 | 0.2 | -0.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.7 | 0.1 | 0.1 | -0.5 | 2.6 |
| Class C | 0.0 | 1.6 | -1.1 | 0.3 | -1.7 | -1.8 |
| Class D | 0.0 | -2.9 | -1.4 | -0.2 | -2.7 | -0.7 |
| Class E | -0.2 | -1.1 | -0.4 | 0.2 | 0.0 | 0.4 |
| All | 0.0 | -0.8 | -0.7 | 0.1 | -1.3 | 0.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

The R-D results were identical to those provided by the proponent for all cases but BQTerrace low-complexity, low QP settings. The magnitude of the mismatch was 0.1% BD bitrate. The mismatch was caused due to a small discrepancy in length of sequence tested.

# Conclusion

The cross-check and software verification of “CE10: core transform proposal 1” is reported. The experimental results yield the exactly same R-D results as provided by the proponent in all but one BQTerrace low-complexity, low QP setting.

# References

[1] Pankaj Topiwala, Wei Dai, Ilkoo Kim, “CE10: core transform proposal 1,” JCTVC-E353/m19881 5th Meeting: Geneva, CH, 16-23 March, 2011.

[2] P. Topiwala, M. Budagavi, A. Fuldseth, R. Joshi,  I.-K. Kim, “Description of Core Experiment 10 (CE10): Core Transform Design,” JCTVC-610/m19491, 4th Meeting: Daegu, KR, 20-28 January, 2011.

[3] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33