| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E420 |
| --- | --- |

| Title: | CE7: Crosscheck of Samsung's ROT results (JCTVC-E380) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-658-4511 Email: rajanj@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Samsung’s rotational transform proposal in CE7 (JCTVC-E380). The results of the crosscheck matched those reported by Samsung. The encoding and decoding times also matched closely.

# Introduction

Samsung’s contribution proposes the applying rotational transform (ROT) on 4×4 or 8×8 lowest frequency DCT coefficients. The choice of ROT is explicitly signaled. In this crosscheck, we confirmed the results reported by Samsung for the case where ROT was implemented as matrix multiplication.

- Results

Results for 4 configurations (HE and LC for intra and random access) as specified in CE7 were crosschecked. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding was done on a LINUX cluster and decoding was done on a PC using Microsoft Visual C++ (32-bit).

Table 1: BD-rate results for ROT implemented as matrix multiplication

# Conclusions

We have verified results reported in Samsung’s proposal rotational transform implemented as matrix multiplication.  For intra configuration, BD-rate of -1.7% and -2.4% is achieved for HE and LC, respectively. This is achieved at a cost of encoding time increase of 47% and 84%, respectively. For random access configuration, BD-rate of -1.0% and -1.4% is achieved for HE and LC, respectively. In this case, the encoding time increase is 4%. The encoding and the decoding times were within margin of error of those reported by Samsung.