| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E421 |
| --- | --- |

| Title: | Crosscheck of Cisco Tiles Proposal (JCTVC-E408) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Muhammed Coban 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document reports cross verification results for Tiles (JCTVC-E408) proposal. Qualcomm’s cross verification results match those produced by Cisco. The software tested had a bug. Results reported in this document are flawed.

# Introduction

This document reports cross verification results for Experiment 2 of Tiles (JCTVC-E408) proposal.

# Results

Cross verification results are summarized in . Rate-distortion measurements using BD-Rate for proposed tools have been performed [2] . Encoding are measured cluster run times.

|  | Intra |  |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | -1.5 | -1.5 | -1.9 |  | -2.1 |  | -2.0 | -2.0 |
| Class B | -2.2 | -1.9 | -2.1 |  | -2.6 |  | -2.7 | -2.6 |
| Class C | -0.8 | -0.6 | -0.7 |  | -1.0 |  | -0.9 | -1.0 |
| Class D | 0.0 | 0.0 | 0.0 |  | 0.0 |  | 0.0 | 0.0 |
| Class E | -5.1 | -5.5 | -4.5 |  | -5.2 |  | -7.3 | -6.3 |
| All | -1.8 | -1.7 | -1.7 |  | -2.0 |  | -2.3 | -2.2 |
| Enc Time[%] | 99% |  |  |  | 100% |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  |  | Random access |  |  |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | -6.4 | -7.8 | -7.9 |  | -5.3 |  | -5.1 | -5.8 |
| Class B | -6.5 | -6.7 | -5.9 |  | -5.7 |  | -5.3 | -4.9 |
| Class C | -2.7 | -2.9 | -3.2 |  | -2.3 |  | -2.1 | -2.2 |
| Class D | 0.0 | 0.0 | 0.0 |  | 0.0 |  | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |  |  |
| All | -4.0 | -4.5 | -4.4 |  | -3.5 |  | -3.2 | -3.3 |
| Enc Time[%] | 102% |  |  |  | 101% |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -6.5 | -8.3 | -6.9 |  | -5.7 |  | -5.5 | -4.8 |
| Class C | -2.6 | -3.6 | -3.6 |  | -2.1 |  | -1.8 | -1.6 |
| Class D | 0.0 | 0.0 | 0.0 |  | 0.0 |  | 0.0 | 0.0 |
| Class E | -17.4 | -17.0 | -16.9 |  | -14.3 |  | -14.7 | -14.4 |
| All | -5.9 | -6.7 | -6.2 |  | -5.0 |  | -4.9 | -4.6 |
| Enc Time[%] | 96% |  |  |  | 99% |  |  |  |
| Dec Time[%] | #NUM! |  |  |  | #NUM! |  |  |  |

Table 1: Experimental results

The computing platform is summarized in Table 2.

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| Compiler | gcc 4.1.2 | gcc 4.1.2 |

Table 2: Computing platform

# Conclusion

Cross verification results of  “Tiles”, JCTVC-E408  is reported.  The results are flawed due to a bug in the software.

# Reference

- “Tiles”, JCTVC-E408, 5th JCT-VC Meeting, Geneva, CH, March 2011.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33

# Patent rights declaration(s)

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).