| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E426 |
| --- | --- |

| Title: | Experimental results of 4 taps/5 bits Chroma DCTIF in HM2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | contribution |  |  |
| Author(s) or Contact(s): | Elena Alshina Jianle Chen Alexander Alshin Nikolay Shlyakhov Woo-Jin Han | Email: | elena_a.alshina@samsung.com jianle.chen@samsung.com alexander_b.alshin@samsung.com n.shlyakhov@samsung.com wjhan.han@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution provides the simulation for 4 taps Chroma interpolation filter with 5 bits precision for coefficients representation. It used in Hm2.0 instead default 4 taps Chroma filter with 6 bits coefficients. Proposed variant shows 0.1% average performance improvement for Chroma components (no performance change for Luma). Because of low bit-depth for filter coefficients proposed variant requires less number of operations and reduce dynamic range of calculations.

- Introduction

DCT based interpolation filter (DCTIF) was used for motion compensation in Samsung’s CfP response [1]. This is 2D separable interpolation filter with easy to generate filter coefficients for arbitrary number of taps. DCTIF12 was selected as default interpolation filter for HE case in first HEVC test model. For second version of HEVC test model 8 taps DCTIF (proposed in [1]) was selected as unique filter for both HE and LC configuration. For Chroma MC 4 taps DCTIF (proposed in [2]) was chosen in HM2.0. Interpolation filter coefficients in HM2.0 represented with 6 bits precision, which allows keeping intermediate buffer size within 16 bits. For this proposal Chroma DCTIF coefficients were generated using the same technique described in [2] but scaled by 32 (in order to keep 5 bits precision).

- Algorithm description

Interpolation filter is applied in Chroma MC part to get fractional position value in one of 63 position (Refer to Fig. 1) .

Fig. 1 Fractional positions for 1/8 pel MC.

2D separable interpolation scheme for 1/8 accuracy. DCTIF is 2D separable interpolation filter. For fractional positions “oa”, “a”, “ob”, “b”, “oc”, “c” and “od” horizontal 1D filetr is used. For fractional positiona “oe”, “d”, “oq”, “h”,”pe”,”n” and”pq” vertical 1D filetr is used. For remaining positions first vertical 1D filetr is applied for extended block, and then horizontal 1D filter is used. Block extension is 2M-1 (2M is number of filetr taps).

1D interpolation filetr.  The input of 1D interpolation funtion is array Ref-M+1, Ref-M+2,…,RefM of integer postion values. The output is interpolated value X, which has fractional position  between Ref0 and  Ref1.

(1)

If only one 1D filter is used for current fractional position then  .

If  two stage interpolation will be applied (inner fractional positions) then   for vertical filtration (no left shift is needed), and   for horizontal 1D filter.

For   fractional positions DCTIF coefficients can be obtained using mirror symmetry:

.

Interpolation filter coefficients used for Chorma MC in HM2.0 are shown in Table 1. Scaling factor for filter coefficients in 64 (6 bits). Interpolation filter coefficients for proposed variant are shown in Table 2. Scaling factor for filter coefficients in 32 (5bits).

Table. 1 DCTIF 4 taps/6 bits for Chroma 1/8 pel accuracy MC.

|  |  | “>>” | “+” | “*” | “+” |
| --- | --- | --- | --- | --- | --- |
| 1/8 | { -3, 60, 8, -1,} | 3 | 5 | 3 | 3 |
| 1/4 | { -4, 54, 16, -2,} | 5 | 5 | 4 | 3 |
| 3/8 | { -5, 46, 27, -4,} | 4 | 7 | 4 | 3 |
| 1/2 | { -4, 36, 36, -4,} | 2 | 4 | 2 | 3 |

Table. 2 DCTIF 4 taps/5 bits for Chroma 1/8 pel accuracy MC.

|  |  | “>>” | “+” | “*” | “+” |
| --- | --- | --- | --- | --- | --- |
| 1/8 | { -1, 29, 5, -1,} | 2 | 5 | 2 | 3 |
| 1/4 | { -2, 27, 8, -1,} | 4 | 5 | 3 | 3 |
| 3/8 | { -3, 23, 14, -2,} | 3 | 7 | 4 | 3 |
| 1/2 | { -2, 18, 18, -2,} | 1 | 4 | 2 | 3 |

Proposed variant of Chroma interpolation filter reduces 1 shift operations per pixel compare to

Chroma interpolation filter used in HM2.0. For some fractional positions the number of multiplications is reduced as well.

- Test Results.

Interpolation filter with coefficients represented in Table 2 was implemented in HM2.0 and tested using test conditions specified in CE3 description [3]. In average across 4 test cases: RA-HE, LD-HE, RA-LC and LD-LC proposed filter shows 0.0%/0.1%/0.1% performance gain for Y,U and V color component respectively. Mostly performance improvement comes from LC case.

Table 3. 4 taps 5 bits Chroma DCTIF performance test results.

|  |  | Random access |  |  | Random access |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | -0.1 | 0.0 | -0.1 | 0.2 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | -0.2 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | -0.4 | 0.0 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | -0.2 | -0.2 |
| Class D | -0.1 | -0.4 | -0.6 | 0.0 | -0.7 | -0.4 |
| Class E | 0.0 | 0.5 | 0.7 | 0.0 | -0.5 | -0.3 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | -0.4 | -0.2 |
| Enc Time[%] | 101% |  |  | 96% |  |  |
| Dec Time[%] | 101% |  |  | 94% |  |  |

- Conclusions

Based on these test results:

- performance improvement for Chroma components:0.0% (Y) /0.1% (U) /0.1%(V)

- number of operations reduction both in mults/adds and shifts/adds form

we recommend study possibility for further reduction of precision for  DCTIF coefficients representation.

# References

- “CE3: Experimental results of DCTIF by Samsung " JCTVC-D344, Daegu, Korea, January 2011.

- “CE4: Experimental results of DCTIF application for Chroma MC by Samsung" JCTVC-D347, Daegu, Korea, January 2011.

- “Description of Core Experiment 3 (CE3): Interpolation for MC (Luma)" JCTVC-D603, Daegu, Korea, January 2011.

# Patent rights declaration(s)

Samsung Electronics, Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).