| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E429 |
| --- | --- |

| Title: | On Profile signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Discussion |  |  |
| Author(s) or Contact(s): | David Singer | Tel: Email: | singer@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution discusses how profiles, and particularly files that are compatible with multiple profiles, are signaled, and presents some of the alternative ways to indicate this.

# Problem Statement

In the AVC standard, the ‘primary profile’ is signaled with the numeric profile_idc indicator in the sequence parameter set. The field is followed by a set of ‘constraint_set’ flags, some of which are used to indicate ‘also compatible with’ other profiles or other constraints.

However, there are a limited number of these flags, not every profile has an associated flag, and the association of profile_idc value to bit position is solely through the document.

We seem to prefer to be able to define profiles as a selection of tools, and in general we are not able to be sure that all profiles will have a ‘perfect nesting’ structure (i.e. that a lower profile is always a subset of a higher profile). If the profiles were nested, of course, we could simply ensure that a file marked as being profile with profile_idc X would also comply to any profile with a profile_idc < X. This is not the case.

In the ISO Base Media File Format (ISO/IEC 14496-12) we have a structure which supplies a ‘best use’ indicator (the major brand) and a list of ‘also compatible with’ indicators (the compatible brands list). This has served us well.  For example, it enables indications that cover different areas: one brand may indicate only restrictions on the file structure itself (such as the version of the specification, and hence toolset required), others may restrict features of the file (e.g. the number of simultaneous tracks) and yet others restrict which codecs may be present, and so on.

A ‘best use’ indicator is also valuable; for example, an AVC bitstream may be scalable (use SVC) or multi-view (use MVC) and still have an AVC-compatible base layer. Under these circumstances, the ‘best use’ is as an SVC or MVC bitstream; reading it as an AVC bitstream means that there is data in the bitstream that is unused.

# Proposal

This contribution proposes we can move to a simple indicator scheme for ‘best use’ profile and ‘also compatible with’ profiles.

# Indicators

The use of a small integer seems to work well as a profile indicator. However, if we can define a general scheme for the ‘also compatible with’ indicators, we don’t need to have them spaced out in a ‘large’ number space. AVC has 66, 77, 100, and so on, leaving room for new profile indicators in between old values, but this is not needed, as the actual value of the indicator, and whether it is greater or less than another value, says very little. So we propose that we start numbering profiles sequentially as they are defined, with little regard for any other meaning to the integers. This gives us a densely-allocated set of values to represent in the ‘also compatible with’ list.

There are a number of possibilities for that list:

- a true list; a set of values terminated by one or more zero values, for example (this allows zero to be used as a padding value, which can help if the compatibility list might need additions after the initial writing).

- a bit-field; we could allocate say 32 bits, and hope we never have more than 32 profiles;

- an extending bit-field; we could one bit in each byte to indicate ‘more bits follow’ or ‘end of list’;

There are variants on the third case here. The simplest way to indicate compatibility is to allocate 7 bits in each byte; bit 1 indicates ‘compatible with profile with profile_idc==1’, and so on.

However, we may be able to leverage the ‘best use’ profile indicator, and have the bits indicating decreasing order. If the ‘best use’ profile indicator has value X, then bit 1 would indicate ‘also compatible with the profile with profile_idc==(X-1)’, and so on. This would encourage defining the profile_idc values in clusters of related profiles. It has the disadvantage of assuming that the ‘best use’ is always the highest profile indicator, or changing the main field to mean ‘highest compatible profile_idc value’.

Indeed, we are in a working group that knows many schemes for compacting bit-fields (some quite devious); but we should probably resist the temptation to use complex compression of this list. We hope, after all, that the overall number of profiles, and hence the maximum possible length of the compatibility list, is short.

# Strawman

Profile_idc is a single-byte integer value, allocated in ‘clusters’ of similar profiles such that files that are compatible with more than one profile are likely to use adjacent values.

There is a field best_use_profile_idc.

Bits are allocated in groups of 7, with the 8th bit in each byte indicating ‘end of list’. The first bit is labeled bit(1), and bit(n) being 1 indicates that the file is also compatible with profile with profile_idc==n. Note that this can therefore accommodate arbitrary numbers of zero bytes, terminating with a byte with the value 1.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).