| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 |  |  | Document: JCTVC-E430 WG11 Number: m19973 |  |  |
| --- | --- | --- | --- | --- | --- |
| Title: | Cross-verification of Panasonics (JCTVC-E226) Parallel Context Processing For Significance Map by Block-based context updates |  |  |  |  |
| Status: | Input Document to JCT-VC |  |  |  |  |
| Purpose: | Core Experiment |  |  |  |  |
| Author(s) or Contact(s): | Vivienne Sze Texas Instruments Inc. | Email: |  | sze@ti.com |  |
| Source: | Texas Instruments Inc. |  |  |  |  |

_____________________________

# Abstract

This contribution describes the cross-check results for Panasonic’s proposal JCTVC-E226.  The coding efficiency results measured in HM-2.0 with the high efficiency configuration as specified in the common conditions.  A coding loss of Intra 0.2%, Low Delay 0.2%, Random Access 0.2% was measured.  Results match those reported by Panasonic.

# Experiment Results

The TMuC-0.9 software is used for the evaluation, the simulation platforms is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz  64 bits Linux machines, the common test conditions and reference configurations specified by the Software AhG are followed.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.4 | 0.4 |
| Class B | 0.1 | 0.3 | 0.3 |
| Class C | 0.1 | 0.3 | 0.2 |
| Class D | 0.1 | 0.2 | 0.1 |
| Class E | 0.2 | 0.5 | 0.4 |
| All | 0.2 | 0.3 | 0.3 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.0 | -0.3 |
| Class B | 0.1 | 0.3 | 0.1 |
| Class C | 0.1 | 0.2 | 0.2 |
| Class D | 0.2 | 0.0 | 0.2 |
| Class E |  |  |  |
| All | 0.2 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.2 | 0.4 | 0.1 |
| Class C | 0.3 | 0.3 | 0.0 |
| Class D | 0.1 | -0.4 | -0.7 |
| Class E | 0.1 | -1.0 | 0.6 |
| All | 0.2 | -0.1 | 0.0 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |

Table 1  Coding efficiency of proposal. (Loss is positive)