| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E431r1 |
| --- | --- |

| Title: | Spatial Scalability for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Koohyar Minoo, David Baylon, and Ajay Luthra 6450 Sequence Drive San Diego, CA 92121 USA | Tel: Email: | +1 (858) 404-2285 kminoo@motorola.com david.baylon@motorola.com aluthra@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

Scalability in video coding allows for efficient delivery of video at different qualities or resolutions.  This contribution presents an approach that provides for scalability with HEVC.  In particular, the focus is on spatial scalability.  For factor of two scalability in both horizontal and vertical directions, results of the proposed approach for Class B – E sequences using HM 2.0 show an average luma BD bitrate savings over simulcast of 30.1% for the Intra Low Complexity case.

# Introduction

One goal in scalable video coding is to be able to efficiently deliver video at different qualities or resolutions.  Such scalability can be important when transmitting over channels with different bandwidth capacities, or to targeted devices with different resolutions or computational capabilities.  Instead of generating independent bitstreams for the different video resolutions or “layers”, by exploiting correlations among the video layers in scalable coding, joint encoding can lead to more efficient results.  The base layer can be independently encoded, and additional enhancement layers can be dependently coded based on the previous layers.  With bitstream scalability is possible to decode additional layers with an incremental increase in bits. This is in contrast to the simulcast case, where the bitstreams for the various layers don’t make use of previous layer bitstreams but instead are independently generated and transmitted.

Examples of scalability include spatial, temporal, bitrate, fidelity (e.g. bitplane resolution), chroma, view (such as stereo or multiview), or a combination of these.  MPEG-4 AVC/H.264 video coding can be used to achieve many of these types of scalability, and it also adds additional coding tools to improve coding efficiency.

The development of next generation coding technology in HEVC can be directly applied to improve the efficiency of single layer coding.  Depending on the GOP structure used for coding, it can also be used as a basis for temporal scalability.  However, its extension to more general forms of scalability has not yet been fully investigated.  This contribution presents a framework that can be used to achieve other forms of scalability.  In particular, the application to spatial scalability is described, and differences from previous scalable frameworks such as in MPEG-4 AVC/H.264 will be discussed.

# Proposed Approach for Scalability

The proposed approach makes use of a decomposition of the input video into features that can be both efficiently represented and better matched to our perception of the video.  Although the most appropriate decomposition may depend on the characteristics of the video, this contribution focuses on a decomposition for a wide variety of content including typical, natural video.  Figure 1 illustrates the decomposition of the input x into two layers through analysis filtering.  In this example, the filtering separates x into different spatial frequency bands.  Although the input x can correspond to a portion of a picture or to an entire picture, the focus in this contribution is on the entire picture.  For typical video, most of the energy is concentrated in the low frequency layer l0 as compared to the high frequency layer l1.  Also, l0 tends to capture local intensity features while l1 captures variational detail such as edges.

Each layer li is then encoded with Ei to produce bitstream bi.  For spatial scalability, the analysis process can include filtering followed by subsampling so that b0 can correspond to an appropriate base layer bitstream.  As an enhancement bitstream, b1 can be generated using information from the base layer l0 as indicated by the arrow from E0 to E1.  The combination of E0 and E1 is referred to as the overall scalable encoder Es.

The scalable decoder Ds consists of base layer decoder D0 and enhancement layer decoder D1.  The base layer bitstream b0 is decoded by D0 to reconstruct the layer l’0.  The enhancement layer bitstream b1 is decoded by D1 together with possible information from b0 to reconstruct the layer l’1.  The two decoded layers, d’0 and d’1 are then used to reconstruct x’ using a synthesis operation.

# Implementation and Results

To illustrate the proposed approach for spatial scalability, critical sampling was used in a two-band decomposition at the picture level.  Both horizontal and vertical directions were subsampled by a factor of two, resulting in a four layer scalable system.  Simulations were performed using HM 2.0 for both encoders Ei and decoders Di.  Although it is possible to improve coding efficiency by exploiting correlations among the layers, these simulations do not make use of any interlayer prediction.

The performance of the proposed approach was compared to the single layer and simulcast cases.  In the single layer case, x is encoded using HM 2.0 directly.  In the simulcast case, the bitrate is determined by adding together the bits for encoding x directly and the bits for encoding l0 directly, while the PSNR is that corresponding to the direct encoding of x.  In the proposed approach, the bitrate corresponds to the bits for all layers, and the PSNR is that for x’.

Table 1 shows the BD bitrate savings of the proposed approach over simulcast for the Class B sequences using Intra High Efficiency and Low Complexity coding.  For these sequences, there was an average savings over simulcast of 38.2% for Intra High Efficiency (HE) and 34.7% for Intra Low Complexity (LC).  Figures 2 and 3 show the Intra HE and Intra LC plots, respectively, for the three cases of (i) single layer, (ii) simulcast, and (iii) the proposed decompositional scalable coding (DSC) approach.  For these sequences, the decomposition concentrates most of the energy in the base layer, and the coding is adapted to the characteristics of each layer.  Table 2 shows that there is an average BD bitrate savings of the proposed approach over simulcast for the Class B -  E sequences of 30.1% for Intra Low Complexity.

# Features of the Proposed Approach

Some features of the proposed approach include the following:

- Efficient representation: By utilizing critically sampled layers, the encoders Ei in the proposed approach operate on the same total number of pixels as the input x.  This is in contrast to SVC, where for spatial scalability there is an increase in the total number of pixels to be encoded, and the memory requirement is also increased.

- General spatial scalability: The approach can extend to other spatial scalability factors, for example, 1:n.  Because the layers can have the same size, there can be a simple correspondence in collocated information (e.g. pixels, CU/PU/TU, motion vectors, coding modes, etc.) between layers.  This is in contrast to SVC, where the size (and possibly shape) of the layers are not the same, and the correspondence in collocated information between layers may not be as straightforward.

- Sharpness enhancement: The proposed approach can be used to achieve sharpness enhancement as additional layers provide more detail to features such as edges.  This type of sharpness enhancement is in contrast to other quality scalable approaches that improve quality only by changes in the amount of quantization.

- Independent coding of layers: The simulation results for spatial scalability indicate that it is possible to perform independent coding of layers while still maintaining good coding efficiency performance.  This makes parallel processing of the layers possible, where the layers can be processed simultaneously.  For the two-layer spatial scalability case with SVC, independent coding of the layers (no inter-layer prediction) would correspond to the simulcast case.  Note that with independent coding of layers, errors in one layer do not affect the other layers.  In addition, a different encoder Ei can be used to encode each li to better match the characteristics of the layer.

- Dependent coding of layers: In the proposed approach, dependent coding of layers can improve coding efficiency.  When the layers have the same size, sharing of collocated information between layers is simple.  It is also possible to adaptively encode layers dependently or independently to trade-off coding efficiency performance with error resiliency performance.

# Summary

This contribution presents an approach to scalability with HEVC.  Using the approach with HM 2.0, results for factor of two spatial scalability in both directions for Class B - E sequences show an average luma BD bitrate savings over simulcast of 30.1% for the Intra Low Complexity case.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Analysis

E0

E1

D0

D1

Synthesis

x

b0

b1

x’

l0

l1

l’1

l’0

Es

Ds

Figure 1.  Example of proposed approach for scalability

| Class B |  | Intra |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Kimono |  | -46.2 | -45.1 |  | -44.1 | -43.7 | -43.4 | -41.0 |
|  | ParkScene |  | -41.2 | -37.4 |  | -37.2 | -39.3 | -33.8 | -28.7 |
|  | Cactus |  | -36.7 | -32.8 |  | -29.9 | -32.0 | -27.5 | -23.9 |
|  | BasketballDrive |  | -36.3 | -33.8 |  | -32.2 | -31.2 | -16.1 | -17.5 |
|  | BQTerrace |  | -30.6 | -27.0 |  | -24.3 | -27.6 | -19.5 | -15.2 |
|  | All |  | -38.2 | -35.2 |  | -33.6 | -34.7 | -28.0 | -25.3 |

Table 1.  BD bitrate savings of the proposed approach over simulcast (Class B)

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class B | -34.7 | -28.0 | -25.3 |
| Class C | -24.5 | -17.0 | -15.2 |
| Class D | -22.0 | -12.8 | -10.3 |
| Class E | -39.0 | -18.9 | -24.8 |
| Class B - E | -30.1 | -19.2 | -18.9 |

Table 2.  BD bitrate savings of the proposed approach over simulcast (Class B – E, Intra LoCo)

Figure 2.  Performance comparison of Class B sequences for Intra/HE

Figure 2.  Performance comparison of Class B sequences for Intra/HE, continued

Figure 2.  Performance comparison of Class B sequences for Intra/HE, continued

Figure 2.  Performance comparison of Class B sequences for Intra/HE, continued

Figure 2.  Performance comparison of Class B sequences for Intra/HE, continued

Figure 3.  Performance comparison of Class B sequences for Intra/LC

Figure 3.  Performance comparison of Class B sequences for Intra/LC, continued

Figure 3.  Performance comparison of Class B sequences for Intra/LC, continued

Figure 3.  Performance comparison of Class B sequences for Intra/LC, continued

Figure 3.  Performance comparison of Class B sequences for Intra/LC, continued