| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E432-r1 WG11 Number: m19978 |
| --- | --- |

| Title: | Unified scaling for 10-bit to 8-bit reference frame compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dzung Hoang | Tel: Email: | dzung.hoang@zenverge.com |
| Source: | Zenverge, Inc. |  |  |

_____________________________

# Abstract

Internal Bit Depth Increase (IBDI) is a technique that can improve the compression efficiency by coding a video sequence at a higher bit-depth. However, the main drawback is that memory storage and bandwidth requirements are increased. Reference frame compression (RFC) techniques have been proposed to reduce the memory storage and bandwidth penalty of IBDI by compressing the reference frames. In addition, RFC can also be used to encode and decode video sequences with bit-depth higher than 8 bits using the same memory footprint as 8-bit video. In summary, Zenverge RFC exhibited average Y BD-rate increase of 1.4% (0.3%) compared to 3.0% (1.0%) for Toshiba Fixed Scaling and 3.1% (1.7%) for HM 2.0 without IBDI for LD-HE (RA-HE, resp.). Zenverge RFC encoder run times are 102% for both LD-HE and RA-HE configurations. Decoder run times are 113% for both LD-HE and RA-HE.

Introduction

Internal Bit Depth Increase (IBDI) [1] is a technique that can improve the compression efficiency by coding a video sequence at a higher bit-depth. The increased compression efficiency comes from the increased arithmetic precision in the video compression pipeline. However, the main drawback is that memory storage and bandwidth requirements are increased. Reference frame compression (RFC) techniques have been proposed to reduce the memory storage and bandwidth penalty of IBDI by compressing the reference frames. In addition, RFC can also be used to encode and decode high bit-depth video sequences using a smaller memory footprint. For example, with RFC, 10-bit video can be compressed in the same memory footprint as 8-bit video. An example use case is in professional video production where it is desirable to maintain compressed video at 10-bit sampling throughout the production pipeline in order to reduce the artifacts of multi-generational compression. Including RFC within a standardized codec would allow for an economical realization of this goal, especially for HD and Ultra HD content.

In this document, we propose an RFC algorithm that is a redesign of JCTVC-D035 to compress 10-bit samples down to 8 bits.

# Description of 10-bit to 8-bit Compression

We are proposing a 10-bit to 8-bit lightweight compression scheme based upon our previous submission (JCTVC-D035). The proposal uses a block size of 4x4 for coding each Y/U/V channel and codes each block using 128 bits. Spatial prediction is based upon the quantized block minimum, which is signaled. The residuals are quantized using a simple power-of-two quantizer. The quantized residuals are coded using a combination of 8-bit and 7-bit sampling to fit within the 128-bit budget. In addition, a reconstruction offset is encoded for each block to reduce the distortion.

## Compression Format

Because the internal bit depth is decreased from 12 bits to 10 bits since the last JCT-VC meeting, this frees up some bits in the compression format with reference to JCTVC-D035. We can put the extra bits into use by coding the residual of some samples using 8 bits instead of 7 bits. Using the trick of coding the index of the minimum sample and skip coding its residual (which is always 0), we can free enough bits to code 7 samples with 8 bits. We chose to use a checkerboard pattern to select the samples to code with 8 bits, as shown in Figure 1. The samples with the same checkerboard “color” as the minimum sample are select to be coded with 8 bits. The position of the minimum sample thus determines the positions of the 8-bit coded samples. Let’s refer to the 8-bit-coded samples as the set P8 and the 7-bit-coded samples as P7. The main motivation for this spatial interleaving of P7 and P8 is to avoid any artifacts that might be visible if the P8 samples are lumped together, in a 4x2 sub-block, for example. Secondarily, the checkerboard pattern may serve to reduce any contouring effects that may be present in the source block.

Figure 1: Checkerboard pattern used for segmenting P7 and P8

The 8-bit residuals in P8 are computed by using a scaling factor one less than that used for the 7-bit residuals in P7. Let S be the scaling factor for P7 and S’ be the scaling factor for P8. Then S’ = max(0, S-1). The max function prevents S’ from going negative.

The compression format is shown below. Note that when S = 0, no bits are transmitted for offset.

Table 1: Compression format for 10-bit to 8-bit RFC

u(2); /* S: [0..3] */

u(10-S); /* M */

u(S); /* offset */

u(4); /* M_index */

for (i=0; i<8; i++) u(7); /* P7 array */

for (i=0; i<7; i++) u(8); /* P8 array */

## Decompression Algorithm

We first present the decompression algorithm based upon the above compression format in Table 2.

Table 2: Pseudocode for Zenverge 10-bit to 8-bit decompression

S’ = max(0, S-1)

offset’ = offset >> 1

min_color = checkerboard color of M_index

i7 = i8 = 0

for (i=0; i<16; i++) {

if ( i == M_index )

D[i] = M + offset

else if ( min_color == checkerboard color for raster scan position i )

D[i] = (P8[i8++] << S’) + M + offset’

else

D[i] = (P7[i7++] << S) + M + offset

}

## Reconstruction Offset and Its Calculation

Since there are two scaling factors S and S’, for P7 and P8 respectively, and only enough bits to signal one offset, we need to consider how to use the single offset to reconstruct both P7 and P8. If we consider the quantized samples as integers, the design choice we made is to have the offset provide the fraction to be added at reconstruction. This is illustrated in Figure 2. In the integer domain, the effect is that offset is added to the reconstructed P7 while (offset >> 1) is added to the reconstructed P8. The figure applies when S > 0 because the offset is not transmitted for S = 0.

P8: 8 bits coded P7: 7 bits coded S bit offset S-1 bit offset M: 10 - S bits coded

Figure 2: Offset applied to P7 and P8 as a fraction

The compressor needs to compute an offset value for each block. One approach is to try all possible values for offset and pick the one that gives the least sum squared error. However, this is a complex operation akin to RDO. An alternative is to apply least squares analysis to derive an explicit formula for computing the optimal offset as follows. Let D(θ) denote the block distortion resulting by applying θ  as offset. Let pixel_value[P8[i]] and pixel_value[P7[i]] denote the original sample values of P8 and P7, respectively. For simplicity, we approximate (θ >> 1) by θ/2.

Dθ=i=06P8i≪S'+M+θ2-pixel_valueP8i2+i=07P7i≪S+M+θ-pixel_valueP7i2+M+θ-pixel_valueM_index2

Now we compute the derivative and solve for the root to find the value of θ that minimizes D(θ).

dDθdθ=i=06P8i≪S'+M+θ2-pixel_valueP8i+2i=07P7i≪S+M+θ-pixel_valueP7i+2M+θ-pixel_valueM_index=0

dDθdθ=7θ2+i=06P8i≪S'+M-pixel_valueP8i+16θ+2i=07P7i≪S+M-pixel_valueP7i+2θ+2M-pixel_valueM_index=0

43θ2+i=06P8i≪S'+M-pixel_valueP8i+2i=07P7i≪S+M-pixel_valueP7i+2M-pixel_valueM_index=0

θ=243i=06pixel_valueP8i-P8i≪S'+M+2i=07pixel_valueP7i-P7i≪S+M+2pixel_valueM_index-M

Substituting E8,  E7, and EM for the sum of error without offset due to P8, P7, and M_index, respectively, we arrive at the following solution.

θ=243E8+2E7+2EM

To further simplify the implementation, we approximate 2/43 with 3/64 and take the floor.

θ≈364E8+2E7+2EM

Because of the simplifications, the derived value for the offset is an approximation to the optimal choice.

## Compression Algorithm

The compression algorithm is presented below.

Table 3: Pseudocode for Zenverge 10-bit to 8-bit compression

M = (minimum pixel value in block)

M_index = index of first occurrence of M in block in raster order

min_color = checkerboard color of M_index

R = (maximum pixel value in block)

for (S=0; (R>>S)-(M>>S)>=128; S++);

mask = (1 << S) - 1

S’ = max(0, S-1)

mask’ = (1 << S’) - 1

M = M & ~mask

err = i7 = i8 = 0

for (i=0; i<16; i++) {

if ( i == M_index ) {

err += 2*(pixel_value[i] & mask)

} else if ( min_color == checkerboard color for raster scan position i ) {

P8[i8++] = (pixel_value[i] - M) >> S’

err += (pixel_value[i] & mask’)

} else {

P7[i8++] = (pixel_value[i] - M) >> S

err += 2*(pixel_value[i] & mask)

}

}

offset = 3*err/64

# Experimental Results

## Encoder Implementation Notes

For the encoder, we implemented the RFC algorithm on top of the HM-2.0-ahg-memory branch. We compared this against the HM-2.0 branch and the fixed scaling implemented in the HM-2.0-dev-toshiba branch. We added code to the HM-2.0-ahg-memory and HM-2.0 branches to compute MD5 sum at runtime to avoid dumping YUV to disk for comparison with the decoder. We did not modify the HM-2.0-dev-toshiba branch. The MD5 computation did not affect the encoder run times by much (less than 1%).

## Decoder Implementation Notes

For the decoder, we implemented the RFC algorithm on top of the HM-2.0-ahg-memory branch. We compared this against the HM-2.0-ahg-memory branch without RFC and HM-2.0-dev-toshiba. We added code to the HM-2.0-ahg-memory and HM-2.0 branches to compute MD5 sum at runtime to avoid dumping YUV to disk for comparison with the decoder. We did not modify the HM-2.0-dev-toshiba branch. In contrast to the encoder, the MD5 computation did affect the decoder run times significantly. Therefore we reran all the decoder runs without the MD5 computation for run time comparison. Since the HM-2.0-dev-toshiba branch does not contain code to perform the memory bandwidth calculations present in HM-2.0-ahg.memory, its run times are better than our modification of HM-2.0-ahg-memory and the original HM-2.0-ahg.memory.

## Experimental Results versus HM 2.0 with IBDI

All experiments were conducted on Linux 64-bit platforms on the Amazon AWS cloud using the same instance type (c1.xlarge).

We first present results with reference to HM 2.0 with IBDI enabled. We ran encoding simulations using the configurations where IBDI is enabled in the common conditions: namely, random-access high-efficiency and low-delay high-efficiency. The results are shown below in Table 4 through Table 6.

Both Toshiba Fixed Scaling and HM 2.0 without IBDI exhibited high losses for chroma for several sequences, as highlighted in Table 5 and Table 6. In contrast, Zenverge RFC does not exhibit any anomalous results for chroma.

In summary, Zenverge RFC exhibited average Y BD-rate increase of 1.4% (0.3%) compared to 3.0% (1.0%) for Toshiba Fixed Scaling and 3.1% (1.7%) for HM 2.0 without IBDI for LD-HE (RA-HE, resp.) configuration. Zenverge RFC encoder run times are consistently 102% for both LD-HE and RA-HE configurations. Decoder run times are 113% for both LD-HE and RA-HE.

Table 4: Zenverge RFC versus HM 2.0

|  |  | Zenverge RFC vs HM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LD-HE |  |  | RA-HE |  |  |
|  |  | Y BD-Rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | Traffic |  |  |  | 0.3 | 0.0 | 0.1 |
| 4K | PeopleOnStreet |  |  |  | 0.1 | 0.0 | -0.1 |
|  | Average |  |  |  | 0.2 | 0.0 | 0.0 |
| Class A | Nebuta |  |  |  | 0.2 | 0.0 | 0.0 |
| 4K 10bit | SteamLocomotive |  |  |  | 0.6 | 0.4 | -0.2 |
|  | Average |  |  |  | 0.4 | 0.2 | -0.1 |
| Class B | Kimono | 0.1 | -0.1 | 0.4 | 0.1 | -0.1 | 0.0 |
| 1080p | ParkScene | 0.4 | -0.1 | -0.3 | 0.2 | 0.1 | 0.0 |
|  | Cactus | 2.0 | -0.4 | 0.7 | 0.3 | 0.0 | 0.2 |
|  | BasketballDrive | 0.2 | 0.3 | -0.2 | 0.2 | 0.4 | 0.0 |
|  | BQTerrace | 0.9 | 0.4 | 0.5 | 0.5 | -0.2 | -0.1 |
|  | Average | 0.7 | 0.0 | 0.2 | 0.3 | 0.0 | 0.0 |
| Class C | BasketballDrill | 1.0 | 0.8 | 0.9 | 0.3 | 0.1 | 0.1 |
| WVGA | BQMall | 0.6 | 0.6 | 0.3 | 0.2 | 0.2 | 0.2 |
|  | PartyScene | 0.5 | 0.2 | 0.1 | 0.3 | 0.1 | 0.1 |
|  | RaceHorses | 0.2 | 0.4 | 0.5 | 0.0 | -0.2 | -0.2 |
|  | Average | 0.6 | 0.5 | 0.4 | 0.2 | 0.1 | 0.0 |
| Class D | BasketballPass | 0.1 | -0.2 | -0.1 | 0.2 | 0.4 | 0.2 |
| WQVGA | BQSquare | 0.6 | 1.4 | 1.7 | 0.6 | 0.1 | -0.2 |
|  | BlowingBubbles | 0.3 | -0.3 | 0.5 | 0.2 | 0.0 | 0.0 |
|  | RaceHorses | 0.1 | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 |
|  | Average | 0.3 | 0.3 | 0.6 | 0.3 | 0.2 | 0.0 |
| ClassE | Vidyo1 | 4.3 | 0.2 | 0.6 |  |  |  |
| WQVGA | Vidyo3 | 5.0 | 0.3 | 0.7 |  |  |  |
|  | Vidyo4 | 5.7 | 0.8 | 0.5 |  |  |  |
|  | Average | 5.0 | 0.4 | 0.6 |  |  |  |
| All | Average | 1.4 | 0.3 | 0.4 | 0.3 | 0.1 | 0.0 |
| Enc Time [%] |  | 102% |  |  | 102% |  |  |
| Dec Time [%] (with ahg-memory measurements) |  | 113% |  |  | 113% |  |  |

Table 5: Toshiba Fixed Scaling versus HM 2.0

|  |  | Toshiba Fixed Scaling vs HM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LD-HE |  |  | RA-HE |  |  |
|  |  | Y BD-Rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | Traffic |  |  |  | 1.4 | 2.6 | 3.7 |
| 4K | PeopleOnStreet |  |  |  | 0.6 | 2.8 | 2.8 |
|  | Average |  |  |  | 1.0 | 2.7 | 3.3 |
| Class A | Nebuta |  |  |  | 0.1 | 2.9 | 2.6 |
| 4K 10bit | SteamLocomotive |  |  |  | 2.3 | 23.9 | 40.7 |
|  | Average |  |  |  | 1.2 | 13.4 | 21.7 |
| Class B | Kimono | 1.8 | 4.5 | 6.0 | 1.1 | 2.5 | 3.5 |
| 1080p | ParkScene | 2.2 | 4.0 | 6.0 | 1.1 | 2.0 | 2.9 |
|  | Cactus | 3.4 | 4.3 | 5.5 | 1.5 | 4.1 | 4.2 |
|  | BasketballDrive | 2.0 | 5.1 | 5.0 | 1.1 | 3.4 | 2.9 |
|  | BQTerrace | 3.4 | 17.8 | 23.4 | 1.9 | 6.4 | 9.2 |
|  | Average | 2.5 | 7.1 | 9.2 | 1.3 | 3.7 | 4.5 |
| Class C | BasketballDrill | 1.8 | 3.5 | 2.6 | 1.1 | 2.3 | 1.8 |
| WVGA | BQMall | 2.0 | 5.1 | 5.6 | 1.1 | 3.1 | 3.1 |
|  | PartyScene | 0.9 | 4.8 | 4.9 | 0.6 | 1.4 | 1.7 |
|  | RaceHorses | 0.7 | 1.7 | 1.6 | 0.4 | 1.0 | 1.2 |
|  | Average | 1.4 | 3.8 | 3.7 | 0.8 | 1.9 | 1.9 |
| Class D | BasketballPass | 0.8 | 2.1 | 1.5 | 0.6 | 1.8 | 1.0 |
| WQVGA | BQSquare | 1.1 | 21.3 | 19.7 | 0.8 | 4.1 | 4.3 |
|  | BlowingBubbles | 1.0 | 5.3 | 8.2 | 0.6 | 1.6 | 1.7 |
|  | RaceHorses | 0.7 | 1.4 | 1.7 | 0.6 | 1.1 | 0.7 |
|  | Average | 0.9 | 7.5 | 7.7 | 0.7 | 2.1 | 1.9 |
| ClassE | Vidyo1 | 8.6 | 26.7 | 24.3 |  |  |  |
| WQVGA | Vidyo3 | 8.7 | 20.0 | 26.1 |  |  |  |
|  | Vidyo4 | 9.0 | 15.8 | 19.0 |  |  |  |
|  | Average | 8.8 | 20.8 | 23.1 |  |  |  |
| All | Average | 3.0 | 9.0 | 10.1 | 1.0 | 3.9 | 5.2 |
| Enc Time [%] |  | 102% |  |  | 102% |  |  |
| Dec Time [%] (w/o ahg-memory measurements) |  | 77% |  |  | 77% |  |  |

Table 6: HM 2.0 w/o IBDI versus HM 2.0

|  |  | HM w/o IBDI vs HM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LD-HE |  |  | RA-HE |  |  |
|  |  | Y BD-Rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | Traffic |  |  |  | 2.1 | 4.3 | 5.2 |
| 4K | PeopleOnStreet |  |  |  | 0.9 | 4.2 | 5.3 |
|  | Average |  |  |  | 1.5 | 4.3 | 5.2 |
| Class A | Nebuta |  |  |  |  |  |  |
| 4K 10bit | SteamLocomotive |  |  |  |  |  |  |
|  | Average |  |  |  |  |  |  |
| Class B | Kimono | 2.9 | 6.9 | 7.7 | 2.7 | 5.9 | 7.3 |
| 1080p | ParkScene | 2.0 | 6.2 | 9.0 | 1.5 | 3.5 | 5.0 |
|  | Cactus | 3.2 | 5.5 | 6.0 | 2.2 | 6.4 | 5.8 |
|  | BasketballDrive | 3.4 | 9.7 | 7.7 | 3.0 | 8.2 | 6.3 |
|  | BQTerrace | 4.2 | 26.3 | 32.1 | 3.0 | 11.9 | 13.8 |
|  | Average | 3.2 | 10.9 | 12.5 | 2.5 | 7.2 | 7.6 |
| Class C | BasketballDrill | 2.1 | 3.0 | 2.9 | 1.5 | 2.9 | 3.0 |
| WVGA | BQMall | 2.6 | 7.3 | 7.5 | 1.9 | 4.6 | 4.8 |
|  | PartyScene | 1.2 | 6.7 | 7.8 | 1.1 | 2.7 | 2.9 |
|  | RaceHorses | 1.4 | 2.9 | 4.0 | 1.0 | 2.4 | 3.3 |
|  | Average | 1.8 | 5.0 | 5.6 | 1.4 | 3.1 | 3.5 |
| Class D | BasketballPass | 1.2 | 3.1 | 2.0 | 1.0 | 2.7 | 2.2 |
| WQVGA | BQSquare | 1.2 | 24.2 | 24.5 | 1.1 | 3.8 | 5.3 |
|  | BlowingBubbles | 1.0 | 6.9 | 9.3 | 1.0 | 2.6 | 2.9 |
|  | RaceHorses | 0.9 | 2.2 | 2.8 | 0.9 | 1.8 | 2.0 |
|  | Average | 1.1 | 9.1 | 9.6 | 1.0 | 2.7 | 3.1 |
| ClassE | Vidyo1 | 7.0 | 21.2 | 15.7 |  |  |  |
| WQVGA | Vidyo3 | 6.8 | 18.6 | 13.4 |  |  |  |
|  | Vidyo4 | 7.8 | 14.8 | 15.4 |  |  |  |
|  | Average | 7.2 | 18.2 | 14.8 |  |  |  |
| All | Average | 3.1 | 10.4 | 10.5 | 1.7 | 4.5 | 5.0 |
| Enc Time [%] |  | 100% |  |  | 100% |  |  |
| Dec Time [%] (with ahg-memory measurements) |  | 99% |  |  | 97% |  |  |

## Experimental Results versus HM 2.0 without IBDI

Now we present the experimental results compared to HM 2.0 with IBDI disabled to show the usefulness of IBDI with and without RFC compared to no IBDI.

In summary, Zenverge RFC exhibited average Y BD-rate increase of -1.6% for LD-HE (-1.4% for RA-HE); Toshiba Fixed Scaling exhibited -0.1% for LD-HE (-0.7% for RA-HE); and HM with IBDI exhibited -2.9% for LD-HE (-1.6% for RA-HE).

Table 7: Zenverge RFC versus HM w/o IBDI

|  |  | Zenverge RFC vs HM w/o IBDI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LD-HE |  |  | RA-HE |  |  |
|  |  | Y BD-Rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | Traffic |  |  |  | -1.8 | -4.1 | -4.8 |
| 4K | PeopleOnStreet |  |  |  | -0.7 | -4.1 | -5.1 |
|  | Average |  |  |  | -1.3 | -4.1 | -5.0 |
| Class A | Nebuta |  |  |  |  |  |  |
| 4K 10bit | SteamLocomotive |  |  |  |  |  |  |
|  | Average |  |  |  |  |  |  |
| Class B | Kimono | -2.7 | -6.6 | -6.7 | -2.5 | -5.7 | -6.7 |
| 1080p | ParkScene | -1.6 | -5.9 | -8.5 | -1.3 | -3.3 | -4.8 |
|  | Cactus | -1.2 | -5.6 | -5.0 | -1.8 | -5.9 | -5.3 |
|  | BasketballDrive | -3.1 | -8.6 | -7.3 | -2.8 | -7.2 | -5.9 |
|  | BQTerrace | -3.2 | -20.5 | -23.8 | -2.5 | -10.8 | -12.2 |
|  | Average | -2.3 | -9.4 | -10.3 | -2.2 | -6.6 | -7.0 |
| Class C | BasketballDrill | -1.0 | -2.1 | -2.0 | -1.3 | -2.7 | -2.9 |
| WVGA | BQMall | -1.9 | -6.3 | -6.7 | -1.7 | -4.1 | -4.4 |
|  | PartyScene | -0.8 | -6.1 | -7.1 | -0.7 | -2.5 | -2.7 |
|  | RaceHorses | -1.2 | -2.4 | -3.3 | -1.0 | -2.5 | -3.4 |
|  | Average | -1.2 | -4.2 | -4.8 | -1.2 | -3.0 | -3.3 |
| Class D | BasketballPass | -1.1 | -3.2 | -2.1 | -0.8 | -2.2 | -2.0 |
| WQVGA | BQSquare | -0.5 | -18.4 | -18.1 | -0.5 | -3.5 | -5.2 |
|  | BlowingBubbles | -0.8 | -6.8 | -8.1 | -0.7 | -2.5 | -2.8 |
|  | RaceHorses | -0.9 | -1.9 | -2.5 | -0.7 | -1.6 | -1.9 |
|  | Average | -0.8 | -7.6 | -7.7 | -0.7 | -2.5 | -3.0 |
| ClassE | Vidyo1 | -2.4 | -17.4 | -13.1 |  |  |  |
| WQVGA | Vidyo3 | -1.6 | -15.5 | -11.2 |  |  |  |
|  | Vidyo4 | -1.9 | -12.2 | -12.8 |  |  |  |
|  | Average | -2.0 | -15.0 | -12.4 |  |  |  |
| All | Average | -1.6 | -8.7 | -8.6 | -1.4 | -4.2 | -4.7 |
| Enc Time [%] |  | 102% |  |  | 102% |  |  |
| Dec Time [%] (with ahg-memory measurements) |  | 113% |  |  | 113% |  |  |

Table 8: Toshiba Fixed Scaling versus HM w/o IBDI

|  |  | Toshiba Fixed Scaling vs HM w/o IBDI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LD-HE |  |  | RA-HE |  |  |
|  |  | Y BD-Rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | Traffic |  |  |  | -0.8 | -1.6 | -1.4 |
| 4K | PeopleOnStreet |  |  |  | -0.3 | -1.4 | -2.4 |
|  | Average |  |  |  | -0.5 | -1.5 | -1.9 |
| Class A | Nebuta |  |  |  |  |  |  |
| 4K 10bit | SteamLocomotive |  |  |  |  |  |  |
|  | Average |  |  |  |  |  |  |
| Class B | Kimono | -1.1 | -2.3 | -1.6 | -1.5 | -3.3 | -3.5 |
| 1080p | ParkScene | 0.1 | -2.1 | -2.7 | -0.5 | -1.5 | -2.1 |
|  | Cactus | 0.2 | -1.2 | -0.5 | -0.7 | -2.2 | -1.6 |
|  | BasketballDrive | -1.4 | -4.3 | -2.6 | -1.8 | -4.5 | -3.3 |
|  | BQTerrace | -0.8 | -6.5 | -6.5 | -1.1 | -4.9 | -4.1 |
|  | Average | -0.6 | -3.3 | -2.8 | -1.1 | -3.3 | -2.9 |
| Class C | BasketballDrill | -0.3 | 0.5 | -0.4 | -0.4 | -0.6 | -1.2 |
| WVGA | BQMall | -0.5 | -2.1 | -1.8 | -0.9 | -1.4 | -1.7 |
|  | PartyScene | -0.4 | -1.8 | -2.7 | -0.5 | -1.3 | -1.1 |
|  | RaceHorses | -0.7 | -1.2 | -2.2 | -0.6 | -1.4 | -2.1 |
|  | Average | -0.5 | -1.1 | -1.8 | -0.6 | -1.2 | -1.5 |
| Class D | BasketballPass | -0.3 | -1.0 | -0.6 | -0.4 | -0.9 | -1.2 |
| WQVGA | BQSquare | 0.0 | -1.4 | -2.6 | -0.3 | 0.3 | -0.9 |
|  | BlowingBubbles | 0.0 | -1.5 | -1.0 | -0.3 | -1.0 | -1.1 |
|  | RaceHorses | -0.2 | -0.9 | -1.1 | -0.3 | -0.7 | -1.3 |
|  | Average | -0.1 | -1.2 | -1.3 | -0.3 | -0.6 | -1.1 |
| ClassE | Vidyo1 | 1.6 | 5.1 | 8.2 |  |  |  |
| WQVGA | Vidyo3 | 1.8 | 1.4 | 11.7 |  |  |  |
|  | Vidyo4 | 1.1 | 1.0 | 3.5 |  |  |  |
|  | Average | 1.5 | 2.5 | 7.8 |  |  |  |
| All | Average | -0.1 | -1.1 | -0.2 | -0.7 | -1.8 | -1.9 |
| Enc Time [%] |  | 102% |  |  | 102% |  |  |
| Dec Time [%] (w/o ahg-memory measurements) |  | 78% |  |  | 79% |  |  |

Table 9: HM with IBDI versus HM w/o IBDI

|  |  | HM vs HM w/o IBDI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LD-HE |  |  | RA-HE |  |  |
|  |  | Y BD-Rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | Traffic |  |  |  | -2.1 | -4.1 | -4.9 |
| 4K | PeopleOnStreet |  |  |  | -0.9 | -4.1 | -5.0 |
|  | Average |  |  |  | -1.5 | -4.1 | -4.9 |
| Class A | Nebuta |  |  |  |  |  |  |
| 4K 10bit | SteamLocomotive |  |  |  |  |  |  |
|  | Average |  |  |  |  |  |  |
| Class B | Kimono | -2.8 | -6.5 | -7.1 | -2.6 | -5.6 | -6.8 |
| 1080p | ParkScene | -2.0 | -5.8 | -8.3 | -1.5 | -3.4 | -4.8 |
|  | Cactus | -3.1 | -5.2 | -5.6 | -2.1 | -6.0 | -5.5 |
|  | BasketballDrive | -3.3 | -8.8 | -7.1 | -2.9 | -7.6 | -5.9 |
|  | BQTerrace | -4.1 | -20.8 | -24.3 | -2.9 | -10.6 | -12.1 |
|  | Average | -3.1 | -9.4 | -10.5 | -2.4 | -6.6 | -7.0 |
| Class C | BasketballDrill | -2.0 | -2.9 | -2.8 | -1.5 | -2.8 | -2.9 |
| WVGA | BQMall | -2.5 | -6.8 | -7.0 | -1.9 | -4.4 | -4.6 |
|  | PartyScene | -1.2 | -6.3 | -7.2 | -1.1 | -2.6 | -2.8 |
|  | RaceHorses | -1.4 | -2.8 | -3.8 | -1.0 | -2.4 | -3.2 |
|  | Average | -1.8 | -4.7 | -5.2 | -1.4 | -3.0 | -3.4 |
| Class D | BasketballPass | -1.2 | -3.0 | -2.0 | -1.0 | -2.7 | -2.2 |
| WQVGA | BQSquare | -1.2 | -19.5 | -19.7 | -1.1 | -3.7 | -5.0 |
|  | BlowingBubbles | -1.0 | -6.5 | -8.5 | -1.0 | -2.5 | -2.8 |
|  | RaceHorses | -0.9 | -2.2 | -2.7 | -0.9 | -1.8 | -2.0 |
|  | Average | -1.1 | -7.8 | -8.2 | -1.0 | -2.7 | -3.0 |
| ClassE | Vidyo1 | -6.5 | -17.5 | -13.6 |  |  |  |
| WQVGA | Vidyo3 | -6.3 | -15.7 | -11.8 |  |  |  |
|  | Vidyo4 | -7.2 | -12.9 | -13.3 |  |  |  |
|  | Average | -6.7 | -15.4 | -12.9 |  |  |  |
| All | Average | -2.9 | -9.0 | -9.1 | -1.6 | -4.3 | -4.7 |
| Enc Time [%] |  | 100% |  |  | 100% |  |  |
| Dec Time [%] (with ahg-memory measurements) |  | 101% |  |  | 103% |  |  |

## Memory Bandwidth Measurements

Memory bandwidth measurements were made using code in the HM-2.0-ahg-memory branch. Results comparing Zenverge RFC versus HM 2.0 are shown below.

Table 10: Memory bandwidth increase of Zenverge RFC versus HM 2.0 for RA-HE

|  | Random Access (Zenverge RFC vs HM 2.0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Memory bandwith increase % |  |  |  |  |  |  |  |
|  | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A | -1.4% | -12.4% | -20.1% | -23.1% | -28.1% | -40.2% | -29.4% | -31.7% |
| Class B | -3.0% | -12.9% | -19.2% | -21.7% | -25.4% | -39.9% | -26.7% | -30.1% |
| Class C | 0.9% | -11.2% | -20.2% | -23.9% | -29.7% | -45.1% | -34.4% | -38.4% |
| Class D | 7.0% | -7.7% | -18.5% | -23.3% | -29.8% | -46.5% | -35.3% | -39.4% |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.2% | -11.4% | -19.5% | -22.8% | -27.8% | -42.5% | -30.9% | -34.4% |

Table 11: Memory bandwidth increase of Zenverge RFC versus HM 2.0 for LD-HE

|  | Low Delay (Zenverge RFC vs HM 2.0) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Memory bandwith increase % |  |  |  |  |  |  |  |
|  | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.5% | -11.5% | -19.1% | -22.1% | -26.4% | -41.1% | -29.6% | -32.8% |
| Class C | 3.7% | -10.1% | -20.4% | -24.6% | -30.9% | -47.3% | -37.8% | -42.0% |
| Class D | 10.9% | -6.7% | -19.1% | -24.3% | -31.7% | -49.6% | -38.3% | -43.3% |
| Class E | -12.4% | -17.0% | -21.6% | -23.1% | -32.9% | -48.0% | -30.3% | -32.8% |
| All | 1.2% | -11.0% | -19.9% | -23.5% | -30.1% | -46.1% | -33.9% | -37.7% |

# References

T. Chujoh and R. Noda, “Internal bit depth increase except frame memory,” ITU-T SG16 Q.6 Document, VCEG-AF07, San Jose, April 2007.

# Patent rights declaration(s)

Zenverge, Inc., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).