| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E433 |
| --- | --- |

| Title: | Proposed text of JCTVC-D260 for WD2 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
|  | Wei Liu 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4712 wei.liu@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

The HM2.0 adopted a parallel processing friendly and lower complexity alternative to the context selection of significance map in TMuC 0.9 from JCTVC-D260. This contribution provides JCTVC-D262 text for WD2.

# Context Increment for a transform block

CABAC is used to encode the significance of a transform coefficient in a transform block based on the context increment, CtxInc. The CtxInc depends on the size of the transform block and the position (X,Y) of the transform coefficient inside a block.

## Context Increment for 4x4 and 8x8 transform block

For 4x4 blocks the context increment for position (X,Y) is

CtxInc      =  ( Y << 2 ) + X

For 8x8 blocks the context increment for the position (X,Y) is

CtxInc      = ( ( Y >> 1 ) << 2 ) + ( X >> 1)

## Context Increment for 16x16 and 32x32 transform block

For the calculation of context increment of Width xWidth transform blocks for Width == 16 or  Width ==32, the transform block with transform coefficient Coeff[ X, Y ] at position (X, Y) is divided into 4 regions:

- Region 1 consists of 4 positions where 0 <= X <=1 and 0 <= Y <= 1.

- Region 2 consists of the horizontal positions Y = = 0 and 2 <= X < Width.

- Region 3 consists of the vertical positions X = =0 and 2 <= Y < Width.

- Region 4 consists of all the positions of the block not in Region 1, 2, and 3.

### Context Increment for Region 1

The Region 1 consists of four positions, each position has its own context. The context increment is

CtxInc  =  ( Y << 1 ) + X

### Context Increment for Region 2

The Region 2 consists of the horizontal positions not in the Region 1. The context increment is

CtxInc  = ( Coeff [ X-1, 0 ] ? 1 : 0 ) + ( Coeff [ X-2, 0 ] ? 1 : 0 )

### Context Increment for Region 3

The Region 3 consists of the horizontal positions not in the Region 1. The context increment is

CtxInc  = ( Coeff [  0, Y-1 ] ? 1 : 0 ) + ( Coeff [ 0, Y-2 ] ? 1 : 0 )

### Context Increment for Region 4

The Region 4 consists of all other positions not in Region 1, 2, 3. If we let

CExt[ X, Y ] = Coeff[ X, Y]

for all  0 <= X < Width and 0 <= Y < Width, and

CExt[ X, Y] = 0

for -2 <= X < 0 or -2 <= Y < 0.

The context increment for position (X,Y) is

uiCtxInc  =                                                                                                 ( CExt [ X, Y-2 ] ? 1 : 0 )  +

( CExt [ X-1, Y-1 ] ? 1 : 0 )  +  ( CExt [ X, Y-1 ] ? 1 : 0 )  +

( CExt [ X-2, Y ] ? 1 : 0 ) +  ( CExt [ X-1, Y ] ? 1 : 0 )

# References

- Cheung Auyeung, Wei Liu, “Parallel processing friendly simplified context selection of significance map”, JCTVC-D260, 4th Meeting: Daegu, KR, 20-28 January, 2011.

- Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, “WD2: Working Draft 2 of High-Efficiency Video Coding”, JCTVC-D503, 4th Meeting: Daegu, KR, 20-28 January, 2011.

# Patent rights declaration(s)

Sony Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).