| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E435 |
| --- | --- |

| Title: | QuYK: A Universal, Lossless Compression Method for Quantization Matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gergely Korodi and Dake He 295 Phillip Street Waterloo, Ontario, Canada N2L 3W8 | Tel: Email: | +1 519 888 7465 {gkorodi, dhe}@rim.com |
| Source: | Research In Motion Ltd. |  |  |

_____________________________

# Abstract

The introduction of large block sizes in HEVC, and the need for transmitting quantization matrices for large blocks in the video sequence, makes the efficient lossless compression of these matrices an important task.  Two algorithms are proposed to solve this problem.  The first algorithm is applicable to general quantization matrices, and is universal in the sense that it is asymptotically optimal for any stationary, ergodic source model which generates the matrices.  In addition to the universality, this algorithm is designed with consideration of storage and computational complexity on the decoder side. The second algorithm is designed for a special case of quantization matrices. Its objective is to illustrate how compression efficiency can be improved by assuming that the input matrix satisfies a given set of constraints. Experimental results show that the universal algorithm compresses the 32x32 matrix in JCTVC-D024 into 664 bits and the specialized algorithm results in 474 bits. In comparison, for the same matrix, the best lossless method proposed in JCTVC-D024 produces 866 bits, and the lossless method in the AVC produces 1578 bits.

# Introduction

Let Q be an nxn matrix, where n is a power of 2, n >= 4.  Assume that Q is a quantization matrix for DCT-coefficients used in video coding.  In this case, each element in Q is an 8-bit unsigned integer, corresponding to the values qij in {0, …, 255 } (1 <= i, j <= n).  The objective is to provide for the matrix Q a compact representation, from which the original can be uniquely reconstructed at low computational complexity.

Since the properties of quantization matrices are the subjects of ongoing research, and keeping the format broad and open comes with practical benefits, the main proposal for the coding of quantization matrices is a universal compression algorithm, which is described in Section 2.

To have a better understanding about the difference in performance between a general-purpose and a special-purpose algorithm, in Section 3 an algorithm is described for a well-defined class of matrices, also discussed in [1].  This algorithm takes advantage of some special properties commonly present in those quantization matrices, thereby improving compression efficiency for that class.

# QuYK: A Universal Lossless Compression Method

In this section a lossless algorithm is proposed for compression of quantization matrices. This algorithm is universal in the sense that it is asymptotically optimal for any stationary, ergodic source which generates quantization matrices, and thus offers good compression performance for a broad range of quantization matrices.  A further strength of this algorithm is that its decoding complexity, both computational and regarding memory requirements, is very low.

This algorithm is a modified variant of the grammar-based compression algorithm proposed in [2], now commonly known as the YK algorithm. For this reason, the algorithm is called the QuYK (pronounced as ``Quick’’) algorithm.

Note that if additional prior knowledge of the data (quantization matrices) is available, the context-dependent YK algorithm proposed in [3] and its variant can be used to further improve the compression performance by taking advantage of the a priori knowledge.

Sequential Transform

Modified YK Encoder

Matrix Q

A value sequence

bitstream

Fig. 1 An illustration of the QuYK encoder

The encoding algorithm has two parts, as illustrated in Figure 1.  The first part, described in Section 2.1, transforms the matrix Q into a sequence, using the differences of consecutive values.  The second part encodes this sequence into a lossless representation of the matrix using a grammar-based transform, which is explained in Section 2.2.  The decoding process is reviewed in Section 2.3.

Throughout this document  the concept “ith semi-diagonal” Is used.  These are elements with indices qi-k+1,k where i is a fixed index from 1 to 2n-1, and k is a running index from 1 to i if i <=n, and from i-n+1 to n otherwise.  Accordingly, the “ith half semi-diagonal” is defined, where k is a running index from 1 to i/2 if i <= n, and from i-n+1 to i/2 + 1 otherwise.  Conceptually, a semi-diagonal is a 45° line starting from the left edge or bottom edge of the matrix Q, and going to the other edge; the half semi-diagonal stops at the main diagonal of Q.  The starting element of semi-diagonal i, i=1,...,2n-1 is called the base of that semi-diagonal, and it is denoted by B(i).  That is, B(i) = qi,1 if i <= n and B(i) = qn,i-n+1 if i > n.

2.1 Sequential transform

The first part transforms the matrix Q into a sequential representation.  There are various scanning methods to do this, so rather than being restrictive, here only some examples are provided .  Apart from the scanning order, a differential coding scheme DC is needed, which maps signed integers to unsigned values.  As an example, the following could be used DC(0) = 0, DC(1)=1, DC(-1)=2, DC(2)=3, and in general DC(k) = 2|k| - χ(k > 0).

If Q is not symmetrical, that is, S1>0 using the notations of Section 3.1, zig-zag scanning may be applied, where all symbols, except the first one, is coded using the difference of its predecessor.  The coding order and the resulting output symbols therefore are q11, DC(q21-q11), DC(q12-q21), DC(q13-q12), DC(q22-q13), DC(q31-q22), DC(q41-q31), DC(q32-q41), DC(q23-q32), DC(q14-q23), DC(q15-q14), DC(q24-q15), etc.  Another scanning technique is to encode the first column, then the last row, and then the remaining symbol of each semi-diagonal: q11, DC(q21-q11), DC(q31-q21),...,DC(qn1-qn-1,1), DC(qn2-qn1), DC(qn3-qn2), ..., DC(qnn-qn,n-1). The semidiagonal i is encoded by using its already known base B(i)=qi1 to start the differences: DC(qi-1,2-qi1), DC(qi-2,3-qi-1,2), ..., DC(q1i-q2,i-1).  For i>n the semi-diagonal is coded with the obvious changes in the indices.

If Q is symmetrical, then the scanning order omits the elements above the main diagonal of Q.  The first scanning order becomes q11, DC(q21-q11), DC(q31-q21), DC(q22-q31), DC(q32-q22), DC(q41-q32), DC(q51-q41), DC(q42-q51), DC(q33-q42), DC(q43-q33), DC(q52-q43) etc.  The second scanning order encodes the first column and the last row as before: q11, DC(q21-q11), DC(q31-q21),...,DC(qn1-qn-1,1), DC(qn2-qn1), DC(qn3-qn2), ..., DC(qnn-qn,n-1), then semi-diagonal i is encoded only up to the main diagonal: DC(qi-1,2-qi1), DC(qi-2,3-qi-1,2), ..., DC(q(i+1)/2,(i+1)/2-q(i+3)/2,(i-1)/2).  For i>n the semi-diagonal is coded with the obvious changes in the indices.

The resulting input sequence is called D.

Example.  The coding process is illustrated on the following matrix:

8,  11,  23,  26,  50,  53,  89,  92

11,  20,  29,  47,  56,  86,  95, 134

23,  29,  44,  59,  83,  98, 131, 137

26,  47,  59,  80, 101, 128, 140, 167

Q =   50,  56,  83, 101, 125, 143, 164, 170

53,  86,  98, 128, 143, 161, 173, 188

89,  95, 131, 140, 164, 173, 185, 191

92, 134, 137, 167, 170, 188, 191, 197

Using the second scanning order, the 8x8 Q matrix is transformed into the following sequence, considering that it is symmetrical:

D = { 8, 5, 23, 5, 47, 5, 71, 5, 83, 5, 59, 5, 35, 5, 11, 6, 5, 6, 6, 5, 5, 6, 6, 6, 5, 5, 5, 6, 6, 6, 5, 5, 6, 6, 5, 6 }

2.2 Grammar transform

In this section the final irreducible grammar G is constructed for D, by the irreducible grammar transform  described in Section III in [2].  This creates production rules R0, …, RM of the form Rk → G(Rk) = vk,1...vk,nk, with R0 denoting the input sequence D.  On the right-hand side of the rule, vk,j can be either a symbol from D, or a variable from G.  For each d in D define h(d) = 0, and for each production rule Rk, k = 0, …, M, let h(Rk) = max{h(vk,1), …, h(vk,nk)} + 1.  Sort the values h(R1), …, h(RM) in increasing order, and for each k = 1, …, M, let t(k) be the position of h(Rk) in the sorted sequence.  Relabel each variable according to k → t(k).  The resulting grammar forms the basis of the encoded output.

Example.  For the set D above, the final irreducible grammar is

R0 → 8 5 23 5 47 5 71 5 83 5 59 5 35 5 11 R1 R2 R3 5 R3 R4 6

R1 → 6 5

R2 → R4 5

R3 → 6 R2

R4 → 6 R1

For this grammar h(R1) = 1, h(R2) = 3, h(R3) = 4, h(R4) = 2,  the variables are relabeled as (R1, R2, R3, R4) → (R1, R3, R4, R2).  The final reordered grammar is:

R0 → 8 5 23 5 47 5 71 5 83 5 59 5 35 5 11 R1 R3 R4 5 R4 R2 6

R1 → 6 5

R2 → 6 R1

R3 → R2 5

R4 → 6 R3

Once this grammar is formed, the output is constructed, which is a single sequence of values, each value stored in a b-bit representation, in the following way.  A value A is fixed, which should be larger than any value in D, and another value B, such that A + M < B, and B + nk – 2 < 2b for each k = 1,…,M.  These values are known to both the encoder and the decoder, so they do not need to be transmitted in the compressed file.  Then in the relabeled final grammar, output G(Rk) for k = 1, …, M, such that

- For each d in D, write d in the output

- For each Rk, k = 1, …, M, write A + k in the output

- If the right-hand side of G(Rk) has length g > 2, then write B + g – 2 before G(Rk).

Finally, the sequence is terminated by writing B, followed by G(R0) expanded as above, but without its length, followed by A.

Example.  In this example the following is used:  b=8, and set A=128, B=224.  The first rule to write is R1, which is of length 2, and produces

6 5

This is followed by R2 → 6 R1, which is represented as (6, A+1):

6 129

Similarly, R3 and R4 become (A+2, 5) and (6, A+3), respectively:

130 5 6 131

This leads us to R0, for which B is written first, then G(R0), terminated by A:

224 8 5 23 5 47 5 71 5 83 5 59 5 35 5 11 129 131 132 5 132 130 6 128

The description of sequence D, hence matrix Q, is now complete.  The final output is

6, 5, 6, 129, 130, 5, 6, 131, 224, 8, 5, 23, 5, 47, 5, 71, 5, 83, 5, 59, 5, 35, 5, 11, 129, 131, 132, 5, 132,  130, 6 128.

2.3 Decoding with grammar transform

Corresponding to the encoding process, the decoding process of the QuYK algorithm consists of two parts, as illustrated in Figure 2: 1) decoding and parsing the grammar into a sequence, and 2) reconstructing the matrix from the sequence.

Modified YK Decoder

Inverse Sequential Transform

bitstream

A value sequence

Matrix Q

Fig. 2 An illustration of the QuYK decoder

As such, decoding works as follows, by sequentially processing the encoded values.  First  the production rules R1, …, RM  are reconstructed as follows. If a rule Ri starts with a value B+k, then we know the length |G(Ri)| of G(Ri)  is k+2, otherwise |G(Ri)| is 2.  Consequently, the next |G(Ri)| is read to initiate the reconstruction of Ri→G(Ri). Any value less than A refers to a symbol, and it is copied into the rule.  Any value of the form A+k, but less than B, refers to the rule Rk, which is already fully decoded by the time it is first referred, as guaranteed by the grammar construction algorithm.  At this point G(Rk) is substituted in place of A+k.  Finally, the first and only occurrence of B signals that the start rule R0, which is terminated by the unique symbol A, by which time has been fully recovered in the original sequence.

For the decoding procedure two variants are proposed: one being memory-efficient, the other being speed-efficient.  In the memory-efficient version production rules are stored in their grammar form, as illustrated by the example above, with both symbols and variables occurring at the right-hand side, and when R0 is processed, the variables are recursively expanded on reference.  In the speed-efficient version each rule is expanded to symbols as it is decoded, and subsequent references can simply copy those symbols without any recursive calls.

Example.  Continuing the example of Section 2.2, the decoder creates

R1 → 6 5

R2 → 6 6 5

R3 → 6 6 5 5

R4 → 6 6 6 5 5

R0 → 8 5 23 5 47 5 71 5 83 5 59 5 35 5 11 6 5 6 6 5 5 6 6 6 5 5 5 6 6 6 5 5 6 6 5 6

R0 now gives sequence D, from which the reconstruction of the original matrix Q is straightforward.

Experimental results show that QuYK compresses the 32x32 matrix in [1] into 664 bits. In comparison, for the same matrix, the best lossless method proposed in [1] produces 866 bits, and the lossless method in the AVC produces 1578 bits.

It is clear that though the matrix in [1] has special characteristics like symmetry, QuYK, being universal, is already competitive against the algorithm in [1]. In the next section, a method is further proposed to take advantage of such special characteristics. As a result, the method compresses the 32x32 matrix in [1] into 474 bits.

# A Compression Method for a Special Class of Matrices

When the data intended to be encoded is known to fall into a class with special properties, a dedicated compression algorithm can be designed to benefit from the presence of these.  By adding a residual correction algorithm, the algorithm can nevertheless work for any input matrix, though the compression will be efficient only if not too many corrections need to be made.

The algorithm starts by applying various transforms to the matrix.  These transforms, if deemed applicable, will change the shape of the elements to be coded, and subsequently the coding algorithm.  The transforms and the corresponding coding methods are listed in order.

In simple terms, the algorithm consists of three parts: (1) classification of the matrix, (2) curve fitting and coding, and (3) residual coding.  Part (1) is optional. First it is checked if the quantization matrix is symmetrical to the main diagonal (135°).  If the outlying elements have substantially low magnitude, the matrix is regarded as symmetrical.  Next  it is checked if mirroring elements to the anti-diagonal (45°) sum up to a constant value (inverse symmetry). Based on the symmetries found in Q, it is classified as one of three classes: Class 1 has no symmetry and the whole matrix is processed, Class 2 has 135° symmetry but no 45° symmetry and the lower triangle is processed, and Class 3 has both 135° and 45° symmetries and only the elements below the main diagonal and above the anti-diagonal are processed.

Once the symmetrical constraints; if detected in Part (1); are eliminated, the algorithm enters part (2) working on the semi-diagonals, which is defined as a sequence of values parallel to the anti-diagonal.  Based on the class of the matrix, these semi-diagonals are modeled either by a quadratic (Class 1) or a linear (Classes 2 and 3) expression.  That is, the best fitting quadratic or linear curve to each semi-diagonal is determined, and the descriptions of these curves is encoded.  Since a quadratic function is specified with three parameters, and the linear function only with two, encoding these functions takes usually significantly less space for large matrices, than encoding the corresponding semi-diagonals, whose lengths on average are proportional to the matrix size.  However, the fitting of the curves may not allow for a lossless reconstruction of the semi-diagonal values.  In case such a loss is not permissible by the application, part (3) includes an efficient residual-coding mechanism to correct the non-matching values.

## Classification of the matrix

For each i, j = 1, …, n, set qij = (qij + qji) / 2 if i <= j, and qij = (qij – qji) / 2 otherwise.  Evaluate S1 = (∑j>i |qij|) / (n*n).

f S1 is greater than a given constant T1, then it is determined that Q does not satisfy the 135º constraint and therefore belongs to Class 1. The encoder writes “0” to the output, and proceeds to the coding process described in Section 3.2.1.

If Q satisfies the 135º symmetry, it is reduced to a lower triangle matrix, and subsequently the algorithm will be working on half semi-diagonals.  To continue the classification process, the encoder computes the sum

R = (∑i<=j qij) / ((n*n + n)/2)

where the qij values run through each half semi-diagonal.  Then evaluate

S2 = (∑i+j<=n |qij + qn-j+1,n-i+1 – R|) / ((n*n + n)/2)

where this time the i index runs through 1 to n, and the j index runs through the ith half semi-diagonal.

If S2 is greater than a given constant T2, we regard that Q does not satisfy the 45° constraint and therefore belongs to Class 2.  The encoder writes “10” to the output, and proceeds to the coding process described in Section 3.2.2.

If S2 <= T2, then Q belongs to Class 3. The encoder writes “11” to the output followed by R, and proceeds to the coding process described in Section 3.2.3.

## Curve Fitting and Coding

### Class 1

This coding part fits quadratic curves to the semi-diagonals i=4,...,2n-4.  The outlying semi-diagonals (1, 2, 3, 2n-3, 2n-2, 2n-1) are short and they are encoded using exponential Golomb codes, using the following sequence: q11, DC(q21-q11), DC(q12-q21), DC(q13-q12), DC(q22-q13), DC(q31-q22), qnn, DC(qn-1,n-qnn), DC(qn,n-1-qn-1,n), DC(qn,n-2-qn,n-1), DC(qn-1,n-1-qn,n-2), DC(qn-2,n-qn-1,n-1).

Encoding the rest of Q in this case uses two discrete sets C1 and D1, known to both the encoder and decoder.  The size of these sets is always a power of 2.  The algorithm processes each semi-diagonal i = 4, ..., 2n-4 independently.  For the ith semi-diagonal, evaluate the expression

arg minbi0,bi1,bi2,xi ∑k (qi-k+1,k – (bi2k2 + bi1k + bi0 + xi))2

where the numbers bi0, bi1, bi2 run through all elements of D1, and xi runs through C1; k runs through the indices of the semi-diagonal.  Finally, for each semi-diagonal i, the encoder writes bi0, bi1, bi2 and xi in the output, using log2|D1|, log2|D1|, log2|D1| and log2|C1| bits, respectively.  After this the algorithm proceeds to the planar residual encoding phase described in Section 3.3.

### Class 2

This coding part fits linear functions to the half semi-diagonals i=5,...,2n-5.  The other semi-diagonals (1, 2, 3, 4, 2n-4, 2n-3, 2n-2, 2n-1) are output element-wise using exponential Golomb codes, using the following sequence: q11, DC(q21-q11), DC(q31-q21), DC(q22-q31), DC(q32-q22), DC(q41-q32), qnn, DC(qn,n-1-qnn), DC(qn,n-2-qn,n-1), DC(qn-1,n-1-qn,n-2), DC(qn-1,n-2-qn-1,n-1), DC(qn,n-3-qn-1,n-2).  The rest are processed as follows.

The first part of this process is to evaluate which half semi-diagonals follow overwhelmingly increasing or decreasing tendencies.  For the ith semi-diagonal (i=5,...,2n-5) compute

si = ∑k χ(qi-k+1,k <= qi-k,k+1),

where the χ function takes the value 1 if the condition is satisfied, and 0 otherwise.  Next, set σi = χ(si > i/2-1) for odd i values, and σi = χ(si <= i/2-1) for even i values.  The result of this computation is a bit sequence σi, i=5..,2n-5 which indicates the half semi-diagonals that decrease (σi = 0) or increase (σi = 1) for odd i values; for even i values this representation is the opposite.  Therefore, if the signs of the difference alternate between neighboring semi-diagonals, the σi sequence will be all 0.  This sequence is encoded with the linear residual encoding phase described in Section 3.2.3.

Once the σi values are specified, encoding Q continues by using two discrete sets C2 and D2, known to both the encoder and decoder.  The sizes of these sets are always powers of 2, and they are typically small, containing 4 or 8 elements.  The algorithm processes each half semi-diagonal independently.  For the ith half semi-diagonal, i = 5 ..., 2n-5 evaluate the expression

arg minbi,xi ∑k (qi-k+1,k – (bik + B(i) + xi))2

where the numbers bi run through all elements of (-1)σi D2 if i is odd and (-1)σi+1 D2 if i is even, and xi runs through C2; k runs through the indices of the half semi-diagonal.  Furthermore, for each half semi-diagonal, let ρi = |B(i) – B(i-1)|.  Once these values are determined, the encoder prepares the representation of Q.

First, the encoder outputs the ρk sequence using exponential Golomb codes in the following table.

| Values | Codes |
| --- | --- |
| 0 | 0 |
| 1-2 | 10x |
| 3-6 | 110xx |
| 7-14 | 1110xxx |
| 15-30 | 11110xxxx |
| 31-62 | 111110xxxxx |
| . . . | . . . |

Next, for each semi-diagonal i = 5...2n-5 the encoder writes bi and xi in the output, using log2|D2| and log2|C2| bits, respectively.  After this the algorithm proceeds to the planar residual encoding phase described in Section 3.3.

### Class 3

The coding of this class proceeds as in Section 3.2.2, except for this time only the half semi-diagonals from 1 to n need to be encoded, instead of 1 to 2n-1, as was the case in Section 3.2.2.  That is, the σi and ρi sequences are defined and coded as before, but only for i=1,...,n. This part is also concluded by the planar residual encoding phase described in the next section.

## Residual coding

The previous sections provided a compact, but so far possibly only approximate representation of the matrix Q.  In the case that the representation was not exact, the values that do not match need to be corrected.  Two simple residual encoding schemes are described: one for linear, the other for two dimensional (planar) data.

The linear residual encoding algorithm takes an input sequence v1,...,vn.  If all of these values are 0, a single “0” is written to the output, and the algorithm terminates.  Otherwise, a “1” is written to the output, and the algorithm proceeds to the next step.

In this step, the algorithm receives an input sequence v1,...,vn, and this time it knows that the sequence contains at least one non-zero value.  If the subsequence v1,...,vn/2 is all zero, output “00” and recursively process vn/2+1,...,vn.  If the subsequence vn/2+1,...,vn is all zero, output “01” and recursively process v1,...,vn/2.  Finally, if both parts contain non-zero values, output “1” and recursively process both.  This algorithm terminates when n=1; in this case, if the original sequence was not binary, a further symbol is output at this position, indicating the correct value.

The planar residual encoding algorithm takes an input matrix M[1,...,n][1,...,n].  It is denoted by UL, UR, LL, LR the upper left ([1,...,n/2][1,...,n/2]), upper right ([1,...,n/2][n/2+1,...,n]), lower left ([n/2+1,...,n][1,...,n/2]) and lower right ([[n/2+1,...,n]][ [n/2+1,...,n]]) quadrants of M.  If all of these values are 0, a single “0” is written to the output, and the algorithm terminates.  Otherwise, a “1” is written to the output, and the algorithm proceeds to the next step.

In this step, the algorithm receives the input matrix M, and this time it knows that the matrix contains at least on non-zero value.  The algorithm determines which quadrants contain non-zero values, and outputs the corresponding codeword from the following table:

| Non-zero quadrants | Codeword |
| --- | --- |
| UL | 0010 |
| UR | 0011 |
| LL | 0101 |
| LR | 1001 |
| UL, UR | 0100 |
| UL, LL | 0110 |
| UL, LR | 1010 |
| UR, LL | 0111 |
| UR, LR | 1011 |
| LL, LR | 1101 |
| UL, UR, LL | 1000 |
| UL, UR, LR | 1100 |
| UL, LL, LR | 1110 |
| UR, LL, LR | 1111 |
| UL, UR, LL, LR | 000 |

Next, the algorithm recursively processes each non-zero quadrants. The algorithm terminates when the matrix is reduced to a single value; in this case a further symbol is output at this position, indicating the correct value.

## 3.4.	Example

The coding process of Section 3 is illustrated on the same matrix used in Section 2.1.  Following the instructions of Section 3.1, S1=0, R=204.66 and S2=9.59.  Hence Q satisfies the 135° symmetry, but assuming that T2 < 9.59, which is reasonable, not the 45° symmetry, therefore it belongs to Class 2.  The sequence “10” is written to the output at this point.

Continuing with Section 3.2.2, the half semi-diagonals (1, 2, 3, 4, 2n-4, 2n-3, 2n-2, 2n-1) are encoded next.  That is, the values (8, DC(3), DC(12), DC(-3), DC(9), DC(-3), 197, DC(-6), DC(-3), DC(-3), DC(-12), DC(-3)) are encoded with the code table in Section 3.3.  This results in the sequence

1110001 11010 111101000 11011 111100010 11011 111111101000110 1110101 11011 11011 111101001 11011

The next step is to evaluate s5 = 0, s6 = 2, s7 = 0, s8 = 3, s9 = 0, s10 = 2, s11 = 0.  This yields σ5 = ... = σ11 = 0,  so the σ sequence is encoded by a single “0” bit.

The next step looks at B(5)=50, B(6)=53, B(7)=89, B(8)=92, B(9)=134, B(10)=137, B(11)=167, and obtains ρ5 = 24, ρ6 = 3, ρ7 = 36, ρ8 = 3, ρ9 = 42, ρ10 = 3, ρ11 = 30.  Encoding these values results in the sequence

111101001 11000 11111000101 11000 11111001011 11000 111101111

In this example, we set D2 = { 1, 2, 3, 4 } and C2 = {0, -0.5}.  Then, since all of the half semi-diagonals form an arithmetic progression with difference 3, we have bi = 2 and xi = 0 for all i = 5, ..., 11.  The encoded sequence is

10 0 10 0 10 0 10 0 10 0 10 0 10 0

This concludes Section 3.2.2, and part (3) as described in Section 3.3 begins.  However, the reconstructed matrix already matches the original, so at this point the encoder outputs a single “0”, and the algorithm terminates.  The resulting encoded sequence is

10111000 11101011 11010001 10111111 00010110 11111111 10100011 01110101

11011110 11111101 00111011 01111010 01110001 11110001 01110001 11110010

11110001 11101111 10010010 01001001 001000

with the total length of 166 bits.  This compares well with the best lossless result reported in [1] (proposed method level 1) for the same matrix, which was 178 bits, and the AVC method, which produces 344 bits.

# Annex A: Proposed syntax and semantics for QuYK

In line with the encoder description, the decoder has two parts: the first part decodes the grammar and reconstructs the sequential form of the matrix; the second part reassembles the matrix from this sequential description.

Decoding the grammar takes four parameters:

- size: the size of the matrix (n in Section 2.1)

- bits: number of bits representing a syntax element (b in Section 2.3)

- startRule: identifies where the start rule begins in the encoded sequence (B in Section 2.3)

- stopRule: the number of terminal symbols (A in Section 2.3)

The algorithm produces the array sequence (D in Section 2.3) of length seqLength.

It is not specified how the variables bits, startRule and stopRule are sent to the decoder.  One option would be to encode the value (bits-1) as u(3), and then compute stopRule = 1<<(bits-1), startRule = (1<<bits) – (1<<(bits-2)).

The following specification for parseQuantizationMatrix decodes and reconstructs the sequence encoded by the method in Section 2.2.

| parseQuantizationMatrix(size, bits, startRule, stopRule) { | Description |
| --- | --- |
| varIndex = 1 |  |
| symbol | u(bits) |
| while (symbol != startRule) { |  |
| varLength = 2 |  |
| if (symbol > startRule) { |  |
| varLength = symbol - startRule + 2 |  |
| symbol | u(bits) |
| } |  |
| ruleLength[varIndex] = 0 |  |
| for (k = 0; k < varLength; k++) { |  |
| if (symbol < stopRule) |  |
| prodRule[varIndex][ruleLength[varIndex]++] = symbol |  |
| else { |  |
| symbol -= stopRule |  |
| memcpy(prodRule[varIndex] + ruleLength[varIndex], prodRule[symbol], |  |
| ruleLength[symbol]) |  |
| ruleLength[varIndex] += ruleLength[symbol] |  |
| } |  |
| symbol | u(bits) |
| } |  |
| varIndex++ |  |
| } |  |
| seqLength = 0 |  |
| symbol | u(bits) |
| while (symbol != stopRule) { |  |
| if (symbol < stopRule) |  |
| sequence[seqLength++] = symbol |  |
| else { |  |
| symbol -= stopRule |  |
| memcpy(sequence + seqLength, prodRule[symbol], ruleLength[symbol]) |  |
| seqLength += ruleLength[symbol] |  |
| } |  |
| symbol | u(bits) |
| } |  |
| } |  |

How to reconstruct the original quantization matrix from the sequence depends on the scanning order used, and whether the matrix was symmetric or not.  Here we give the decoder for symmetric matrices, using the second scanning order, as in the example of Section 2.2. The process for general matrices can be similarly derived.

| reconstructSymmetricMatrix(size, sequence, seqLength) { | Description |
| --- | --- |
| len= 0 |  |
| matrix[0][0] = sequence[len++] |  |
| prev = matrix[0][0] |  |
| for (k = 1; k < size; k++) { |  |
| matrix[k][0] = prev + UInt2Int(sequence[len++]) |  |
| prev = matrix[k][0] |  |
| } |  |
| for (k = 1; k < size; k++) { |  |
| matrix[size - 1][k] = prev + UInt2Int(sequence[len++]) |  |
| prev = matrix[size - 1][k] |  |
| } |  |
| for (k = 1; k < size; k++) { |  |
| prev = matrix[k][0] |  |
| for (j = 1; k - j >= j; j++) { |  |
| matrix[k - j][j] = prev + UInt2Int(sequence[len++]) |  |
| prev = matrix[k - j][j] |  |
| } |  |
| } |  |
| for (k = 1; k < size - 1; k++) { |  |
| prev = matrix[size - 1][k] |  |
| for (j = 1; size - j - 1 >= k + j; j++) { |  |
| matrix[size - j - 1][k + j] = prev + UInt2Int(sequence[len++]) |  |
| prev = matrix[size - j - 1][k + j] |  |
| } |  |
| } |  |
| } |  |

The description becomes complete with the definition of the UInt2Int function, which is the inverse of theDC function in Section 2.1:

| UInt2Int(uiValue) { | Description |
| --- | --- |
| if (uiValue & 1) |  |
| return (uiValue + 1) / 2 |  |
| else if (uiValue > 0) |  |
| return - uiValue / 2 |  |
| else |  |
| return 0 |  |
| } |  |

# References

[1] Minhua Zhou and Vivienne Sze, “Compact representation of quantization matrices for HEVC,” JCTVC-D024, January 2011.

[2] En-hui Yang and John C. Kieffer, “Efficient universal lossless data compression algorithms based on a greedy sequential grammar transform---Part one: Without context models,” IEEE Trans. Info. Theory, vol. 46, pp. 755-777, May 2000.

[3] En-hui Yang and Da-ke He, “Efficient Universal Lossless Data Compression Algorithms Based on a Greedy Sequential Grammar Transform---Part Two: With context models,’’ IEEE Trans. Info. Theory, vol. 49, pp. 2874-2894, 2003.

# Patent rights declaration(s)

Research In Motion Ltd. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).