| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E437 WG11 Number: m19984 |
| --- | --- |

| Title: | On Intra Coding and MDIS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This document seeks to improve the mode dependent intra smoothing (MDIS) technique that was adopted in the previous meeting. MDIS filters the source pixels used for intra prediction with different degrees of smoothing, with the degree determined by the intra prediction direction.  While the technique provides gains for most sequences in the test set, this document observes losses in coding efficiency for smaller resolution sequences.  The document reports that using edge directed smoothing in the construction of the neighborhood mitigates these issues.  It is proposed to adopt the edge directed smoothing into the HM.

# Introduction

Intra prediction is currently realized by partitioning a largest coding unit (LCU) into one or more blocks through a recursive splitting process. Each block is predicted spatially and subsequently refined, and the prediction is performed sequentially using neighboring reconstructed blocks.

Mode dependent intra smoothing is a technique to selectively apply intra smoothing to the neighbor pixels of intra prediction. The two smoothing filter is shown below as described in JCTVC-D282. For each prediction block, the codec then select either filter 0 (no filter) or filter 1 or filter 2 according to the LUT shown in Table 1. The filter index selected depends on the block size and prediction mode.

c

c

c

c

c

Prediction samples

c

c

c

c

c

Prediction samples  Filtered 1

3

tap filter

c

c

c

c

c

Prediction sampled Filtered 2

3

tap filter

Fig. 1. Filter Types in MDIS (from JCTVC-D282)

Table 1. Table of MDIS Filter Index

We compared the performance of HM2.0 with MDIS off to HM2.0. It shows an average gain of about 0.4% for intra, and 0.8% for intra loco.  We believe that this is reasonable gain for the complexity of the method, and we support the tool.  However, we also observe sequences that are not well served by the method.  For example, BasketballDrill shows a drop in coding efficiency of 1.2% and 2.4% for high efficiency and low complexity configurations, respectively.  It is our concern that MDIS may show similar performance on other sequences that are not in the test set.

Table 2 RD Performance of HM2.0 with MDIS off vs. HM2.0

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.8 | 0.8 | 0.8 | 1.7 | 1.0 | 0.9 |
| Class B | 0.4 | 0.4 | 0.4 | 0.9 | 0.2 | 0.1 |
| Class C | -0.1 | 0.1 | 0.1 | -0.1 | -0.1 | 0.0 |
| Class D | 0.2 | 0.3 | 0.3 | 0.4 | 0.2 | 0.1 |
| Class E | 0.5 | 0.6 | 0.5 | 1.3 | 0.4 | 0.4 |
| All | 0.4 | 0.4 | 0.4 | 0.8 | 0.3 | 0.3 |

| Intra All frame | NO MDIS vs HM2.0 Intra |  |  | NO MDIS vs HM2.0 Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | BD-rate U | BD-rate V | BD-rate Y | BD-rate U | BD-rate V |
| Traffic | 0.4 | 0.6 | 0.7 | 1.1 | 0.5 | 0.5 |
| PeopleOnStreet | 0.6 | 0.8 | 0.9 | 1.7 | 0.8 | 0.8 |
| Nebuta | 1.5 | 1.0 | 0.9 | 2.1 | 1.1 | 1.1 |
| SteamLocomotive | 0.7 | 0.9 | 0.8 | 1.8 | 1.5 | 1.1 |
| Kimono | 0.5 | 0.5 | 0.5 | 1.2 | 0.6 | 0.6 |
| ParkScene | 0.5 | 0.6 | 0.6 | 1.3 | 0.7 | 0.7 |
| Cactus | 0.6 | 0.6 | 0.5 | 1.1 | 0.5 | 0.3 |
| BasketballDrive | 0.6 | 0.7 | 0.7 | 0.9 | 0.2 | 0.3 |
| BQTerrace | 0.0 | -0.3 | -0.4 | 0.0 | -0.8 | -1.4 |
| BasketballDrill | -1.2 | -0.9 | -0.9 | -2.4 | -1.5 | -1.1 |
| BQMall | 0.4 | 0.5 | 0.5 | 0.8 | 0.4 | 0.4 |
| PartyScene | 0.2 | 0.3 | 0.4 | 0.4 | 0.3 | 0.3 |
| RaceHorses | 0.3 | 0.3 | 0.3 | 0.9 | 0.3 | 0.3 |
| BasketballPass | 0.2 | 0.3 | 0.3 | 0.5 | 0.1 | 0.0 |
| BQSquare | 0.1 | 0.1 | 0.0 | 0.2 | 0.0 | 0.0 |
| BlowingBubbles | 0.2 | 0.5 | 0.4 | 0.4 | 0.4 | 0.3 |
| RaceHorses | 0.2 | 0.4 | 0.4 | 0.7 | 0.1 | 0.2 |
| Vidyo1 | 0.2 | 0.7 | 0.5 | 1.4 | 0.2 | 0.0 |
| Vidyo3 | 0.7 | 0.7 | 0.7 | 1.2 | 0.4 | 0.5 |
| Vidyo4 | 0.5 | 0.3 | 0.3 | 1.3 | 0.5 | 0.6 |

# MDIS with Edge Detection

In studying the issue, we determined that using an edge directed smoothing operation in the construction of the prediction samples could equalize the tool performance.  The motivation is that if the neighbor pixels used for intra smoothing are on an edge, intra smoothing is poorly motivated.

Figure 2. illustrates the process, which is described as follows: Let p(i-1), p(i), p(i+1) denote the 3 pixels used for intra smoothing, p1(i) store the filtered pixels from filter 1, and p2(i) stores the filtered pixels from filter 2. For the case that the difference between p(i-1) and p(i+1) is greater than a threshold, we set p2(i)=p1(i)=p0(i).  For the case that the difference between p(i-1) and p(i+1) is less than a threshold, we generate p2(i) and p1(i) as is currently done in the HM.

In our experiments, we determined that it is best to vary the threshold by block size and resolution.  This is intuitive, as the larger blocks are frequently chosen to code smooth regions, and these smooth regions benefit from more smoothing of the prediction neighbors.  Similarly, reliably detecting edges in the higher resolution sequences require an analysis window greater than p(i-1), p(i) and p(i+1), and the threshold should be increased due to this.

Fig. 2 Proposed MDIS with edge detector

## Results

We implemented the proposed method into the HM2.0.  Results show that our enhanced MDIS addresses the coding variability of MDIS.  Specifically, the technique provides 1.6% and 2.1% bit-rate reduction compared to the current MDIS for BasketballDrill with intra and intra_loco configurations, respectively.

Table 3. Summary RD results of edge directed MDIS vs. HM2.0

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 | 0.1 | -0.1 | 0.1 |
| Class B | 0.0 | -0.1 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class C | -0.4 | -0.2 | -0.2 | -0.3 | -0.3 | -0.3 |
| Class D | 0.0 | 0.1 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class E | 0.1 | 0.2 | 0.1 | 0.2 | 0.0 | 0.1 |
| All | -0.1 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |

| Intra All frame | Proposed vs Hm2.0 Intra |  |  | Proposed vs HM2.0 Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Y | BD-rate U | BD-rate V | BD-rate Y | BD-rate U | BD-rate V |
| Traffic | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| PeopleOnStreet | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Nebuta | 0.1 | 0.1 | 0.2 | 0.2 | 0.0 | 0.0 |
| SteamLocomotive | 0.0 | 0.3 | 1.1 | 0.0 | -0.2 | 0.5 |
| Kimono | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| ParkScene | 0.1 | 0.1 | -0.2 | 0.1 | 0.0 | 0.0 |
| Cactus | 0.0 | -0.1 | 0.1 | 0.1 | -0.1 | 0.2 |
| BasketballDrive | 0.0 | -0.2 | -0.1 | 0.0 | 0.1 | 0.0 |
| BQTerrace | -0.1 | -0.1 | 0.1 | 0.0 | -0.3 | -0.5 |
| BasketballDrill | -1.6 | -1.3 | -1.3 | -2.1 | -1.7 | -1.3 |
| BQMall | 0.2 | 0.3 | 0.2 | 0.4 | 0.2 | 0.1 |
| PartyScene | 0.1 | 0.2 | 0.2 | 0.2 | 0.1 | 0.1 |
| RaceHorses | 0.0 | 0.0 | 0.1 | 0.4 | 0.1 | 0.0 |
| BasketballPass | -0.1 | 0.1 | 0.1 | 0.1 | 0.0 | -0.1 |
| BQSquare | 0.0 | -0.1 | -0.1 | 0.1 | 0.0 | -0.1 |
| BlowingBubbles | 0.0 | 0.2 | 0.2 | 0.1 | 0.2 | 0.0 |
| RaceHorses | -0.1 | 0.1 | 0.1 | 0.2 | 0.0 | 0.1 |
| Vidyo1 | -0.3 | -0.3 | -0.8 | 0.0 | 0.0 | 0.0 |
| Vidyo3 | 0.4 | 0.2 | 0.8 | 0.0 | 0.0 | 0.0 |
| Vidyo4 | 0.1 | 0.7 | 0.1 | 0.2 | 0.0 | 0.0 |

# Conclusion

We propose to modify the MDIS in HM2.0 with an edge detector. This modification results in a 1.8% and 2.3% rate reduction for the BasketballDrill sequence which MDIS. Meanwhile, it has very little impact on all the other sequences. Therefore it makes MDIS more robust and improves the overall coding efficiency.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC-D282, “CE13: Mode Dependent Hybrid Intra Smoothing,” 4th JCT-VC Meeting, Daegu, Jan. 2011.

- JCTVC-D600, “Common test conditions and software reference configurations,”, 4th JCT-VC Meeting, Daegu, Jan. 2011.