| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E438 |
| --- | --- |

| Title: | Cross-verification of simplification and improvement of merge mode coding (JCTVC-E292) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peisong Chen Yunfei Zheng Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-845-1347 peisongc@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-E292 proposed by Tsinghua University and Huawei. Proposed tools have been evaluated on the common test condition, which is defined in JCTVC-D600. The source code was provided by Huawei to cross verify. It is confirmed that the results generated by the cross-checkers match ones provided by Huawei.

# Cross verification of JCTVC-E292

JCTVC-E292 proposed a method to reduce the complexity of merge based motion information coding. There are two modifications. One is removal of temporal collocated candidate and the other is a new process to derive left and right candidates for merge coding. In all of the following tests, the anchor data are generated by HM-2.0 under the common test condition defined in JCTVC-D600.

Removal of temporal collocated candidates

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | -0.1 | -0.2 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.2 | -0.2 | -0.1 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.5 | -0.5 | -0.5 | -0.1 |
| Class C | 0.0 | 0.1 | -0.1 | -0.3 | -0.2 | -0.4 |
| Class D | -0.3 | -0.5 | -0.9 | -0.4 | -0.5 | -0.6 |
| Class E | -0.3 | -1.0 | -0.1 | -0.4 | -0.7 | -0.3 |
| All | -0.1 | -0.3 | -0.4 | -0.4 | -0.4 | -0.3 |

Removal of temporal collocated candidate and new left and right candidate derivation

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.2 | -0.1 | -0.2 | -0.2 |
| Class B | -0.1 | -0.1 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class C | -0.1 | -0.2 | -0.2 | -0.2 | -0.2 | -0.1 |
| Class D | -0.1 | -0.1 | -0.1 | -0.2 | -0.3 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 | -0.2 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.3 | -0.6 | -0.6 | -0.3 |
| Class C | 0.0 | 0.0 | -0.2 | -0.4 | -0.3 | -0.5 |
| Class D | -0.3 | -0.1 | -0.5 | -0.4 | -0.6 | -0.5 |
| Class E | -0.3 | -0.2 | -0.1 | -0.5 | -0.7 | -0.4 |
| All | -0.2 | -0.1 | -0.3 | -0.5 | -0.6 | -0.4 |

It is confirmed that the results generated by the cross-checkers match ones provided by Huawei.

# References

[1]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, January 2011.