| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E440 WG11 Number: m19988 |
| --- | --- |

| Title: | CE1: Cross-check result of Huawei’s STDM (JCTVC-E294) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This report provides cross-check results of STDM proposed by Huawei in JCTVC-E294. The software package obtained from proponents was successfully compiled and tested under the common test conditions. The encoder was compiled in Linux OS and was tested in a parallel computing farm system. The decoder was complied with VS2008 and tested on a single server with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM.

The accompanied spreadsheet contains the detailed data of the results, and the average performance data of BD rate and encoding and decoding time are shown in following table. Only the version w/o MV refinement was tested with random access cases. The BD rate results perfectly match the data provided by the proponent (There may be a typo in the abstract of JCTVC-E294.The average gain for RA@HE case should be 0.91%, not 0.93%). Besides, it is noted that the decoding time was tested with the writing of YUV output.

Table 1 Crosscheck results of random access cases (w/o refinement)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.3 | -1.5 | -0.8 | -1.0 | -0.9 |
| Class B | -0.8 | -0.9 | -0.9 | -0.7 | -0.8 | -0.7 |
| Class C | -0.9 | -1.1 | -1.1 | -0.6 | -0.7 | -0.7 |
| Class D | -1.1 | -1.1 | -1.0 | -0.7 | -0.8 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -1.1 | -1.1 | -0.7 | -0.8 | -0.8 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 105% |  |  | 106% |  |  |