| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E442 |
| --- | --- |

| Title: | Cross-verification of MediaTek proposal on motion vector decimation (JCTVC-E092) and context dependent intra mode coding (JCTVC-E201) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Peisong Chen Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-845-1347 peisongc@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-E092 and JCTVC-E201 proposed by MediaTek. Proposed tools have been evaluated on the common test condition, which is defined in JCTVC-D600. The source code was provided by MediaTek to cross verify. It is confirmed that the results generated by the cross-checkers match ones provided by MediaTek.

# Cross verification of JCTVC-E092

MediaTek proposal (JCTVC-E092) introduced a motion vector decimation method for storing the motion vector data at lower resolution. In this proposal, the motion vectors from the bottom right block in a compression granularity are used as the motion vectors of the whole granularity instead of the top left one in HM2.0. In the following tests, the anchor data are generated by HM-2.0 under the common test condition defined in JCTVC-D600.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 | -0.2 |
| Class B | -0.2 | -0.1 | -0.1 | -0.1 | 0.0 | 0.0 |
| Class C | -0.3 | -0.2 | -0.3 | -0.2 | -0.2 | -0.2 |
| Class D | -0.4 | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.0 | -0.2 | -0.2 | -0.3 | -0.2 |
| Class C | -0.4 | -0.2 | -0.3 | -0.5 | -0.3 | -0.6 |
| Class D | -0.6 | -0.6 | -0.6 | -0.5 | -0.6 | -0.4 |
| Class E | -0.5 | -0.9 | -0.4 | -0.6 | -0.5 | -0.5 |
| All | -0.4 | -0.4 | -0.3 | -0.4 | -0.4 | -0.4 |

# Cross verification of JCTVC-E201

MediaTek proposed a context dependent intra mode coding. The codeword for the intra mode of the current block is adaptively selected from 9 coding tables according to the intra modes of its neighboring blocks. The test is run for the high efficiency all intra coding case. The anchor data are generated by HM-2.0. Both the anchor and the proposed method use the common test condition defined in JCTVC-D600. It is confirmed that the results generated by the cross-checkers match ones provided by MediaTek.

|  | Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.3 | -0.2 |
| Class B | -0.4 | -0.3 | -0.3 |
| Class C | -0.3 | -0.2 | -0.1 |
| Class D | -0.3 | -0.2 | -0.2 |
| Class E | -0.6 | -0.2 | -0.2 |
| All | -0.4 | -0.2 | -0.2 |

# References

[1]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, January 2011.