| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E443 |
| --- | --- |

| Title: | Cross-verification of MediaTek’s proposal on Reduction in Combinations of Reference Picture Indices for Bi-Prediction ( JCTVC-E052) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross verification results on JCTVC-E052 proposed by MediaTek. Proposed tools have been evaluated on the common test condition, which is defined in JCTVC-D600. The source code was provided by MediaTek to cross verify. It is confirmed that the results on low delay configuration match ones provided by MediaTek.

# Introduction

MediaTek’s proposal [1] is a technique related to the low delay configurations. By reducing the combination of the reference picture indices for Bi-prediction, the encoding complexity is decreased. It is because less motion estimation is performed at the encoder. The BD-rate and the measurement of encoding and decoding time are shown in this report.

# Results

## Test condition and machine platform

Experiment was carried out based on HM-2.0 and the configuration and common test condition are as JCTVC-D600 [2].

The encoding time was measured on Linux (64-bit) cluster and the decoding time was measured on Windows 7 with Inter Xeon CPU, 2.8G Hz and 6GB memory.

## BD-rate and measurement of encoding and decoding time

The experimental results are summarized in the following tables. The detailed results are shown in attached excel-file. Table 2-1 shows the coding performance for the changes on inter prediction and Table 2-2 shows the coding performance for the further changes on the derivation on motion vector predictors.

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 0.9 | 0.8 | 0.6 | 0.9 | 1.1 |
| Class C | 0.6 | 0.6 | 0.7 | 0.3 | 0.4 | 0.2 |
| Class D | 0.5 | 0.5 | 0.3 | 0.4 | 0.3 | 0.3 |
| Class E | 0.6 | 0.6 | 1.0 | 0.4 | 0.6 | 0.3 |
| All | 0.6 | 0.7 | 0.7 | 0.4 | 0.5 | 0.5 |
| Enc Time[%] | 82% |  |  | 78% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table2-1: Results for low delay configurations

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.6 | 1.0 | 0.8 | 0.4 | 0.9 | 1.2 |
| Class C | 0.4 | 0.6 | 0.5 | -0.1 | 0.1 | 0.1 |
| Class D | 0.5 | 0.4 | -0.1 | 0.2 | 0.8 | 0.6 |
| Class E | 0.5 | 0.5 | 0.5 | 0.2 | 0.8 | 0.4 |
| All | 0.5 | 0.6 | 0.4 | 0.2 | 0.6 | 0.6 |
| Enc Time[%] | 84% |  |  | 79% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table2-2: Results for low delay configurations

# Conclusion

In this contribution, the cross verified results are reported for MediaTek’s proposal.

# References

[1] T.-D. Chuang, J.-L. Lin, Y.-W. Huang, S. Lei,“ Reduction in Combinations of Reference Picture Indices for Bi-Prediction”, JCTVC-E052, 5th. JCT-VC Meeting, Geneva, CH, March 2011.

[2]JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, January 2011.