| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E446_r1 |
| --- | --- |

| Title: | Coefficient coding improvement in LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seongwan Kim Jaeho Lee Sangyoun Lee Yonsei University | Tel: Email: | +82-2-2123-5768 knauer@yonsei.ac.kr jhlee82@yonsei.ac.kr syleee@yonsei.ac.kr |
| Source: | Yonsei University |  |  |

_____________________________

# Abstract

In this report, additional run-level coding in LCEC, for intra and inter, is proposed. The results show that an average coding gain of 0.6 % is achieved for intra, and 0.4% for random access case.

# Introduction

In current TM, run-level coding is performed for inter and intra, separately. In inter, run-level table from TEN [1] is used and Qualcomm method [2] is used for intra. In run level coding for inter, the same mapping table from { run, level } pairs to coded number, cn is used for all transform sizes, 4x4, 8x8, 16x16 and 32x32. In intra, 2 VLC tables are used for 4x4 and other larger sizes. Herein, coding parameter is determined using coded tr1s and maxrun.

# Proposed coefficient coding method

## Intra code table modification

In run-level coding for intra block [2], 2 VLC tables are used. One is for 4x4, and another table is for remaining transform sizes, 8x8, 16x16 and 32x32. In proposed method, a table is added for considering 8x8 and larger sizes (16x16 and 32x32), separately.

Additionally, the mapping table of the VLC table indices, which is currently based on maxrun, is extended to consider transform size and coded tr1s. Therefore, the mapping tables are modified as Table I. Proposed method uses six tables with 670 elements for intra coding. It is 2.7times larger than HM2.0.

Table 1. Reference table size

|  | HM 2.0 |  | Proposed |  |
| --- | --- | --- | --- | --- |
|  | VlcTableNumber | Param. table | VlcTableNumber | Param. table |
| 4x4 | 1x29 (unified) | 5x15 | 1x15 | 5x15 |
| 8x8 |  | 5x29 (same table) | 5x29 | 5x29 |
| >= 16x16 |  |  | 5x29 | 5x29 |

## Run-level table re-allocation in Inter block in LCEC

In current TM, run-level table for inter blocks is used to map {run, level} pairs to code number, cn. This table is applied to all transform sizes, equally. Based on the statistical distribution of run and level according to each transform size, the hit ratio of {lev=0 & run = maxrun+1} and {lev=1 & run=0} is different in transform sizes. Accordingly, to optimize run-level table according to each transform size, cn values are switched between {lev=0 & run = maxrun+1} or {lev=1 & run=0} and run-level position from table, which is designed and added.

To reduce additional table size, run-level table is re-designed, optimized for 4x4 when maxrun is from 0 to 14 and for 8x8 when maxrun is from 15 to 27. Accordingly, when transform size is 4x4, re-designed run-level table is used without switching. In 8x8 size, run-level switching is performed only when maxrun is from 0 to 14. For 16x16 and 32x32, run-level switching is performed in all maxruns.

Figure 1. Example in maxrun = 5

# Experimental results

The common test condition for low configuration specified in [2] is used for all the experiments. In case of random access and low delay configurations, proposed LCEC for inter and intra has been performed. The proposed method has been cross-checked by Samsung Electronics Co. The detailed experimental results are available in the attached Excel file.

Table 2 shows the summarized results of the proposed coding scheme of sub-Section 2.1 and 2.2. Average Y BD-rate savings are 0.6%, 0.4%, and 0.0%, respectively. The performance improvements are achieved all intra loco configuration and random access configuration. On the other hand, the proposed method does not obtain BD rate saving in low delay configuration. The encoding and decoding time of the proposed method shows that the proposed method is not affect the complexity in encoder and decoder, highly.

Table 2. Result summary.

|  | Intra LoCo |  |  |  | Random access LoCo |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR | Y BDR | U BDR | V BDR |
| Class A | -0.9 | -0.3 | -0.4 | -0.8 | 0.7 | 0.7 |  |  |  |
| Class B | -0.6 | -0.6 | -0.5 | -0.4 | 0.1 | 0.0 | -0.1 | 0.1 | 1.2 |
| Class C | -0.4 | -0.5 | -0.5 | -0.2 | -0.1 | -0.2 | -0.1 | 0.7 | 0.4 |
| Class D | -0.4 | -0.4 | -0.5 | -0.3 | 0.0 | -0.3 | 0.0 | 0.5 | 0.5 |
| Class E | -0.5 | -0.0 | -0.2 |  |  |  | 0.1 | 0.8 | 2.1 |
| All | -0.6 | -0.4 | -0.4 | -0.4 | 0.2 | 0.0 | 0.0 | 0.8 | 1.0 |
| EncTime[%] | 96% |  |  | 100% |  |  | 97% |  |  |
| DecTime[%] | 97% |  |  | 102% |  |  | 99% |  |  |

# Conclusion

In this report, additional run-level coding scheme for intra and inter in LCEC is proposed, and its experimental results are presented. For coding improvement, the proposed method considers run-level coding in large block sizes. The Section 3 shows that the proposed method achieves the performance improvement in intra only and random access configurations without complexity increase, while no improvement in low delay configuration.

From the results, run-level coding in LCEC should consider large block sizes to obtain coding performance improvement. However, the proposed run-level coding scheme needs more improvement in terms of table parameters and table size. Therefore, we recommend the proposed method to CE for further experiments.

# References

[1] Nokia, LM Ericsson, Tandberg, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson”, Doc JCTVC-A119, April 2010

[2] Marta Karczewicz, Xianglin Wang, Wei-Jung Chen “CE5: Improved coefficient coding with LCEC”, JCTVC-D374, Daegu, Korea, Jan 2011

[3] Frank Bossen “Common test conditions and software reference configurations”, JCTVC-D600, Jan 2011

# Patent rights declaration(s)

Yonsei University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).