| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E451 |
| --- | --- |

| Title: | Impact of Motion Vector Predictor Index Decoding on Parallelization between CABAC and Other Decoder Components |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong Yu, Chris Payson 200 Brickstone Square, Andover, MA01810, USA | Tel: Email: | 978-719-1345 yongyu@broadcom.com, payson@broadcom.com, |
| Source: | Broadcom Corporation |  |  |

_____________________________

# Abstract

This contribution addresses the following issues caused by motion vector prediction candidate in current HEVC design: parallelizing CABC decoder and the other components of coders, parsing robustness, extra external memory bandwidth and on chip storage in a general SOC system. It proposes a modification of motion vector prediction candidate index algorithm. The cost of this change is about 0.3% for high efficiency random access category, 0.2% for low delay category. The proposed modification has no affects on other categories.

# Introduction and Problem Statement

H.264/AVC coding standard is designed in a way which allows CABAC decoder/encoder to convert bits to bins in parallel with the rest of systems at minimal extra computational cost. CABAC is designed to use only local context within a slice, an independent coding element. To facilitate the buffer size management, H.264 coding standard also limit the conversion rate of the bits and bins (the input and output of the CABAC coder) from various studies [1, 2, 3]. Without this limit, a decoder designer has to consider worst-case scenarios where the effective incoming bin rate (due to an unlimited expansion from bits to bins) could become basically unlimited. This feature has helped to design various efficient SOCs and reduce the cost of SOCs significantly. Parallelized CABAC encoder/decoder is popular in various H.264 encoder and decoder SOC designs.

Parallelized CABAC encoder/decoder requires the CABAC itself to be self-independent in a slice. For example, on the decoder side of H.264 codec, to convert the bit streams to bins as defined in the standard, H.264 CABAC only needs the information within the same slice, and the sequence/picture parameter headers.

Now, studies in HEVC found that using collocated motion vector as the motion vector predictor generate remarkable amount of compression gain (up to 2.6%).

However, the introduction of collocated motion vector in the CABAC parsing process makes the parallelization of CABAC coder extremely difficult in the following aspects:

- CABAC coder needs to maintain an independent DPB model in order to find the correct reference frames to get collocated motion vectors. The motion vectors in all the reference frames have to be saved in the life time in the DPB model until a frame is “bumped” out.

- CABAC coder needs to duplicate the construction of motion vectors for the entire frame and save the constructed motion vectors to external memory. Assuming the smallest prediction block size is 4x4, the bandwidth of storing and fetching collocated motion vectors is equivalent to fetching pixels for a normal P frame motion compensation. Most of all, the access of the external memory for the collocated motion vectors is very irregular since the coded size of each “macro block” can vary drastically within a frame and between frames. The erratic access of external memory is almost impossible to be satisfied on a regular SOC.

- Additional one row of internal memory buffer for the motion vector is required. This is equivalent to about 1 row of raw pixels, assuming the smallest prediction block size is 4x4.

This contribution proposed a syntax modification of the coding process of motion vector prediction to maintain the parallelism of CABAC. This contribution also solves the issue raised by parsing robustness ad hoc group [4, 5, 6] for the high efficiency categories, by removing past vector errors from current syntax parsing.

# Proposed Modification

This contribution proposes the following changes. It replaces the calculation of the total number of motion vector prediction candidates, and splits the motion vector candidate index into to two binary syntaxes, collocated_MVP_flag and prediction_direction_flag. See the following table (syntax and description in red show the difference from the existing work draft [7]).

Table 1 Proposed Syntax Change

if( inter_pred_idc[ x0 ][ y0 ] != Pred_L0 ) {

if( num_ref_idx_l1_active_minus1 > 0 )

ref_idx_l1[ x0 ][ y0 ]

mvd_l1[ x0 ][ y0 ][ 0 ]

mvd_l1[ x0 ][ y0 ][ 1 ]

if(any spatial predictor is available)

collocated_MVP_flag

if(collocated_MVP_flag == 0 && both spatial predictors are available)

prediction_direction_flag (either left or above)

}

This proposed algorithm is described as follows.

- The coder first checks the availability of the spatial motion vector predictors. If neither the left nor the above block are available for motion vector prediction (for normal reasons), the MVP for current block will be derived from the collocated block. In case the collocated block is not available, the MVP for the current block is set to zero.

- Otherwise, (either spatial neighbor block is available), a flag “collocated_MVP_flag” is signaled in the bit stream. collocated_MVP_flag = 1 indicates that the MVP is derived from the collocated block. Again, in case the collocated block is not available, the MVP for the current block is set to zero. Collocated_MVP_flag = 0 is used to indicate that the MVP is derived from neighbor blocks.

- .

- If only one neighbor block is available, the MVP of the current block is assigned with the available MVP.  Otherwise, a “prediction_direction_flag” is signaled in the stream to indicate which block the MVP is from (top or above).

With this modification, the CABAC decoders do not need to construct the motion vectors. Neither fetching the collocated motion vectors nor saving the motion vector of the current frame is required. The CABAC decoders only need to save the block type (intra or inter) information. The block type information already must be available regardless, since it is used in the CABAC coding context to decode current block type.

The modified coding process still uses the collocated motion vector as a potential motion vector predictor. However, the impact of losing collocated slices is the same as B frame direct mode. The error from the collocated slices will not propagate to current slice.

The impacts of this modification on complexity of both encoder and decoder are negligible. This modification eliminates motion vector comparison to construct motion vector predictor candidate list.

# Experimental results

The modification proposed in this proposal is based on the latest test model HM2.0. The experiments is conducted follow the guide line described in [8]

Table 2 Summary of bit rate impact from the modified syntax

|  | Y BDR |  |
| --- | --- | --- |
|  | Random access | Low delay |
| Class A | 0.21% | n/a |
| Class B | 0.23% | 0.16% |
| Class C | 0.36% | 0.36% |
| Class D | 0.52% | 0.36% |
| Class E | n/a | 0.10% |
| All | 0.3% | 0.2% |

On average, there is an average bit rate loss of 0.3% in the random access category. The compress loss is mostly from the smaller picture dimension. There is an average of 0.2% for the low delay category.

# Conclusions

The current design of MVP candidate index coding causes significant amount of difficulty on CABAC parallelization. It breaks the conventional independency of CABAC coding on inter slice information. The compression loss is only as much as 0.2 - 0.3%, or equivalently 0.01 dB. In addition, the compression loss is mostly from the smaller picture dimension. We suggest keeping the conventional independency of CABAC coding and changing the MVP candidate index coding process as in this proposal.

# References

[1] Frank Bossen, "Bounding the complexity of arithmetic decoding", Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG, 3rd Meeting Fairfax, Virginia, USA, 6-10 May, 2002.

[2] Eric Viscito, "Macroblock level limit on the number of binary symbols in CABAC", Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 6th Meeting: Awaji, Island, Japan, 5-13 December, 2002.

[3] Gary J. Sullivan et al, "Recommendation ITU T H.264: Advanced video coding for generic audiovisual services"

[4] Jian-Liang Lin et al, "Syntax for AMVP Parsing Error Control", Joint Collaborative Team on Video Coding (JCT-VC), 4th Meeting: Daegu, KR, 20-28 January, 2011.

[5] Bin Li, et al, "Constrained temporal motion vector prediction for error resilience", Joint Collaborative Team on Video Coding (JCT-VC), 4th Meeting: Daegu, KR, 20-28 January, 2011.

[6] J. Jung, G. Clare, "Proposition for robust parsing with temporal predictor" Joint Collaborative Team on Video Coding (JCT-VC), 4th Meeting: Daegu, KR, 20-28 January, 2011.

[7] Thomas Wiegand, et al, "WD1: Working Draft 1 of High-Efficiency Video Coding", Joint Collaborative Team on Video Coding (JCT-VC) 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

[8] Frank Bossen, "Common test conditions and software reference configurations", Joint Collaborative Team on Video Coding (JCT-VC) 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

# Patent rights declaration(s)

Broadcom Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).