| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E462 |
| --- | --- |

| Title: | Cross Verification of JCTVC-E343 Proposed by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joonyoung Park Younghee Choi Seungwook Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | 82-10-6272-9179 jy.park@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document reports the verification results of JCTVC-E343.

# Observation

Qualcomm provided their source codes. We have investigated them and make sure that there is no mismatch between the source code and algorithm descriptions from the document.

# Cross-verification results

The simulation succeeded and produced as same BD BR rate as proponent provided. The encoding and decoding times also show very similar aspect. The spreadsheets which contain the detailed data of the simulation are accompanied.

table 2.1. Result of experiment

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.6 | -0.5 | -0.5 | -0.7 | -0.6 |
| Class B | -0.5 | -0.4 | -0.4 | -0.4 | -0.3 | -0.3 |
| Class C | -0.6 | -0.6 | -0.6 | -0.5 | -0.5 | -0.5 |
| Class D | -1.2 | -1.0 | -1.0 | -1.0 | -0.8 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -0.6 | -0.6 | -0.6 | -0.6 | -0.5 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.1 |
| Class C | 0.0 | 0.2 | 0.2 | -0.1 | -0.2 | -0.3 |
| Class D | -0.1 | 0.3 | -0.1 | 0.0 | -0.2 | -0.1 |
| Class E | 0.0 | -0.4 | 0.2 | -0.1 | -0.3 | -0.2 |
| All | 0.0 | 0.1 | 0.1 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

table 2.2. Result of experiment

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -1.5 | -1.5 | -1.3 | -1.6 | -1.7 |
| Class B | -0.8 | -0.7 | -0.6 | -0.8 | -0.5 | -0.6 |
| Class C | -1.0 | -1.1 | -1.2 | -1.1 | -1.0 | -1.1 |
| Class D | -1.7 | -1.4 | -1.5 | -1.5 | -1.2 | -1.2 |
| Class E |  |  |  |  |  |  |
| All | -1.1 | -1.1 | -1.1 | -1.2 | -1.1 | -1.1 |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.4 | 0.2 | -0.1 | 0.1 | 0.3 |
| Class C | -0.1 | 0.1 | -0.1 | -0.4 | -0.4 | -0.5 |
| Class D | -0.1 | -0.2 | 0.2 | -0.1 | 0.0 | 0.2 |
| Class E | -0.1 | 0.0 | -0.2 | 0.5 | 0.6 | 0.7 |
| All | -0.1 | 0.1 | 0.1 | -0.1 | 0.1 | 0.1 |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |