| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E466 |
| --- | --- |

| Title: | Cross-Verification of Samsung’s Contribution (JCTVC-E288) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Y. Zheng I. S. Chong 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-651-9568 Email: zhengy@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The cross-verification reports the rate-distortion results and processing times of Samsung’s contribution JCTVC-E288 on loop filter with directional features. The rate-distortion results from this cross-verification match those reported by Samsung. The encoding and decoding times are comparable to those reported in JCTVC-E288.

# Introduction

In JCTVC-E288,  a block-based filter adaptation scheme with directional features (FAD) was presented. Blocks of 4x4 sizes are classified using four directional gradients. All 16 pixels within each 4x4 block are classified into the same class. For each class, the filter design follows the same procedure as the ALF in HM 2.0. The proposed filtering method will have up to 4 filters as compared to up to 16 as ALF in HM 2.0.

# Objective Verification Results

## Rate-Distortion Results

Tables 2 shows the rate-distortion results as BD-rate differences. The common test conditions are those specified in JCTVC-D600 for intra only, random access, and low delay configurations. The conclusion is that the BD-rate differences match those reported by Samsung in JCTVC-E288.

Table 1 Rate-distortion results

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.0 | 0.0 |
| Class B | 0.5 | -0.1 | -0.1 |
| Class C | 0.3 | 0.0 | 0.0 |
| Class D | 0.2 | 0.1 | 0.1 |
| Class E | 0.6 | -0.2 | -0.2 |
| All | 0.4 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 107% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.8 | 0.2 | 0.0 |
| Class B | 0.7 | 0.0 | 0.1 |
| Class C | 0.1 | -0.1 | 0.0 |
| Class D | -0.1 | 0.1 | 0.0 |
| Class E |  |  |  |
| All | 0.4 | 0.0 | 0.0 |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 114% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.3 | -0.2 | -0.2 |
| Class C | 0.0 | 0.1 | 0.1 |
| Class D | -0.2 | -0.2 | -0.5 |
| Class E | 0.2 | -0.3 | -0.3 |
| All | 0.1 | -0.1 | -0.2 |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 106% |  |  |

## Processing Times

Tables 2 also reports the encoding and decoding times for Samsung’s JCTVC-E288.

The encodings were executed on a Linux computing cluster consisting of machines with various computing performance and memory configurations. The encoder executable was built using GCC version 4.1.2. The differences in encoding times between our measurements and Samsung’s are likely attributable to the variation in execution times on our computing cluster. Overall, our encoding times are similar to those from Samsung’s.

The decodings were executed on CPUs with equal processing capabilities and memory configurations for both benchmark and the proposed software. The decoding times are bigger than reports of Samsung (6-24% bigger).

# Source Code Evaluation

The implementation is consistent with JCTVC-E288.

# Conclusion

The cross-verification matches the rate-distortion results reported in Samsung’s contribution (JCTVC-E288) for the three coding configurations. The encoding times are similar but decoding times are bigger than Samsung’s report.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).