| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E467 |
| --- | --- |

| Title: | CE12 Subset1: Cross-check of SKT/SKKU’s proposal on Deblocking Filter (JCTVC-E417) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

SKKU proposed in JCTVC-E417 modifications to the deblocking mechanism in the current HM2 design. This contribution reports the results of the cross-check done by the Institute for Infocomm Research.

# Introduction

This contribution proposes the following modifications to the deblocking process of HM2:

- Determining boundary strength based on the Intra prediction modes of Intra Prediction Unit (PU).

- Additional conditions are added when deciding on whether to filter pixels in the ‘second positions’.

- The “weak filter” of the HM2 deblocking algorithm is modified.

# Simulation results

The cross-check was carried out following common test conditions defined in JCTVC-600Error: Reference source not found.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -0.6 | -0.6 | -1.5 | -0.6 | -0.7 |
| Class B | -1.2 | -0.8 | -0.7 | -1.1 | -0.7 | -0.6 |
| Class C | -0.9 | -0.8 | -0.8 | -0.9 | -0.7 | -0.6 |
| Class D | -0.8 | -0.7 | -0.8 | -0.8 | -0.6 | -0.5 |
| Class E | -1.3 | 0.0 | -0.1 | -1.0 | 0.3 | 0.3 |
| All | -1.2 | -0.6 | -0.6 | -1.1 | -0.5 | -0.4 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -0.3 | -0.6 | -1.1 | -0.7 | -0.6 |
| Class B | -1.1 | -0.7 | -0.6 | -0.9 | -0.7 | -0.6 |
| Class C | -0.9 | -0.7 | -0.7 | -0.8 | -0.7 | -0.8 |
| Class D | -0.6 | -0.8 | -0.6 | -0.6 | -0.8 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -1.0 | -0.6 | -0.6 | -0.9 | -0.7 | -0.6 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.4 | -0.2 | -0.4 | -1.0 | -0.5 | -0.2 |
| Class C | -1.0 | 0.0 | -0.2 | -0.7 | -0.4 | -0.3 |
| Class D | -0.9 | -0.5 | -0.4 | -0.5 | -0.1 | -0.1 |
| Class E | -2.9 | -0.9 | -0.1 | -1.9 | -0.3 | 0.2 |
| All | -1.4 | -0.4 | -0.3 | -1.0 | -0.3 | -0.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

# Conclusions

We have cross-checked proposal JCTVC-E417 and verified the results provided by SKKU.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan. 2011.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).