| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E468 |
| --- | --- |

| Title: | CE6.a.4: Cross-check report for JCTVC-E266 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | Microsoft |  |  |

_____________________________

# Abstract

This document reports cross-verification results for Samsung’s proposal JCTVC-E266.

# Introduction

In JCTVC-E266, Samsung proposes to consider prediction from Luma to Chroma. We got the source code from Samsung and independently compiled and ran the code with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

The objective results we got match Samsung’s. The encoding/decoding times compared with the anchors’ are also similar to what Samsung reported.

# Experimental results

## Test 1

In test 1, Luma to Chroma is applied to all block sizes. Summary results are shown below. Detail results can be found at JCTVC-E468_1.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -16.2 | -8.3 | -1.1 | -14.6 | -9.4 |
| Class B | -0.8 | -7.2 | -3.7 | -0.8 | -7.3 | -4.2 |
| Class C | -1.1 | -6.7 | -7.4 | -1.2 | -6.4 | -7.1 |
| Class D | -0.7 | -4.7 | -4.6 | -0.8 | -4.8 | -4.6 |
| Class E | -0.1 | -3.3 | -3.7 | -0.2 | -4.8 | -5.2 |
| All | -0.8 | -7.8 | -5.6 | -0.8 | -7.7 | -6.1 |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -17.2 | -9.3 | -0.2 | -15.5 | -8.3 |
| Class B | -0.4 | -8.1 | -3.6 | -0.4 | -8.0 | -4.0 |
| Class C | -0.5 | -6.1 | -6.3 | -0.6 | -6.1 | -6.3 |
| Class D | -0.3 | -4.2 | -3.7 | -0.4 | -4.3 | -4.1 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -8.8 | -5.6 | -0.4 | -8.4 | -5.6 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -2.8 | -1.7 | -0.1 | -3.5 | -2.0 |
| Class C | -0.2 | -2.7 | -2.7 | -0.3 | -2.8 | -3.0 |
| Class D | -0.3 | -1.5 | -1.3 | -0.1 | -2.3 | -1.6 |
| Class E | -0.1 | -2.3 | -2.9 | -0.1 | -4.3 | -5.5 |
| All | -0.2 | -2.3 | -2.1 | -0.2 | -3.2 | -2.8 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

## Test 2

In test 2, prediction from Luma to Chroma is only applied when block size<=8. Summary results are shown below. Detail results can be found at JCTVC-E468_2.xls

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.8 | -8.6 | -5.1 | -1.1 | -13.6 | -8.8 |
| Class B | -0.5 | -4.7 | -2.7 | -0.6 | -6.0 | -3.7 |
| Class C | -1.0 | -6.4 | -7.1 | -1.2 | -6.2 | -7.0 |
| Class D | -0.6 | -4.5 | -4.4 | -0.8 | -4.7 | -4.6 |
| Class E | -0.1 | -1.9 | -2.8 | -0.2 | -3.8 | -4.5 |
| All | -0.6 | -5.4 | -4.4 | -0.8 | -7.0 | -5.7 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |